A New Spatial Database For Modelling Hydrological

Responses Within Southern Africa

K B Meier & R E Schulze

Department of Agricultural Engineering

University of Natal, Pietermaritzburg,

Private Bag X01, Scottsville, 3209

Introduction

In the assessment of local impacts of environmental change on hydrology, a region has to be disaggregated into relatively homogeneous response zones.  An example of this is the delineation of southern Africa in 712 zones by Dent, Lynch and Tarboton (1990).  For water resource studies, however, these response zones should be hydrologically linked, to enable for example:

*	the routing of streamflow from one zone to the next,

*	impacts on water supply and demand, and consequences of extreme events to be assessed,

*	land use change impacts on hydrological response to be evaluated at individual catchment/subcatchment areas.

In southern Africa, Quaternary catchments have been identified by the Department of Water Affairs and Forestry (DWAF) as the hydrological unit for operational purposes.

The objectives of this study were to:

1)		obtain the Quaternary catchment boundaries in digital form and determine which Quaternary catchments would require further disaggregation as a result of high intra-catchment variation in rainfall patterns,

2)		assign a currently operational, daily rainfall station to each Quaternary catchment to represent historical rainfall over the catchment,

3)		assess the quality of the rainfall data and infill any of the missing or suspect daily values,

4)		assign typical

*	monthly values of maximum and minimum temperatures as well as reference potential evaporation, and, additionally,

*	values of soils and land cover parameters for hydrological modelling purposes,

5)		use the ACRU distributed hydrological model to evaluate Quaternary catchment water resources under current conditions,

6)		change catchment parameters to indicate environmental changes which could occur in the catchment, examples of this being:

*	changing vegetation variables to simulate afforestation impacts at the catchment scale,

*	or perturbing the rainfall, temperature and evaporation values assigned to the catchments to anticipated future values derived from General Circulation Model (GCM) predictions,

7)		re-evaluate the catchment water resources in the light of possible environmental change, and

8)		evaluate the impacts of these environmental changes.

The Southern Afircan Quaternary Catchments

The DWAF has subdivided southern Africa into 22 Primary drainage regions.  These regions have further been divided into Secondary drainage regions.  There are a maximum of nine Secondary regions per Primary drainage region and each Secondary region is subdivided into Tertiary sub-drainage regions with again a maximum of nine per Secondary.  The Tertiary catchments were then subdivided into Quaternary catchments with a maximum of twelve Quaternary catchments per Tertiary region (Midgley, Pitman and Middleton, 1994).  It is at the Quaternary catchment level that a database has been developed in this project.

The southern African Quaternary catchments coverage was digitised by and obtained from Steffen, Robertson and Kisten (SRK).  These catchments cover South Africa, Swaziland and Lesotho.  In total there are 1946 Quaternary catchments with an average area of 650 km2.  A diagram depicting the southern African Quaternary catchments is given in Figure 1.

Using GIS utilities, the coefficient of variation (CV) of the gridded mean annual precipitation (MAP) values was obtained for each catchment.  The gridded MAP values are at one minute by one minute of a degree latitude/longitude resolution (Dent, Lynch and Schulze, 1988).  From the results, those catchments with a high CV were noted, indicating possible catchments in which further delineation would be considered necessary for more detailed hydrological measurements.

Catchment Rainfall

"Rainfall is the fundamental driving force and pulsar input behind most hydrological processes" (Schulze, Dent, Lynch, Schäfer, Kienzle and Seed, 1995, Pg AT3-1).  Rainfall - runoff models are particularly sensitive to the rainfall input and any error in rainfall estimates are amplified in streamflow simulations (Schulze et al., 1995).  For this reason, when selecting hydrological information to be used in modelling, the selection of the rainfall information should take the largest portion of the time spent on data acquisition.  This practice was adopted in selecting the rainfall information for the Quaternary catchments.  Over 9000 daily rainfall stations were considered in the process of selecting a representative rainfall station for each of the Quaternary catchments.  The rainfall stations were selected for each catchment to represent, as closely as possible, the typical rainfall pattern occurring over that catchment.  Criteria for each catchment were:

*	rainfall stations had to be currently operational so as to be easily updated in the future, and

*	stations with record lengths of a minimum of 20 years of continuous data were considered to account adequately for inter-annual rainfall variations.

The procedure adopted in the selection of the rainfall stations was as follows.

In the ARC-INFO GIS, a coverage of the southern African rainfall stations, which included station information obtained from the Computing Centre for Water Research (CCWR) and the meteorological departments of Swaziland and Lesotho, was created and unioned with the coverage of the Quaternary catchments.  A program was written to analyse all the rainfall station records so as to give a general idea of the station information and quality.  This provided information inter alia on the first and last year that rainfall was recorded at a station, the last year, the number of complete years of record, and a statistical analysis on the years which contained missing records.  Stations were then ranked depending on the quality of the data.  A further program was written to:

*	automatically select all rainfall stations which would qualify in terms of all the criteria set out for the study,

*	were located within the catchment, and

*	were only positioned in catchments with low CVs.

If a catchment contained two or more representative rainfall stations within it, the station with the best quality data was assigned to represent that catchment.  Using this procedure, a total of 720 catchments were assigned representative rainfall stations.

For catchments which either did not have any stations within them, or if all stations within the catchment did not fit the required criteria, a different method was adopted to select the best representation of rainfall.  In this case the coverage of the Quaternary catchments with the rainfall stations was displayed on GIS and this image was overlain onto the minute by minute grid of MAP.  In this way it was possible to visually identify the stations which best fitted the catchment MAP, as shown in Figure 2.  Station and catchment information could be accessed manually using ARC-INFO.  By this procedure, rainfall stations were chosen for all catchments which had not already been assigned a representative station.  In total, 1200 rainfall stations have been chosen to represent rainfall at all the catchments, with some stations having to represent more than one catchment due to a lack of representation over areas.  Once the stations had been selected to represent the Quaternary catchments, missing and suspect rainfall values had to be infilled.

Synthesising Missing Rainfall Records

For rainfall stations with missing or suspect values, a technique had to be applied to give an accurate reflection of the magnitude of rainfall on any particular day.  After considering numerous methods it was decided to use an inverse distance weighting technique to infill these values.  The reasons for using this method included:

*	rainfall values infilled using this method would be historically representative and not stochastically generated, and hence represent as closely as possible the rainfall actually occurring on the day (this being important when cascading runoff from one catchment to the next one downstream),

*	a number of surrounding rainfall "driver" stations could be used in the synthesising process, and

*	inverse distance weighting provided the fastest computational method of those considered, with time being of value when considering the number of station records which had to be synthetically generated.

The procedure of inverse distance weighting involves the weighting of the recorded rainfall from stations surrounding a driver station inversely, depending on the distance of those stations from the driver station.  This gives the closest station the highest weighting and the furthest station the lowest.  In this case, the inverse of the distance squared was used so as to give the closest station a much higher representation as indicated in Equation 1.  Because of the spatial variability of the rainfall it was necessary to account as closely as possible for the actual rainfall pattern surrounding the driver station.  For this reason, the following sequence of computational steps was used, and is shown in Figure 3:

*	Identify all rainfall stations within one degree latitude and longitude of the driver station,

*	divide the area surrounding the driver station into four quadrants using the latitude and longitude of the driver station as the bisectors,

*	identify the closest 10 rainfall stations from each quadrant and retrieve the rainfall records from those stations from the CCWR rainfall database,

*	search through the driver station record for the first missing or suspect day of rainfall, and

*	identify the closest station in each quadrant with an actual value of rainfall occurring on that day and use that rainfall value for patching.

Mathematically this procedure is thus expressed as

where

rs	=	synthesised rainfall for a specific day of the driver station (mm)

r1,2,3,4	=	actual rainfalls which fell at the closest stations in quadrants one to four respectively (mm)

d1,2,3,4	=	distance of each of the four closest stations from each quadrant to the driver station respectively (°)

In cases where a certain quadrant had no rainfall values on the day required, a single rainfall value was used from each of the remaining quadrants.  If two quadrants had no stations with representative rainfall for that day, two stations were selected from each of the remaining quadrants so as to minimise the chance that the stations used for patching would be located close to each other, thus recording the rainfall of a similar area.  If three quadrants had no stations with recorded rainfall on the day in question, then three rainfall stations were used in the final quadrant for the reason indicated above.  The average number of patched days per rainfall station was 700 over a 21 year record period from January 1972 to December 1993.

Monthly A-Pan Evaporation, Maximum Temperature and Minimum Temperature

Minute by minute of a degree gridded values of maximum temperature (Tmax), minimum temperature (Tmin) (Schulze and Maharaj, 1993) and reference potential evaporation (Er) (Schulze and Maharaj, 1991) were calculated from regional and monthly multiple regression equations using measured values against latitude and longitude, altitude, distance from sea and a topographic (valley) index.  These values were then averaged for each Quaternary catchment using ARC-INFO, as shown in Figure 4, with the result that for each catchment the month-by-month values for Tmax, Tmin and Er were obtained.

Soils and Land Cover Parameters

For hydrological modelling purposes the following land cover / land use information is required on a month-by-month basis:

*	interception loss by the vegetation, in mm.rainday-1,

*	fraction of the effective root system in the topsoil, and

*	an evapotranspiration index, expressed by the crop coefficient.

These were derived for natural vegetation using the Acocks (1988) Veld Types (Schulze and Hohls, 1993) and values were area weighted for each catchment.

Soils information for modelling includes:

*	the thickness of the topsoil and subsoil profiles,

*	the soil water contents at

-	permanent wilting point

-	drained upper limit

-	saturation (porosity) for the topsoil and subsoil respectively, and

*	the fraction of "saturated" soil water to be redistributed from one soil horizon to the next on a daily basis, if the first soil water content is above drained upper limit.

The soils information was derived by the techniques described in Schulze, Angus, Lynch and Furniss, (1990) and area weighted for each Quaternary catchment.

Linking the Database to the ACRU Model

The Quaternary catchment database has been set up as an input to the ACRU agro-hydrological model.  Each Quaternary catchment is modelled as a lumped catchment.  In this way a catchment is considered individually with no links to other catchments.  The Quaternary catchments are, however, hydrologically linked and simulated streamflow can be routed from one catchment to the next using selected routines.

An Example of Applying this Database: Impacts of Afforestation on Streamflow

An example of the application of this database is a spatial study of the impacts of afforestation on streamflow at the Quaternary catchment scale, throughout southern Africa.  In this case study it was assumed that each Quaternary catchment with an average MAP exceeding 600 mm, was covered by veld in good condition.  A base run was performed using the ACRU model and an estimate of the average streamflow for each Quaternary catchment was obtained.  A further run was then performed assuming that 100% of the Quaternary catchment to be planted with six year old Eucalyptus Grandis trees on pitted site preparation.  It was then possible to asses the impact that this afforestation could have, on streamflow at the Quaternary scale, throughout the country.  A map showing the simulated impacts is given in Figure 5.  In reality it is unreasonable to assume that 100 % of a catchment will be afforested, however, this example gives an indication of hydrological response changes at the unit area with conversion to forest.  If a lower percentage afforestation were required, multiplication of the streamflow reduction by that percentage would indicate the impact.

Discussion and Conclusions

The Quaternary catchment database has many possible future applications including the assessment of the impacts of environmental changes on water resources, agriculture and terrestrial ecosystems.  It is being applied in the assessment of the impacts of afforestation and global climate change on water resources at the Quaternary catchment scale in southern Africa.  The example used in this paper has been, the impact of converting grassland to commercial afforestation  with results indicating that in areas of lower annual rainfall the percentage reduction in streamflow is far higher than in catchments with higher rainfall.  This suggests a greater impact of afforestation on water yield in the areas with lower rainfall.

The Quaternary catchments, although too large to be considered completely hydrologically homogeneous, provide the smallest operational areas at which studies are performed at a country-wide scale in southern Africa.  The database developed includes information for each catchment on daily rainfall, evaporation, maximum and minimum temperature and hydrological soils and natural vegetation information.  This information can be considered as the minimum information necessary to provide basic analysis of hydrological impacts.  It is therefore considered as the base information on which a comprehensive database can be built in the future.  Information to be included in the database in the future include a higher detail of information on actual land use and soils, daily streamflow information as well as daily values for temperature, evaporation and solar radiation.

References

Acocks, J.P.H. (1988). Veld types of South Africa.  Memoirs of the Botanical Survey of South Africa no. 57, Leistner, O.A. (Ed).  Botanical Research Unit, Department of Agriculture and Water Supply. 146 pp.

Dent, M.C., S.D. Lynch & H. Tarboton (1990).  Detailed delimitation of rainfall regions in southern Africa.  Water S.A., 16, 1-4.

Dent, M.C., S.D. Lynch & R.E. Schulze (1988).  Mapping mean annual and other rainfall statistics over southern Africa. University of Natal, Department of Agricultural Engineering, ACRU Report 27, Water Research Commission, Pretoria. Report No 109/1/89.

Midgely D.C., W.V. Pitman & B.J. Middleton (1994).  Surface water resources of South Africa 1990.  Water resources 1990 joint venture.  Water Research Commission, Pretoria.  In Press.

Schulze, R.E., G.R. Angus, S.D. Lynch & P.W. Furniss (1990)  Primary productivity over southern Africa : An example of agricultural resource determination.  Agricultural Engineering in South Africa, 22, 22-23.

Schulze, R.E., M.C Dent, S.D. Lynch, N.W. Schäfer, S.W. Kienzle & A.W. Seed (1995).  Rainfall.  In: Schulze, R.E. Hydrology and Agrohydrology: A Text to Accompany the ACRU 3.00 Agrohydrological Modelling System. Water Research Commission, Pretoria, Report TT69/95. AT3-1 to AT30-38.

Schulze, R.E. & B.C. Hohls (1993).  A generic hydrological land cover and land use classification with decision support systems for use in models.  Proceedings of the Sixth South African National Hydrological Symposium, Pietermaritzburg. 547-556

Schulze, R.E. & M. Maharaj (1991).  Mapping A-pan equivalent potential evaporation over southern Africa. Proceedings of the Fifth South African National Hydrological Symposium, Stellenbosch,  4B-4-1 to 4B-4-9

Schulze, R.E. & M. Maharaj (1993).  Regional adiabatic lapse rates in southern Africa for maximum and minimum temperatures.  Department of Agricultural Engineering, University of Natal, Pietermaritzburg.  Mimeographed.

Figure 1.	The 1946 Quaternary catchments of southern Africa.  Source: SRK/DWAF

Figure 2.	The selection of a representative rainfall station for each catchment.  Where:

(1)	Rainfall stations which do not have current rainfall records

(2)	Rainfall stations which have current rainfall records but have less than 20 years of recorded data

(3)	Rainfall stations with current rainfall records and with at least 20 years of record

Figure 3.	Synthesising rainfall using inverse distance weighting.  Where:

rs	=	synthesised rainfall for a specific day of the driver station (mm)

r1,2,3,4	=	actual rainfalls which fell at the closest stations in quadrants one to four respectively (mm)

d1,2,3,4	=	distance of each of the four closest stations from each quadrant to the driver station respectively (°)

rx	=	the closest rainfall station in a quadrant but with no rainfall for patching

Figure 4.	The averaging of the July, A-pan gridded evaporation over a Quaternary catchment.

Figure 5	Percentage reduction in streamflow for catchments with veld in good condition changing to 100 % afforestation with Eucalyptus Grandis.