WR90 - Surface Water Resources of South Africa 1990 - The Final Update

B J Middleton*, W V Pitman**, and D C Midgley***

Steffen, Robertson and Kirsten*, Stewart Scott Inc**, Watermeyer Legge Piésold & Uhlmann***

INTRODUCTION

Water is arguably South Africa's most precious natural resource.  Research has shown that groundwater resources are relatively meagre.  South Africa's major water supplies, whether for rural or for urban development, are and will probably always be derived mostly from surface sources.

The climate of South Africa is semi-arid, with average rainfall of 475 mm per annum, compared with, for example, 735 mm for the USA and 860 mm for the world.  Rainfall in the country varies tremendously, from highs of greater than 2 000 mm in the Cape Coast and Eastern Escarpment, to less than 100 mm in the Northern Cape.  For the country as a whole only about 9 percent of the precipitation, (ie 43 mm or about 52 x 109 m3/annum) reaches the river systems as runoff.  The economically exploitable proportion of this is estimated at 34 x 109 m3/annum.  It is predicted that all of this will be fully exploited by the year 2020.

A means of evaluating surface water resources, and the results of such an evaluation, are essential to planners and designers of water supply schemes as well as to managers of completed schemes.  Whether one is concerned with large-scale water developments involving whole regions or merely with on-farm improvements, water resources information is essential, for example, for the sizing of storage or diversion works, for decision-making on the operation of storage or for simulating the behaviour of water control systems.  Equally important is information and data for the managers of the system, including water quality and environmental considerations.

Four comprehensive surveys of South Africa's surface water resources have been undertaken over the last 40 years, the first in 1952 and the last currently in print.

The aim of the current study, "Surface Water Resources of South Africa 1990", (the WR90 study) is to provide a basis for a preliminary planning of water resources development.  The survey is also to make available, in a single set of documents, valuable data and information for water resources planning and development.

The publication is in 6 volumes.  The information and time-series of the hydrometeorological data are provided in a set of appendices, whereas the spatial information is shown in a set of 10 maps created using Geographic Information Systems.  A user's manual is provided which gives the methodology used to create the information and maps, outlines problems and pitfalls and also provides a user's guide to the documents which includes worked examples.

AIM AND OBJECTIVES OF THE WR90 STUDY

The Water Research Commission of the Republic of South Africa in March 1990 appointed a consortium of consulting engineering firms to undertake the project "Surface Water Resources of South Africa 1990" (hereinafter called the WR90 Study).

The aim of the WR90 Study was "........ to update and improve the 1981 survey of the Surface Water Resources of South Africa by the Hydrological Research Unit of the University of the Witwatersrand, Johannesburg."

The primary objective of the WR90 Study was " to provide a basis for a preliminary planning of water resources development.  The survey will also make available in a single set of documents valuable data and information for water resources planning and development."

The background to the study, the grouping of drainage regions into the report volumes, the format of the report series and the use thereof are outlined in the next sections.

BACKGROUND TO THE STUDY

In 1981 the Hydrological Research Unit (HRU) of the University of the Witwatersrand, Johannesburg, published a six-volume report entitled "Surface Water Resources of South Africa".  Each volume comprised two parts - Part 1 (text): methodology, description of the particular region covered, and worked examples of water resources assessment problems, and Part 2 (Appendices).  As compilation of the 1981 six-volume series had commenced about 5 years earlier (in 1975) the data base for most of the six volumes comprised records ending in 1976.

"Surface Water Resources of South Africa" (referred to here as the 1981 Survey) proved extremely popular among a wide variety of users.  Additional information was considered necessary, and therefore, with funds provided by the Water Research Commission (WRC), members of the original HRU team produced:

∙	an Addendum and

∙	a Guide to the Survey.

These were published in 1983.  The Addendum contained flow duration curves and evaporation information as well as corrections of some of the mistakes that had been pointed out by users.  The Guide contained information on legal aspects, warnings and possible pitfalls as well as further examples of problem solving, by both hand-calculation and computer techniques.

In response to widespread requests for a revised and updated survey, in 1988 the Water Research Commission sponsored Prof Midgley, Dr W V Pitman and Mr B J Middleton to undertake a study of what would be entailed in "updating water resource information in South Africa".  Work started in 1988 with a critical review of the 1981 Survey, followed by distribution of a questionnaire among persons likely to be in a position to offer useful comments on possible improvements.  The response to the questionnaire reflected general enthusiasm for an updating of the Survey and many valuable suggestions for improvement were received.  From an analysis of the replies to the questionnaire a list of major issues was compiled along with a preliminary suggested format for a proposed revised survey.  These proposals served as agenda for a series of workshops held at several centres in South Africa.  In addition, many informal discussions were held with interested water planners.  The outcome of the workshops and informal discussions was the compilation of preliminary ideas on a format for the proposed revised survey.  A report was submitted to WRC in November 1988.

In March 1990 a contract was concluded between WRC and the consortium of consulting engineering firms to undertake the project "Surface Water Resources of South Africa 1990" (WR90 Study).

The steering committee appointed by WRC comprised representatives of the Department of Water Affairs and Forestry (DWAF), Department of Agriculture, Computing Centre for Water Research (CCWR), Department of Agricultural Engineering of University of Natal, Department of Civil Engineering of University of Stellenbosch, CSIR Forest Science and Technology, Institute for Water Research of Rhodes University, Department of Landscape Architecture of University of Pretoria, as well as of the contracting consulting firms.

The project was started in January 1990 and ended in 1995.

METHODOLOGY

Grouping of Drainage Regions

It was decided to provide all water resource information for the Country at a quaternary sub-catchment level.  South Africa is divided, in terms of work done by the DWAF, into 22 primary drainage regions, further sub-divided into secondary and tertiary sub-catchment areas.  The WR90 team added a number of tertiary catchments and then defined the quaternary sub-catchment boundaries, to create some 2 000 sub-catchments in the Country.  Information provided for each sub-catchment is rainfall, evaporation, runoff, land cover and water-use (including urban areas, afforested areas), soil types, natural vegetation and underlying geology.

The primary subdivision of the country accords with DWAF Drainage Region Map HR 0001.64.1965 and, for treatment in the six volumes of this project, the primary drainage regions have been grouped as shown in the map on Fig. 1. and as named in Table 1.

Table 1 : Grouping of drainage regions

| Volume No | Name of Volume | Primary Drainage Regions |
| --- | --- | --- |
| I | Limpopo-Olifants | A, B |
| II | Vaal | C |
| III | Orange (excl) Vaal | D, F |
| IV | Western Cape | E, G, H, J, K, L |
| V | Eastern Cape | M, N, P, Q, R, S, T |
| VI | Eastern Escarpment | U, V, W, X |

Figure 1: Grouping of Drainage Regions and Numbering of the Report Series

Analysis

The analysis for the study was aimed at providing information on streamflow and yield for all catchments, gauged and ungauged, in the country.  A significant proportion of the country is not covered by streamgauges, and so use was made of a computer model to derive synthetic streamflow sequences, from which analyses could be done.  Simulation and the preceding calibration for the catchments commanded by each reliable streamflow and reservoir gauge was undertaken by means of the program WRSM90 (Pitman & Kakebeeke 1991).  WRSM90 is of modular construction, with four different types of module (runoff, channel, reservoir and irrigation) linked by a network of routes:

∙	The runoff module is the heart of WRSM90.

∙	The channel module has the main function of collecting the inflows from various routes and redistributing these along the outflow routes.  The channel module also makes provision for river bed losses and evaporative losses from wetlands or aquifers.

∙	The reservoir module is used to represent a single reservoir.

∙	The irrigation module is for irrigation use.

Land-use components can be allowed to change with time to mimic the actual historical changes that have taken place over the period for which streamflow data are available.  These land-use features include irrigation, exotic forests and urbanised areas, reservoirs (capacities and surface areas) and all kinds of water transfers (e.g. effluent discharges to and abstractions from reservoirs and streams).  Major identified channel losses can also be incorporated in the calibration process.  With WRSM90 it is possible to model large complex catchments that have been subject to changing land-use developments over time.  This feature allows one to make use of virtually all reliable streamflow gauges for calibration purposes.

The analysis process was undertaken in a number of steps:

∙	Data collection and collation

The model uses monthly rainfall and evaporation data and this was obtained for all rainfall and evaporation gauges with reasonable length records.  The CCWR supplied much of this data from its records.  The data were then checked and patched where necessary, using standard practice.

Streamflow data is required for calibration purposes, as was land cover and water use data.  All reliable streamflow and reservoir gauge information was accessed from the DWAF, who provided support in updating records and reviewing the data queries.  The land cover and water use data are the most difficult to obtain.  Where basin studies had been undertaken by the DWAF, these data were readily accessible.  In other areas the WR90 project employed knowledgable local practitioners to obtain representative data.

∙	Model calibration

The computer program was run with trial values of the model parameters, in many cases taken from the earlier 1981 Survey or from basin studies.  The resulting MAR and statistics were compared with those of the monthly streamflow observed at each gauge.  Graphs showing various features of the modelled monthly streamflow sequence were also compared visually with those of the observed sequence.  Parameter values were then adjusted until agreement was satisfactory, that is, the observed and simulated hydrograph coalesce within the reasonably-to-be-expected rainfall sampling and modelling errors, with the statistics of both series agreeing reasonably well.

∙	Regionalisation of model parameters/synthetic hydrology

Model parameters were then transferred from gauge catchments to adjoining ungauged catchments using as indicators the natural characteristics and hydrological response of the catchments.  Smoothing of parameters then took place and synthetic hydrology was generated for each quaternary catchment.  These results were checked with actual records where possible.

∙	Yield analysis

The streamflow information was then further analysed to give deficient flow-duration-frequency and storage-draft-frequency curves.

FORMAT OF REPORTS

Each of the six volumes of the WR90 Study is in three parts viz a User's Manual, a set of Appendices and a Book of Maps.

The User's Manual is common to the whole country and contains six chapters:

Chapter 1 : Introduction

Chapter 2 : Methodology

Chapter 3 : Problems and pitfalls

Chapter 4 : Legal aspects

Chapter 5 : User's guide (including worked examples)

Chapter 6 : References

Each chapter has its own contents page.

The User's Manual is structured both to give background information to water resources in South Africa, and

to guide the users through the information which is presented in the Appendices and Book of Maps.  Background information includes the methodology used in the study so that the user will understand the limitations of the information, and the legal framework within which water resources problems must be solved.  The problems and pitfalls chapter highlights these areas where problems have previously been experienced by practitioners in water resources, and also where data are insufficient for final answers.  The User's guide chapter helps the user through a number of practical problems, using the data in the report series as basis.

A set of Appendices is specific to the drainage regions grouped in the volume and contains 3 primary sections, viz data, streamflow modelling and yield analysis, grouped as follows:

Appendix 1:	Introduction (description of the drainage regions, relevant white papers and basin studies and problems encountered during analysis)

Data

Appendix 2:	Rainfall (list of rainfall stations and data on rainfall zones)

Appendix 3:	Evaporation (list of evaporation stations, data on evaporation zones, pan and crop factors, Symons/Class A pan conversions)

Appendix 4:	Streamflow (list of streamflow stations, naturalised streamflow records, statistics of naturalised streamflow records)

Appendix 5:	Land cover and water use (inventory of dams, irrigation, water transfers, urban/impervious areas, wetlands, aquifers and other channel losses, consumptive water use by afforestation).

Streamflow Modelling

Appendix 6:	Model calibration (calibration summary, regionalised model parameters)

Appendix 7:	Simulated natural streamflow (primary, secondary and tertiary catchments MAR, simulated natural streamflows for tertiary catchments)

Appendix 8:	Quaternary and tertiary catchment information

Appendix 9:	Rainfall-runoff relationship

Yield analysis

Appendix 10:	Low-flow analysis (deficient flow-duration-frequency curves, average monthly distribution of streamflow)

Appendix 11:	Storage yield analysis (storage-draft-frequency curves, seasonal variation in storage adjustment factors, universal evaporation loss equations, sediment accumulation in reservoirs)

Computer information

Appendix 12:	Computer information (Computer information, viz hydrometeorological data and GIS information is available on request on computer media).

Each appendix has its own contents page.

A Book of Maps, for each volume, all maps of A3 size and almost all to the same scale 1:1 000 000.  The book of maps comprises:

Maps 0 : Base map

Maps 1 : Rainfall

Maps 2 : Evaporation

Maps 3 : Runoff

Maps 4 : Land cover and water use

Maps 5 : Streamflow modelling

Maps 6 : Geology

Maps 7 : Soils

Maps 8 : Sediment yield

Maps 9 : Vegetation.

Map information (map pedigree, GIS workspaces, list of coverages).

All maps were prepared using Arc Info Geographic Information System software and plotted to Albers projection with central meridian at 24°E.  Details of the maps follow:

Base Map (Maps Series 0): The area dealt with in each volume has been subdivided into areas of convenient size, for each of which there is a Base Map (1:1 000 000 scale).  To fit an A3 sheet some areas had to be reduced to scale 1:1 500 000.

The base maps show international boundaries, catchment boundaries down to quaternary level, quaternary sub-catchment numbers, selected rivers, river names, major dams with names, urban areas more than 5 km2 in extent plus some reference towns, and the lines of latitude and longitude.

Also shown on the maps are the endoreic areas.  These were created by demarcating areas of internal drainage on the 1:250 000 scale topo sheets, and editing the obvious small errors at the boundaries shown up by overlaying the quaternary sub-catchments.  Two classes of endoreic areas were created, namely local endoreic areas, from which no runoff normally reaches the river system, and global endoreic areas, from which there is runoff to local streams which do not, however, reach the major river system or the ocean.

The base map appears as Map Series 0 and, in an edited version, forms a suppressed background to all the maps in the Book of Maps.

Rainfall (Maps Series 1): Raingauge stations in South Africa are located by section numbers (each section covers an area of 0.5° by 0.5°) and position number within that section.  Overprinted on the base map are the Weather Bureau (WB) section numbers by area.  Gauges are located by dots and the position numbers of raingauges, with differentiation between gauges used and those not used in the WR90 study.  Not all rainfall stations in the regions were selected.  A preliminary screening was done to eliminate all records shorter than 15 years.  However, where data for a particular area were sparse, information from some stations having records shorter than 15 years were used.  Also shown are the boundaries of the rainfall zones used in the analyses, with zone number.  Ranges of Mean Annual Precipitation (MAP) are colour-coded.

Evaporation (Maps Series 2): On the base map background are shown the positions of evaporation gauges, gauge numbers, evaporation zones and zone numbers.  Ranges of Mean Annual Evaporation (MAE) are colour- coded.  The gross evaporation map is essentially the same as that published in the 1983 Addendum to Surface Water Resources of South Africa, (Midgley et al 1983).

Runoff (Maps Series 3): Against the base map background are shown streamflow stations.  The streamflow station positions and data were obtained from DWAF.  It is to be noted that not all the data from DWAF can be usefully employed.  Records at many of the stations were too short or too disjointed or in some way suspect; these were discarded.  Hydro zones, and hydro zone numbers are also given.  Ranges of runoff in mm are colour-coded.  Rainfall-runoff-response numbers are printed for each quaternary sub-catchment along with the gauge number.   The hydro zones and runoff-response groupings were established from the analyses, as was the mean annual riverflow.

Land Cover and Water Use (Maps Series 4): With the base map as background are shown all the major dams with names, minor dams, the afforested areas and type of forest, areas of sugarcane, the Government Water Control Areas, Water Board supply areas and points of major discharge to and abstractions from rivers.  Inter-basin transfers between drainage regions are named and shown with arrows indicating the direction of flow.

Streamflow Modelling (Maps Series 5): On the base map background are shown the areas assigned the same regionalised model parameters.  Each of these areas is demarcated by a number.  Three different parameter groupings are indicated by means of shading.  Regional model parameters were obtained from the analyses.  The numbering within each group gives an approximate indication of the characteristics of generated streamflow; the higher the numbers the more erratic the behaviour.

Geology (Maps Series 6): On the base map background are shown broad geological groupings.  The broad geological groupings were taken from a map derived by Vegter in terms of a project for the Water Research Commission to delineate similar hydrogeological regions in South Africa.  Vegter's map is based on the 1:1 000 000 Geological Map of South Africa, compiled in 1984 by the Geological Survey, Department of Mines and Energy Affairs.

Soils (Maps Series 7): On the base map background are shown broad soil groupings, each colour-coded and numbered.  The groupings were made on a map showing 82 soil types produced by the Department of Agricultural Engineering of the University of Natal from information provided by the Institute for Climate, Soils and Water.  The soil types were analysed according to features most likely to influence hydrological response, viz depth of soil, soil texture, and slope.  Some 11 broad groupings were obtained from the analysis.

Sediment Yield (Maps Series 8): The most recent sediment studies are those by Prof Rooseboom who mapped sediment yields of South African catchments (Rooseboom, 1992).  The map provided is based on his work and shows his 10 regions delineated on the soil erosivity map.

Vegetation (Maps Series 9): Vegetation is perhaps the most significant tell-tale of hydrological behaviour of a catchment.  A map is provided based on Acocks's 1:1 500 000 map: Veld types of South Africa.

MULTIPLE USES

Experience following the publication of the 1981 Survey has been that unexpected interest was aroused among a wide range of disciplines.  Researchers in many different fields were able to find in the publication information of value to their particular study.  Some had to adapt the material to suit their particular purpose; others, unfortunately, used the material incorrectly, with possibly misleading results.  Accordingly, in preparing the WR90 Study for publication, careful thought was given to the needs of a wide spectrum of users.  At the same time attention is drawn to the warnings about misinterpretation or misapplication of the data and results of analyses.

The principal users of the publication are grouped under five lettered headings:

W : Water Resources Engineer

Q : Water Quality Specialist

E : Ecologist/Environment Manager

A : Agriculturist

F : Flood Hydrologist.

Most other interested groups could fall under one or other of these categories.

It was decided to use in the WR90 study the "building block" form of presentation with the material arranged in three layers:

∙	data/information

∙	streamflow information

∙	storage-yield analysis.

Some of the more important information and procedures within each of the three layers of information (viz Data, Streamflow and Yield Analysis) are listed in Table 2, with the likely most interested user-group signified.

SUMMARY AND CONCLUSIONS

The appraisal of the most recent survey of the Surface Water Resources of South Africa, the WR90 Study, has been described.  The methodology has been briefly detailed, and the format of the reports and some details of the outputs of the study given.  As with any study of this type, the most important feature is its wide number of applications, and here a listing is given of some of the disciplines who can make use of the study, and some of the data they can obtain and analysis they can do.

It can be seen that the study performs a vital function in providing essential information to better manage South Africa's water resources.

REFERENCES

Midgley, D C., Pitman, W V., and Middleton B J. (1994).  Surface Water Resources of South Africa 1990.

Reports 298/1/94 User's Guide.  Appendices, reports 298/2.1 - 2.6/94 and Books of Maps, reports 298/3.1 - 3.6/94, Water Research Commission, Pretoria, South Africa.

Rooseboom, A. (1992): Sediment transport in rivers and reservoirs - a southern African Prospective.  Wat. Res. Comm. Report No. 297/1/92.

Hydrological Research Unit (1981).  Surface Water Resources of South Africa.  HRU Reports numbers 8/81 to 13/81, University of the Witwatersrand, Johannesburg.

Midgley, D C, Pitman, W V, AND Middleton, B J (1983).  Addendum to surface water resources of South Africa.  Water Research Commission, Pretoria.

Midgley, D C, Pitman, W V, AND Middleton, B J (1983).  Guide to surface water resources of South Africa.  Water Research Commission, Pretoria.

TABLE 2 : A MATRIX OF INFORMATION, PROCEDURE & RELEVANT MAJOR USER-GROUPS

|  | W | Q | E | A | F |
| --- | --- | --- | --- | --- | --- |
| DATA |  |  |  |  |  |
| Rainfall Station details - Select stations to calculate rainfall over a catchment or area Isohyetal map - Determine MAP at point of interest or over an area Rain zones - Provide input to WRSM90 for simulation - Obtain monthly and annual means and standard deviations - Establish severity, duration and extent of historical droughts - Establish severity, duration and extent of historical floods | x x x x x x |  | x x x x | x x x x | x x x |
| Evaporation Station details - Select S-pan record for reservoir or lake - Select A-pan record for crop water demands Iso-evap map - Determine evaporation for reservoir or catchment Symons Tank/A-pan equation - Determine S-pan data for ungauged areas Pan factors - Convert A-pan to crop or veld evaporation Land cover and water-use Afforestation - Establish reduction in MAR and flood runoff Irrigation areas - Estimate abstractions and reductions in riverflow - Identify streamflow pollution source Reservoirs - Determine abstractions and reduction in riverflow Urban areas - Estimate increase in runoff - Identify pollution source Point sources - Establish accretions or reductions in riverflow - Identify pollution source - Determine low-flow augmentation or reduction All land-use - Provide input to WRSM90 | x x x x x x x x x x x x x x x | x x x x x | x x x x x x x x x x x x | x x x x x x | x x x |
| STREAMFLOW Station details - Select records for model calibration Natural record - Establish natural hydrology at gauge MAR/MAP eqn - Estimate MAR on areas smaller than quaternary Quaternary flows - Provide inputs to reservoir yield analysis - Provide inputs for stochastic analysis - Provide inputs for low-flow analysis - Establish severity, duration and extent of historical drought - Establish severity, duration and extent of historical floods | x x x x x x x x | x | x x x x x | x x | x |
| YIELD ANALYSIS Hydro-zone DDF - Determine low-flow for run-of-river problems - Determine low-flow for storage determination Hydro-zone SDF - Make quick assessment of reservoir storage Groundwater - Obtain indications of river's low-flow regime | x x x x | x x | x x | x |  |