A Frequency Formula for Drought Severity

T C Sharma

School of Environmental Studies

Moi University

P O Box 3900

Eldoret, Kenya.

Introduction

The late Prof V T Chow (1951) gave a formula which when transformed for flood frequency analysis can be written as QT = QM + kT.σQ in which QT is the flood magnitude for the return period of T years, QM and σQ are the mean and standard deviation of the flood sequence Q and kT is a frequency factor dependent on the return period T and the probability distribution function (pdf) of the Q sequence.  The formula is applied on the largest values of flood sequences such as the annual maximum flood (AMF) series, which generally display negligible level of persistence.  The formula is simple yet a powerful model for designing the runoff conveyance structures.  Parallel to the flood frequency formula, a formula can be evolved which can predict the largest drought severities for the desired return periods.  The largest drought severity is the most important parameter useful in the design of water storage systems to fulfil the water shortages during a drought spell.  The other parameter useful in  the design is the critical drought magnitude which can be defined as the ratio of the largest drought severity to the longest drought duration in line with the definition of magnitude proposed by Dracup et. al. (1980).

The drought severities are normally expressed in terms of the cumulative shortages (deficit-sums) below a desired demand level (Yevjevich, 1972).  Rainfall or runoff sequences, constitute the variables for the drought severity analysis.  A drought has to be  measured in relation to the demand level for a given activity and can be related functionally to some parameters of the drought variable.  One such parameter expressing the demand in terms of the drought variable is truncation level.  The demand level varies from activity to activity and, therefore, truncation level is to be fixed beforehand by the designer while designing a water storage system to meet the water shortages during a drought spell.  The other parameter to be fixed by the designer is the return period, T, which inculcates the component of the risk in the design.  Once the aforesaid two parameters are chosen based on some considerations, the third element needed for the design is the largest severity.  In the hydrologic literature, the largest severity is also termed as critical severity (Sen, 1980).  The derivation of a formula for the largest severity parallel to the flood frequency formula is the subject matter of this paper.

The formula has been illustrated using the annual river flow data from the Tana and Lake Victoria basins in East Africa.  The gauging stations used in the Tana basin are 4AC03 and 4BC02 on the main Tana river, which is blood line of Kenya.  The major dams for hydropower generation are built across this river (Fig. 1).  The Lake Victoria basin encompasses several countries in East Africa, but in Kenyan territory the river analyzed is the Sondu which is gauged at station 1JG1.  The outflows of the Lake Victoria used in the present analysis are gauged at Jinja on the Victoria Nile in Uganda (Fig. 1).  The data for the aforesaid stations were acquired from the Ministry of Water Development, Nairobi, Kenya.

Table 1:	Flow statistics for four rivers in East Africa.

| Attribute | Station 4AC03 | Station 4BC02 | Station 1JG1 | Station Jinja |
| --- | --- | --- | --- | --- |
| River Name | Tana (Kenya) | Tana (Kenya) | Sondu (Kenya) | Victoria Nile (Uganda) |
| Catchment are (km2) | 1 632 | 2 876 | 3 481 | 149 000 |
| Mean annual flow (m3 X 108) | 3.55 | 9.70 | 12.91 | 208.07 |
| St. dev. (m3 X 10m8) | 1.61 | 3.80 | 5.49 | 3 823 |
| Mean of the logs (ln) | 19.61 | 0.40 | 0.50 | 23.47 |
| St. dev. of logs | 0.38 | 0.40 | 0.50 | 0.18 |
| Lag 1 serial correlation | negligible | negligible | negligible | 0.59 (0.601n) |
| Sample size | 38 years (1948 - 85) | 43 years (1948 - 90) | 40 years (1947 - 89) | 61 years (1900 - 60) |
| Nature of drought spells | random at all z0 | random at all z0 | random at all z0 | random at all z0 |

Statistical Analysis of Drought Severities

One way of expressing the truncation level is in the form of standardized index (Gibbs, 1975).  The standardized index, z0 is equivalent to standardized value of the drought variable such as annual streamflow sequence i.e. zi = (xi - xm)/σx, where xm and σx are mean and standard deviation of the annual runoff sequence xi and the zi is the standardized sequence (zi has the mean zero and standard deviation 1).  In general droughts are analyzed with respect to the truncation level equivalent to mean annual flow or zo = 0.  The truncation levels z0 > -0.2 also sometimes mean that no drought conditions exist at least for agricultural purposes (Downing et. al., 1984).  The aforesaid values of truncation levels are applicable to flow sequences which are normally distributed in the probabilistic sense.  For non-normally distributed sequence they should be modified appropriately.

If a times series zi is truncated at a level zo, then the event of surpluses (zi > zo) and deficits (zi ≤ zo) would emerge along the time axis (fig. 2).  The truncation level can be assigned its probability quantile as q = P(zi ≤ zo) where q is the probability of drought corresponding to the truncation level zo and P(..) stands for the notation of probability.  Any uninterrupted sequence of deficits can be regarded as a drought length (duration) equal to the number of deficits in the sequence, designated by L (L = 1,2,3 ... j).  Each drought duration is associated with deficit sum, i.e., sum of the individual deficits in the successive epochs of the spell (Fig. 2) and is designated as S.  This deficit sum S is termed as drought severity.  If the data sequence follows a normal probability distribution (or can be normalized) then in view of the central limit theorem, the deficit sum or the drought severity can be regarded to follow truncated normal distribution (Sen, 1980).  Because of the property of normality, the drought probability quantile q and truncation level zo are related and for a given zo, q can be obtained from the standard normal curve or table.  Values of q are 0.5, 0.42, 0.34, 0,27, 0.21 and 0.16 for zo equal to 0, -0.2, -0.4, -0.6, -0.8 and -1.0.

One can expect n (n = 0,1,2,3....i) drought spells (runs) over a period of T years and correspondingly there will be n values of severities.  Each drought spell has a length L (L = 1,2,3,4...j) years and severity S(S = Y1, Y2, Y3....Y).  A designer is interested in the longest value of L designated as LT and the largest value of S designated as ST.  The period T means a sample size of T (T = 10,20....100) years and can be regarded equivalent to return period of T year (Horn, 1989) for the largest values.  The probabilistic relationships for LT and ST can be obtained by applying the theorem of extremes of a random number of random variables as indicated by Todorovic and Woolhiser (1975).  Pursuing the work of Sen (1980), the following expressions can be written for LT and ST.

where q is the probability quantile defined earlier, r = conditional probability of any year being drought year given that the past year is also a drought year.  Simplification of Eq. (1) can yield E(LT) as follows

The drought lengths have been found to follow the geometric probability distribution well (Sen, 1980;  Bogardi et al., 1988), hence P(LT = j) can be evaluated through the following expression (Sen, 1980).

The conditional probability r is related to the first order serial correlation coefficient through the following equation (Sen, 1977).

where ρ is the first order serial correlation coefficient in the sequence of drought variable and t is the dummy variable of integration.  Values of r for various ρ and q can be evaluated through a numerical integration procedure.  Therefore evaluation of the length of the longest drought over a period of T years is reasonably straight forward using Eqs. (3) and (5).  In order to evaluate E(ST), Eq.(2) can be simplified as

where f(ST) means pdf of ST (as f(x) being pdf of x in the probability theory) which is not certain because pdf of S in Eq.(2) is not well known.  Eq.(6), can, therefore, be solved numerically by first evaluating P(S≤Y) in Eq.(2).  If S is assumed to be normally distributed then

where μ s and σs are mean and standard deviation of deficit sums.  The severity S is made of several deficits (d) in succession.  If deficit (d) is regarded to have a truncated normal distribution with mean μ d and standard deviation σd then μ s and σs can be estimated approximately as (Kotz and Neumann, 1963)

The expression for μ d and σd can be worked out as

Sharma (1994) has arrived at the optimal value of k after extensive simulation experiments as

Note the evaluation of E(ST) is quite involved as it requires the numerical integration of several equations.  Further, Eq.(12) implicitely confirms the observation that E(ST) is positively correlated with E(LT) as is cited in the literature (Woo and Tarhule, 1994).  The critical drought magnitude can be written as

Note for a random process of drought generation r = q in all the above equations.  The actual largest severity (expected value) designated as DT can be expressed as

Eq.(14) can be manipulated as

or

where FT can be names as Drought Frequency Factor and can be written in the equation form as

It can be seen that Eq.(16) is analogous to the flood frequency formula QT = Qm + KT. σQ commonly cited in the hydrologic texts.

It should be noted that all above equations are valid in the normalized domain.  If the data have been logtransformed, then the z0 values used for defining the truncation levels denoted by zL should be computed using the following moment linkage relationships.

where μ L, σL and pL are the mean, standard deviation and lag 1 serial correlation coefficient in the log transformed (normalized) domain.

To illustrate the point, fo a lognormal drought variable with cv = 0.40, the zL to represent the truncation level z0 = -0.2 (if the data had been normal) can be worked out as - 0.024 (note, σL = 0.385, μL = -0.074 and Xm = 1 in the modular form).  Likewise z0 = -0.8 for normally distributed data corresponds to zL = -0.81 for the above lognormal data.  These modified z0's or zL's shold be used to evaluate q, r, E(LT) and E(ST) for respective droughts.  When the drought variable is normally distributed the z0 values defining the desired truncation level should be used in all above equations without alterations.

Results and Discussion

The annual runoff sequences from the four gauging stations fitted the lognormal distribution well. Therefore, the flow sequences were normalized by logarithmic transformation and then standardized for further analysis.  Drought spells were analyzed for truncation levels of 0, -0.2, -0.4, 0.6, -0.8 and -1.0 and simple and first order transitional probability matrices were estimated.  The transitional probability matrices of the first order were tested for equality with the simple probability matrices (Sharma, 1994).  The chisquare statistic (Medhi, 1982) used for the aforesaid hypothesis testing indicated that drought spells from the Tana and Sondu basins evolve randomly whereas those from the Victoria Nile obey the Markovian persistence.  It should be noted that annual flow sequences for the Tana and Sondu basin were also found as random whereas that for the Victoria Nile as autoregressive order 1 with the serial correlation coefficient of 0.59 (Table 1).  The corresponding p in the normalized domain is 0.60.

The model equations were validated by comparing the predictions with the observed values.  The values E(LT) and E(ST) for the time periods (historical database T = 40 and 60 years) were predicted and are graphed in Fig. 3.  The plots speak of the satisfactory correspondence and okay the model for the desired predicitions.  Consequently values of E(LT) and E(ST) for the desired period T = 10, 25, 50, 100, 200 and 500 year were predicted for truncation levels z0 from 0 to -1 and serial correlation p fro 0 to 0.7.  The derived parameter namely the critical drought magnitude MT was also computed as the ration of E(ST) to E(LT) and the values are summarised in Table 2.  The following points emerged from the above calculations.

1.	The drought severity values with reference to a higher truncation level are higher than those for a low truncation level for any return period.  Furthermore above values tend to be larger for the Markovian structure of drought spells in relation to the simple random spells at all truncation levels and return periods, T.  This means if a water storage system is to be designed to cater for the water needs during a drought spell, the designer must use E(ST) values applcable to a higher truncation level, which should be chosed based on the consideration of the sensitivity of the water use system to shortages.

2.	The values of MT (Table 2) display a trend of being close to 1 for the truncation levels near to mean annual flow (z0 = 0 to -0.2) and these values stadily decrease for the low truncation levels.  Furthermore MT values tend to be slightly higher for Markovian droughts that those for random droughts.  One can, therefore, use the simple relatiohsip E(ST) = MT.E(LT) for the estimation of the largest severity.  A conservative value of MT can be taken as 1 for the T values from 10 to 500 years and the p values of the drought variable up to 0.7 for the truncation levels from 0 to -0.2.  Such an approximation may be justified from the consideration of the simplicity and ease in applying the complex relationships for the design.  The important part here is the estimation of E(LT) which is easy and straightforward and requires only 4 parameters viz., mean, coefficient of variation, truncation level (or q) and the lag 1 serial correlation coefficient of the normalized flows.  The analysis uses the classical and well tested models of probability such as Poisson and geometric laws and no empiricism is involved as is the case with the estimation of E(ST).

Table 2.	Values of expected longest drought duration, largest severity and the critical drough magnitude at various truncation levels and return periods

| T year | z0 = 0, q = 0.5 |  | z0 = -0.2, q = 0.42 |  | z0 = -0.4, q = 0.34 |  | z0 = -0.8, q = 0.21 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p = 0 | p = 0.5 | p = 0 | p = 0.5 | p = 0 | p = 0.5 | p = 0 | p = 0.5 |
| 10 | 2.63* | 2.92 | 2.19 | 2.60 | 1.79 | 2.22 | 1.19 | 1.55 |
|  | 2.24 | 2.99 | 1.76 | 2.42 | 1.36 | 1.89 | 0.76 | 1.10 |
|  | 0.85 | 1.02 | 0.81 | 0.93 | 0.76 | 0.85 | 0.64 | 0.72 |
| 25 | 3.97 | 5.39 | 3.24 | 4.52 | 2.63 | 3.79 | 1.78 | 2.76 |
|  | 3.65 | 5.51 | 2.89 | 5.38 | 2.26 | 3.46 | 1.39 | 2.21 |
|  | 0.92 | 1.02 | 0.89 | 0.97 | 0.86 | 0.91 | 0.78 | 0.80 |
| 50 | 4.98 | 7.07 | 4.04 | 5.92 | 3.28 | 5.00 | 2.23 | 3.78 |
|  | 4.61 | 7.28 | 3.65 | 5.82 | 2.87 | 4.64 | 1.84 | 3.12 |
|  | 0.93 | 1.03 | 0.90 | 0.98 | 0.88 | 0.94 | 0.83 | 0.83 |
| 100 | 5.98 | 8.76 | 4.85 | 7.32 | 3.92 | 6.21 | 2.67 | 4.80 |
|  | 5.54 | 9.03 | 4.39 | 7.25 | 3.46 | 5.81 | 2.25 | 4.01 |
|  | 0.93 | 1.03 | 0.91 | 0.99 | 0.88 | 0.94 | 0.84 | 0.84 |
| 200 | 6.98 | 10.43 | 5.65 | 8.72 | 4.56 | 7.41 | 3.11 | 5.82 |
|  | 6.45 | 10.78 | 5.12 | 8.68 | 4.03 | 6.99 | 2.64 | 4.92 |
|  | 0.92 | 1.03 | 0.91 | 0.99 | 0.88 | 0.94 | 0.85 | 0.84 |
| 500 | 8.29 | 12.66 | 6.70 | 10.58 | 5.41 | 9.01 | 3.70 | 7.18 |
|  | 7.64 | 13.10 | 6.10 | 10.58 | 4.78 | 8.55 | 3.16 | 6.12 |
|  | 0.92 | 1.04 | 0.91 | 1.00 | 0.88 | 0.95 | 0.85 | 0.85 |

*First line indicates E(LT), second line E(ST) and the third line MT.

3.	Therefore, in the drought severity formula (Eq.16) one can write E(ST) 6,13 ≈ E(LT) for the design purposes. The drought frequency factor can be written as

FT = [ E(LT) - cv-1](22)

A graph of E(LT) for various time periods T, truncation levels zo (or q) and serial correlation coefficient ρ (or conditional probability r) is depicted in Fig. 4.  The drought frequency factor, FT can be estimated rapidly by consulting E(LT) from Fig. 4 and plugging the value of coefficient of variation of the given river in Eq.(22).  A look at Eq.(22) further tells that the value of the drought severity factor FT is greater for higher values of cv, implying that a greater storage is needed for combating a drought in a region where streamflows or rainfalls display larger variabilities.

Conclusions

A frequency formula for drought severities can be expressed as DT = Xm = FT. σx which is analogous to the flood frequency formula QT = Qm = KT.σQ commonly cited in hydrologic texts.  The drought frequency factor FT is dependent on the expected length of the longest drought spell over a period of T years and the coefficient of variation of the drought variable.  The expected length of the longest drought spell, E(LT) is dependent on the truncation level and the lag 1 serial correlation in the sequence of the drought variable.  The structure of the formula indicates that the drought severity decreases with decreasing truncation levels and increase with increasing variability in the flow sequences.

References

Bogardi, J J, Duckenstein L and Ruhambo, O H (1988).  Practical generation of synthetic 	rainfall event time series in semi-arid climatic zone.  Journal of Hydrology, 103, 357 - 373.

Chow, V T (1951).  A general formula for hydrologic frequency analysis.  Trans. Am. 	Geophysical Union, 32(2), 231 - 237.

Downing, T E, Gitu, K W, Kamau, C M and Barton, J (1984).  Drought in Kenya.  In T 	E Downing, K W Gitu and C M Kamau (editors) Coping with Drought in Kenya:  National and Local Strategies, Lynne Reinner Publishers, London, U.K.

Dracup, J A, Lee, K S, Paulson, E G (Jr) (1980).  On the statistical characteristics of

drought events.  Water Resources Research, 16(2), 289-296.

Gibbs, W J (1975).  Drought, its definition, delineation and effects.  Drought - Special 	Environmental Report No. 5, WMO, Geneva, pp 40.

Haan, C T (1977).  Statistical Hydrology.  Iowa State University Press, Iowa,

USA.

Horn, D H (1989).  Characteristics and spatial variability of droughts in Idaho.  ASCE

Journal of Irrigation and Drainage Engineering.  15(1), 111 - 123.

Kotz, S and Newmann, J (1963).  On the distribution of precipitation amounts for periods 	of increasing length.  Journal of Geophysical Research, 68 (12), 3635 - 3640.

Matalas, N C (1967).  Mathematical assessment of synthetic hydrology.  Water 	Resources Research, 5(5), 967 - 988.

Medhi, J (1982).  Stochastic Processes.  Wiley Eastern Limited, New Delhi, India.

Sen, Z (1977).  Run-Sums of annual flow series.  Journal of Hydrology, 35, 311 -

324.

Sen, Z (1980).  Statistical analysis of hydrologic critical droughts.  ASCE Journal of the 	Hydraulic Division, 106 (HY1):  99 - 115.

Sharma, T C (1994).  Stochastic features of drought in Kenya, East Africa.  In:  K W Hipel 	(editor Stochastic and statistical Methods in Hydrology and Environmental Engineerings, Kluwer Academic Publishers, vol 1, 125 - 137.

Todorovic, P and Woolhiser, D A (1975).  A stochastic model of n day precipitation.

Journal of Applied Meteorology, 14, 17 - 24.

Woo, M K and Tarhule, A (1994).  Streamflow droughts of northern Nigerian rivers.

Hydrological Sciences Journal, 35 (1), 19 - 34.

Yevjevich, V (1972).  Stochastic Processes in Hydrology.  Water Resources

Publications, Colorado, USA.

Figure 1 :	Location map of East Africa for drought study.

Figure 2 :	Definition sketch of drought parameters.

Figure 3 :	Comparison of predicted and observed drought indices in East Africa.

Figure 4 :	Prediction of longest drought duration.