GIS Modelling of Sediment Yields in Semi-Arid Environments

1P D Donald, 2A W van Niekerk & 3C S James

1Division of Water Technology, CSIR, P O Box 395, Pretoria, 0001.

2Centre for Water in the Environment, University of the Witwatersrand, Private Bag 3, Wits, 2050.

3Department of Civil Engineering, University of the Witwatersrand, Private Bag 3, Wits, 2050.

Introduction

The CALibrated SImulation of Transported Erosion (CALSITE) model, is a catchment management tool for simulating soil erosion and sediment yield.  CALSITE makes use of raster Geographical Information System (GIS) techniques to map and display soil erosion and sources of sediment yield within a catchment.  CALSITE was originally developed by, Hydraulics Research; Wallingford, for use in developing countries, where data are not readily available, and results are often required quickly.  The functionality of the model must therefore remain as simple as possible, yet produce reasonably accurate results.

CALSITE was developed initially for use in the Magat basin, Philippines and has subsequently been used in the Upper Mahaweli catchment, Sri Lanka; Khon Kaen, Thailand and the Cameron Highlands, Malaysia (Bradbury, 1994).  It has been applied successfully in these tropical environments for predicting source erosion and sediment yield and modelling the effects of changes in land use.  CALSITE is currently being modified for semi-arid conditions with the Sabie River catchment, Eastern Transvaal being chosen for preliminary application and testing.

CALSITE

The data requirements for CALSITE are:

1.	Mean annual rainfall

2.	Land cover

3.	Soil maps

4.	Elevation data

5.	Streams

6.	Sub-catchment maps if possible

If these data are not available in digital format, they are digitized and then converted to raster format.  A pixel size is determined based on the scale at which the data were captured and on the resolution of the answers required, typically in the order of 150 by 150 metres.  Once these data sets have been prepared, the following operations are performed:

1.	The calculation of source erosion for each pixel

2.	The determination of a delivery index.

3.	The calibration of sediment delivery using observed sediment measurements, or alternatively an estimated delivery ratio based upon empirical equations.

4.	The calculation of sediment yield for each pixel.

Sediment yield is calculated from:

SY = SE * DR					(1)

where SY = sediment yield; SE = soil erosion; DR = calibrated delivery index or delivery ratio.

Source erosion

Source erosion is determined with the use of the Universal Soil Loss Equation (USLE), (Wischmeier & Smith 1978):

Ea = R K SL CP						(2)

where Ea = annual soil loss; R = rainfall erosivity; K = Soil Erodibility; SL = slope length and steepness factors; CP = a factor representing the combination of land cover and land conservation practises.

USLE was originally developed for use in the USA, however a number of adaptations of the variables in this equation have been made for different climatic regions.  One of these adaptations (David, 1982) was used for adapting USLE to tropical conditions.  For semi-arid environments, two options were open for depicting source erosion.  The first was to apply USLE to semi-arid environments with the use of techniques described by C.T. Crosby, P.J. McPhee and A.A. Smithen (1983) and P.J. McPhee and A.A. Smithen (1984).  The alternative was to use the Soil Loss Estimator for Southern Africa (SLEMSA), (Elwell, 1978) developed in Zimbabwe:

SE = RK. RX. RC						(3)

where SE = predicted annual soil loss; RK = mean annual soil loss from a standard, conventionally tilled field plot of size 30m x 10m at a 4.5% slope, for a known erodibility, M, under a weed-free bare fallow management; RX = the ratio of soil loss from a plot of length, L metres, and slope 5%, to that lost from the standard plot; RC = the ratio of soil loss from a cropped plot to that from bare fallow.

The values of RK, RX and RC are given by the following equations:

RK = exp [(0.4681 + 0.7663 M) Ln E + 2.884 - 8.1209 M]		(4)

where E = annual rainfall energy

RX =  L (0.76 + 0.53 S + 0.076 S2			(5)

25.65

where S = Slope steepness

RC = e-0.06i			if i < 50 % for crop and natural grass and for

dense pastures and mulch when i > 50 %	(6)

or	RC = (2.3 - 0.01 i)		if i > 50 % for crop and natural grass		(7)

30

where i = percentage of rainfall energy intercepted by cover

SLEMSA was developed as an adaptation of USLE and has similar factors to USLE, however the values attributed to the factors and the calculation of the factors is different and produces a different result. CALSITE uses soil erodibility, rainfall erosivity, slope steepness, slope length and cover factor raster images for calculation of the SLEMSA equation.

Delivery index

The delivery index is calculated from (Bradbury, 1994):

Di = Lc   						(8)

SE/Q

where 		Lc = Pax min [Fx Sy]					(9)

and			Q =  Paz						(10)

Di = delivery index; Lc = Limiting transporting capacity along the flow path from a particular pixel; Q = potential runoff; SE = source erosion; Pa = total annual rainfall;     F = flow parameter (These flow paths are calculated by the aspect driven routing algorithm, Lea, 1992.); S = Channel slope; x, y and z are coefficients.

The equation thus becomes:

DIp = Paz+z(x-1) min [Fx-1 Sy]			(11)

SE

Appropriate coefficients are required for the equation.  The x and y coefficients can be obtained from research done by Govers (1990).  He undertook extensive experimental  studies of sediment transport under conditions of sheet flow in a sloping laboratory channel containing sediment of different sizes.  A table of values was produced for various D50 grain sizes.  Atkinson (1991) analyzed soils data for the Magat catchment and concluded that the d50 grain size was approximately 20µm.  The values therefore for x = 1.66 and y = 1.44. Preliminary investigation on a over 100 soil samples taken throughout the Sabie River catchment has suggested that the D50 grain size is closer to 500µm.  This suggests values of approximately x = 1.2 and y = 1.7 are more appropriate for the Sabie River catchment, as implemented in the current version of CALSITE.  For the initial study on the Sabie River catchment, it was decided to keep the values determined by Atkinson (1991).  Future analysis of the soils data will determine more appropriate values for Sabie River Catchment.

Studies done by Amphlett and Dickenson (1989) on the relationship between rainfall and runoff indicate values for z = 1.66.  This value was derived for gently sloping, relatively low rainfall, conditions.  In order to take steeper slopes and higher rainfalls into account, the exponent was arbitrarily reduced to 1.4 (Bradbury, 1994).  It was again decided to keep the value of z = 1.4 for this study.  Future research into the relationship for the Sabie River catchment may reveal a different value.

The final equation thus becomes:

DIp = Pa2.32 min [F0.66 S1.44]			(12)

SE

Calibration of sediment delivery

The calibration procedure is based on the assumption that if predicted and measured values of sediment delivery do not match each other then it is the delivery ratio value that is incorrect.  In reality it is probably the predicted source erosion that is incorrect (Bradbury, 1994).  This assumption of an incorrect delivery ratio is made for the preliminary Sabie study since no measured source erosion values are available.

The calibration process uses either observed sediment measurements taken from within the catchment or alternatively, if these data is not available, an estimated delivery ratio based upon empirical equations, may be used.

The following processes are carried out in the calibration procedure.  Firstly the predicted delivery index is converted to a delivery ratio based on the graph in figure 1 (Bradbury 1995).  This converts the delivery index values between 0 and 255 to delivery ratio values between 0 and 1, where 0 = all source erosion is deposited and 1 = all source erosion is transported.  The delivery ratio values determined from the graph are used to calculate a sediment yield (Eqn 1).  These predicted sediment yields are then compared against the actual values observed at specific locations in the catchment.  A scaling constant (k) is used to adjust the delivery index, and the sediment yields are recalculated. This process continues iteratively for all pixels until the best fit between the predicted and observed yields is found.  The value of k which produces the closest results to those measured is applied to each pixel across catchment in the final calculation of sediment yield.

Figure 1 	Graph for converting delivery indexes to delivery ratios.

In the application of CALSITE to the Sabie river catchment, current information on actual sediment observations is limited. The following equation (Williams and Berndt, 1972) which was developed for a semi-arid region in Texas, USA, was used to determine the delivery ratio:

Dr = 0.627 SLP0.403					(13)

where Dr = delivery ratio; SLP = % slope of main stream channel

The % slope of the main channel was calculated from the elevation image for the catchment.  The calculated delivery ratio value was then input into CALSITE for the calibration process to continue.  An alternative calibration process needs to be addressed for the Sabie River catchment, whereby measured results can be used for the calibration.  A problem that exists for this catchment is the lack of suitable reservoir surveys, hence another method for calibrating CALSITE will have to be determined.

Preliminary application to the Sabie River catchment

Elevation data were obtained from the national elevation model, produced by the Surveyor General. These data are at a resolution of 200 metres in mountainous regions and 400 metres for the flatter areas.  It was decided to combine and interpolate these data sets to a 300 metre resolution.  This interpolated elevation model was then used as the basis for determining the pixel resolution for all the images.  Soil erodibility information was deduced with the use of the land type maps generated by the Department of Agriculture, and the use of the USLE nomograph for soil erodibility values.   The average percentages of clay, silt and sand were derived from the land type maps.  This information was then used as inputs into the USLE nomograph, Wischmeier W.H. & Smith D.D. (1978), which provided a soil erodibility factor for each land type.  Land cover information was obtained from interpretation of LANDSAT TM imagery, compiled by Forestek and Watertek, CSIR.  The dataset was classified into 14 different classes including Forestry (primary and secondary), grasslands and degraded areas.  Mean annual rainfall and rainfall erosivity maps were obtained from Computer Centre  for Water Research (CCWR), Natal University.  River and sub-catchment data were gathered from the catchment study conducted on the Sabie River Basin (Chunnet et al, 1990).  All the datasets were clipped according to the catchment boundary, and standardised at the 300 metre resolution.

Both the USLE and SLEMSA methods for source erosion prediction were tested on CALSITE.  The values for USLE being input according to guidelines suggested by  C.T. Crosby, P.J. McPhee and A.A. Smithen (1983) and P.J. McPhee and A.A. Smithen (1984).  The values for SLEMSA were obtained from the Natal Agriculture Research Bulletin (1976).  Figure 2 represents the results of the USLE determined source erosion values, and figure 3 represents those obtained with the aid of the SLEMSA equations.

The source erosion values predicted by SLEMSA are far greater than those for the USLE.  This is accentuated in the regions where steeper slopes occur particularly in the upper third of the catchment.

Figure 2	Source erosion as determined with the use of USLE,

for the Sabie River catchment

It appears that SLEMSA predicts excessively high values of erosion for relatively steep slopes.  USLE predicted most of its source erosion values under 500 tons/km2/annum with only 2897 or 2.7% of the total pixel values were greater than 500 tons/km2/annum.  SLEMSA however predicted 24927 pixel values, 22.9% above 500 tons/km2/annum.  For the preliminary investigation it was thus decided to use USLE, to determine the source erosion values.

Figure 3	Source erosion as determined with the use of SLEMSA,

for the Sabie River catchment

The final sediment yield predicted by CALSITE is represented in figure 4.  As a first evaluation of the models effectiveness at predicting sediment yields, CALSITE predictions were compared with the results obtained by Van Niekerk and Heritage (1994) in there modelling of sediment yields in the Sabie River catchment.

Figure 4	Sediment yield, for the Sabie River catchment, as determined using the USLE calculation to predict source erosion

| River Catchment | Van Niekerk & Heritage, annual yields (Tons/Km2) | CALSITE, annual yields (Tons/Km2) |
| --- | --- | --- |
| Upper Sabie | 327 | Most pixels in 100 - 150 range with smaller % falling in the 150 - 200 range |
| Maritsane | 345 | Most pixels < 50 with smaller % falling in the 50 - 150 range |
| Noord Sand | 402 | In the steeper slopes pixel values are > 400 with the less steep slopes producing results in the 50 - 100 range |
| Middle Sabie | 378 | Most of the region falls into < 50 range except in areas which show up as highly degraded on the landcover image, where values are 300 - 400 and sometimes > 400 |
| Sand | 401 | The highest proportion of pixels are in the 200 - 300 range with the steeper slopes having 300 - 400 values, especially in degraded areas. Lower slopes are < 50 |
| Lower Sabie | 246 | All values are < 50 |

Table 1 	Estimated sediment yield of Van Niekerk and Heritage (1994), compared to CALSITES values

The sediment yield map of Rooseboom et al. (1992) gives an estimated catchment yield value of  155  t/km2.a. For small subcatchments of the order of 800 km2 the predicted yields are approximately 40 t.km2.a within the 20% confidence band t/km2.a within the 50% confidence band and 360 t./km2.a within the 80% confidence band.

Conclusions

At this stage the model has proved useful in using quantitative techniques to provide qualitative comparisons of sediment yields.  The actual quantitative results produced need to be thoroughly evaluated, in a semi-arid environment before they can be used with any

confidence.

The ability to predict sediment yield values per pixel has the advantage above methods which predict values per sub-catchment in that, as can be seen by CALSITE's results, it is not always the full catchment that is causing a sediment problem.  Specific areas within a sub-catchment can therefore be focused on for needing attention.  If landuse is to be changed within a sub-catchment, it is only necessary to reevaluate the pixels affected by the landuse change, rather than the whole sub-catchment.

There are clearly shortfalls in both the USLE and SLEMSA source erosion, and delivery ratio prediction methods.  There are also shortfalls, in that neither gully erosion nor channel erosion are determined in the model.  Further research also needs to be conducted into the determination of the delivery function for the Sabie River catchment. The variables for the delivery index equations need to be determined, and alternative calibration methods need to be sought, to make the model better suited to semi-arid conditions.

Given these shortfalls, however, the model can still be used as an effective tool for catchment management, in terms of evaluating the affects of changing land use, and the relative sediment yields produced by different scenarios.

Acknowledgements

The authors are grateful to HR, Wallingford for the use of CALSITE and for collaborating in the adaptation of CALSITE to semi-arid conditions.

References

Amphlett M.B. and Dickenson A. (1989). Dallao soil erosion study, Magat, Philippines.

Summary Report (1984-1987). Report No. OD 111, HR Wallingford, in collaboration with the National Irrigation Administration, Government of the Philippines.

Atkinson, E. (1992). Sediment delivery in river systems. OD/TN 48. ODU HR 	Wallingford, UK.

Bradbury P.A. (1994). A Catchment Planning Tool for Simulating Soil Erosion and Sediment 	Yield. Report No. OD/P 122, Overseas Development Unit, HR Wallingford, UK.

Bradbury P.A. (1995). CALSITE VERSION 3.1, Calibrated Simulation of Transported 	Erosion, user manual. Overseas Development Unit, HR Wallingford, UK.

Chunnet, Fourie and Partners. (1990).  Water resources planning of the Sabie River 	catchment.  SA Dept Water Affairs report no. P.X.300/00/0490, vols 1 to 10.

Crosby C.T., McPhee P.J. & Smithen A.A. (1983). Introduction of The Universal Soil Loss 	Equation in the Republic of South Africa. Presentation to the American Society of Agricultural Engineers.

David W.P. (1982).  Erosion and Sediment Transport. Seminar Paper. College of 	Engineering and Agro-Industrial Technology. UPLB College, Laguna, The Philippines.

Elwell H.A. (1978). Soil Loss Estimation: Compiled works of the Rhodesian multi

disciplinary team on soil loss estimation.

Elwell H.A. (1979). Modelling Soil Losses in Zimbabwe Rhodesia. Presentation to the

American Society of Agricultural Engineers and the Canadian Society of Agricultural Engineering.

Elwell H.A. (1984). Soil Loss Estimation: A modelling Technique, In: Hadley R.F. and

Walling D.E. (eds), Erosion and Sediment Yield, Some methods of measurement and modelling. Geo Books, Norwich, England.

Govers, G. (1990). Empirical relationships for the transport capacity of overland flow.

Erosion, Transport and Deposition Processes, (Proc. of the Jerusalem Workshop, March-April, 1987) IAHS Publ. No. 189.

McPhee P.J. & Smithen A.A. (1984). Applications of the USLE in the RSA.  Source

Unknown.

Natal Agriculture Research Bulletin No. 7: Soil Loss Estimator for Southern Africa,

(1976).  Dept Agricultural Technical Services, Natal University.

Rooseboom A., Verster E., Zietsman H.L. and Lotriet H.H. (1992). The development of 	the new sediment yield map of Southern Africa.  WRC report no. 297/2/92

Wischmeier W.H. & Smith D.D. (1978). Predicting rainfall erosion losses: a guide to

conservation planning. USDA-SEA Agric. Handbook 537, Agriculture Research Service, USDA, Washington DC, USA.

Van Niekerk A. and Heritage G.L. (1994). The use of GIS techniques to evaluate 	sedimentation patterns in a bedrock controlled channel in a semi-arid region.  In: Proceedings, Int conference of Integrated Basin Management, Wallingford, England.