The Application of a Daily Surface-Groundwater Model to Simulate Low Flows

K Sami, V Y Smakhtin & D A Hughes

Institute for Water Research

P O Box 94

Rhodes University

Grahamstown 6140

Introduction

The characterisation of a river's low-flow regime is of great importance in determining its waste assimilation capacity and ecological flow requirements. However, the assessment of low flows in South Africa is severely hampered by the scarcity of good quality observed records of a sufficiently long duration, and by land use changes which introduce temporal trends into observed data. To extend the area which can be characterised beyond adequately gauged sites, and to simulate past natural and future low-flow conditions, daily time-series simulations are required.

Because low-flow characteristics can vary widely according to the relative importance of several distinct surface-subsurface interaction processes, it is important that the model employed be flexible enough to adequately conceptualise and simulate each of the processes involved.  These include the generation of baseflow from the intersection of the regional groundwater table with the surface, the lateral drainage of deep soil profiles resulting in re-emergence form a saturated seepage face, as well as the re-emergence of percolating water as springs from fracture systems located above the regional groundwater body.

The weathered and fractured bedrock zone between the soil and groundwater greatly influences these processes in semi-arid and in mountain catchments, however, it has traditionally been neglected by both surface and groundwater models. The few rainfall-runoff models which have attempted to forge a link between the soil zone and groundwater have attempted to interface with existing finite element or finite difference groundwater models (e.g. Chiew et al., 1992), whose parameters are difficult to quantify spatially in fractured aquifers with scarce hydraulic data. Alternatively they have been dependant on parameters which are not easily physically quantified (e.g. Arnold et al., 1993), such as empirical power functions. These approaches have also ignored the significance of preferential pathways on recharge when large soil moisture deficits exist,  and the role of bedrock fracture density in limiting recharge even when partial areas of the soil exceed field capacity.

Surface-subsurface interaction algorithms to simulate the above mentioned baseflow generation processes have been incorporated into the semi-distributed Variable Time Interval (VTI) Model (Hughes & Sami, 1994).  The model has been applied to four perennially flowing South African headwater catchments of different low-flow response in order to test the ability of these algorithms for simulating low-flows. Two catchments are located in the Sabie system, one is part of the Tugela system in KwaZulu-Natal and the other is part of the Diep river of the Southern Cape.

Since conventional goodness-of-fit criteria are based on how well observed hydrograph shapes, flood peaks or runoff volumes are reproduced, they emphasise storm runoff and provide relatively little information on the success of low-flow regime simulations.  Therefore, simulations are evaluated using low-flow characterisation indicators. These include comparisons of observed and simulated daily flow duration curves, hydrograph recession rates, the frequency and duration of low-flow spells below a reference discharge, the frequency, magnitude and duration of dry season freshes, and total baseflow volumes.

The Variable Time Interval model (VTI)

The theory and structure of the model has been described elsewhere (Hughes & Sami, 1994) and are only summarised here. Four functions link surface and groundwater. These simulate: drainage from a lower soil layer into a percolating storage which conceptually represents water in transit to the water table; the re-emergence of percolating water as springs above the regional water table due to structural controls or a geological impedance; piezometric surface dynamics resulting from changes to aquifer storage due to increments from percolating storage, lateral transfers and borehole abstraction; and groundwater seepage from the intersection of the water table with the surface.

The soil profile is divided into two layers whose moisture contents are represented by Normal distributions. The distributions are defined by a standard deviation parameter whose value varies depending on the current soil moisture content. The portion which may contribute to saturated area runoff and soil baseflow is equivalent to the proportion of the soil zones which are saturated, while the portion of the sub-area which may potentially contribute to recharge is assumed equivalent to the proportion of the lower soil zone distribution in excess of field capacity. To account for the spatial position of such areas, lateral and vertical distribution factors are calculated for each time step.

Recharge is controlled by the proportion of the lower layer above field capacity and an adjusted hydraulic conductivity parameter (KG) defined as:

KG (mm h-1) = TM / DEPTH * S * 1000/24	(1)

where DEPTH is aquifer thickness (m). The use of storativity to adjust KG accounts for the fact that some of the potential recharge area may be underlain by unfractured rock or rock not hydraulically connected to the main groundwater body. This adjustment has been found to be particularly necessary in fractured rock situations, where the opportunity for drainage from even saturated soils is limited by the occurrence of surface fractures.

Water leaving the lower soil layer as recharge increments the percolating groundwater store (PSTORE). Water in PSTORE is considered to be evenly distributed throughout the area above the water table (fig. 1). Water is transferred from PSTORE either as increments to the water table, transfers to adjacent sub-area PSTORE's, or as springflow directly into the stream channel. The fraction of PSTORE which may re-emerge as springs (PROP) is determined by the proportion that lies above the groundwater drainage vector (DVECT) line which intersects the sub-area surface (fig. 1). DVECT represents the resultant of the lateral and vertical components of percolation. For example, drainage through an unconsolidated layer would be represented by a large vector slope (e.g. > 20), whereas lateral flow over a gently dipping impermeable horizon would have a low vector slope. For re-emergence to occur the drainage vector must therefore be less than the mean catchment slope. The rate of emergence depends on the PROP fraction, the degree of saturation in PSTORE and KG. Increments to the aquifer are calculated from the fraction of PSTORE which lies below DVECT (PGWATER) and KG, however, an anisotropy adjustment corrects KG using DVECT to account for lower conductivities in the vertical direction in structured formations.

Drainage out of aquifer storage can occur as flow to other sub-areas, groundwater abstraction or as baseflow if the water table intersects the surface. The seepage rate is calculated using Darcy's equation, with the saturated 'seepage face' set equal to the length of the intercept between lines representing the hydraulic gradient and the catchment slope (fig. 1). The conductivity is assumed to be the mean of KG and the soil hydraulic conductivity while the gradient varies exponentially between the mean channel and catchment slopes according to the length of the seepage face.

The Study Catchments

Sabie1 (Gauged catchment X3H001) and Sabie11 (X3H011): The Sabie catchments are located in a mountainous region of Eastern Transvaal. They receive relatively high summer rainfall and are largely afforested by commercial timber plantations. Rainfall is primarily convective in nature, with large events occurring from the convergence of tropical air masses. A strong orographic effect results in considerable variation in MAP across each catchment (1000-1600 mm). Sabie1 is underlain by gently dipping highly jointed shales and quartzites of the Timeball Formation in its headwater mountain region and by highly soluble dolomites, cherts and shales of the Chuniespoort Group in its lower reaches. A significant tract of alluvium is also present. Soils are well to excessively drained but are significantly deeper on dolomite substrate. Hydrological processes vary with geology across the catchment. A line of perennial springs and waterfalls is associated with the resistant quartzite layers high up the slopes. Below these waterfalls significantly less surface runoff is generated from the dolomite regions. However, baseflow is sustained by the intersection of the water table with the surface as a valley bottom spring line. A re-emergence of bank storage from the alluvium is also thought to occur.

Sabie11 is underlain by porphyritic gneissose granite of the Nelspruit suite. Its headwater region consists of a cliff line of predominantly interbedded quartzite, shale, and sandstone of the Wolkberg Group. The topography is sharply dissected with steep slopes and frequently exposed granite domes. Soils are well to excessively drained and are greater than 1 metre in depth. Hydromorphic soils develop in bottomlands due to the accumulation of localised aquifers in the underlying weathered granite. Baseflow is maintained by the re-emergence of groundwater from these valley bottom aquifers.

Diep (K4H003): The Diep river catchment is located in the Outeniqua mountains of the southern Cape and is covered by natural bush and managed forest plantations. Precipitation originates from the advection of moist maritime air or from the passage of cold fronts. In general, rain events are of low intensity and long duration but may be punctuated by high intensity cells. The MAP is approximately 700-900 mm, however, a complex pattern of orographic effects and rain shadows makes it difficult to quantify. The area is underlain by highly folded, steeply dipping to overturned beds of quartzite, shale and tillite of the Table Mountain Group. Soils are relatively shallow and sandy in nature. Subsurface piping also plays a significant role in their hydrologic response. A multitude of structurally controlled springs and seeps on the hillslopes maintain baseflow throughout the year.

Mooi (V2H002): The Mooi River drains the Drakensberg escarpment and flows to the Tugela through well incised valleys. The catchment is underlain primarily by Beaufort Series sandstones and mudstones and covered by highland sourveld. These have a low permeability thus percolation to groundwater is restricted. This results in frequent surface saturation, reflected as vleis and wetlands in topographic lows. The MAP varies from 800-1300 mm across the catchment with a weighted areal mean of 986 mm. Precipitation occurs predominantly in the summer months, with a greater proportion of long duration low intensity events than elsewhere in the Tugela system. Soils are deep and of a clay loam to clay texture but with high infiltration rates and permeabilities.

Results and Discussion

Summaries of the different sources of baseflow in each catchment are given in table 1. A visual comparison of observed and simulated hydrographs (figs. 2-5) demonstrates that the model is capable of generating excellent simulations of daily runoff in terms of peaks, hydrograph shape. This is substantiated by the calculated fit statistics (table 2). Fit statistics are also given for the log of discharge values since this places a greater emphasis on low flow values. Diep exhibits the least successful calibration. This is strongly related to the difficulties experienced in quantifying rainfall inputs. A low coefficient of efficiency for Sabie11 is attributed to peaks which exceed the gauging weir capacity 0.5% of the time. Observed versus simulated flow volumes were 566.75 Mm3 versus 577.59 Mm3 for Sabie1, 571.43 Mm3 versus 593.24 Mm3 for Sabie11, 2574.5 Mm3 versus 2637.0 Mm3 for Mooi, and 131.33 Mm3 versus 157.75 Mm3 for Diep. Given these observations, it could be concluded that the simulations were very successful hydrologically over a range of catchment types. However, such measures do not place sufficient emphasis on how well the model simulates surface-subsurface interactions and their impact on low flows. In addition, they provide limited information on the temporal distribution of low flows.

Table 1. Simulated mean annual runoff from subsurface pathways.

|  | Sabie11 | Sabie12 | Sabie111 | Sabie112 | Mooi2 | Diep2 |
| --- | --- | --- | --- | --- | --- | --- |
| Simulation period | 1979-89 | 1979-89 | 1979-89 | 1979-89 | 1972-82 | 1962-76 |
| Rainfall (mm) | 1250 | 1058 | 1222 | 774 | 856 | 773 |
| Dominant geology | shale/qtz. | dol./alluv. | gran. | gran./alluv. | ms/ss | TMS |
| Soil baseflow (mm) | 51 | 11 | 119 | 15 | 22 | 50 |
| Spring baseflow (mm) | 148 | 0 | 146 | 0 | 0 | 45 |
| G'water baseflow (mm) | 146 | 232 | 33 | 90 | 27 | 24 |
| Sim. runoff (mm) | 358 | 246 | 374 | 102 | 228 | 148 |

1 = Headwater sub-area; 2 = Catchment outlet sub-area

Figure 2. Observed and simulated hydrographs for Sabie 1, 1979-82.

Figure 3. Observed and simulated hydrographs for Sabie 11, 1979-82.

Figure 4. Observed and simulated hydrographs for Diep, 1963-66.

Figure 5. Observed and simulated hydrographs for Mooi, 1973-76.

Table 2. Fit statistics of observed against simulated runoff.

|  | Sabie1 |  | Sabie11 |  | Mooi |  | Diep |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Area (km2) | 174 |  | 212 |  | 937 |  | 71 |  |
| Simulated peak runoff (m3 s-1) | 37.77 |  | 107.29 |  | 179.97 |  | 73.47 |  |
| Observed peak runoff (m3 s-1) | 35.97 |  | 28.901 |  | 306.90 |  | 45.55 |  |
| Slope | 0.82 |  | 0.51 |  | 1.11 |  | 0.70 |  |
| Slope - log values | 0.90 |  | 0.94 |  | 0.94 |  | 0.89 |  |
| Intercept | 0.31 |  | 0.87 |  | -0.49 |  | 0.06 |  |
| Intercept - log values | 0.05 |  | 0.04 |  | 0.09 |  | -0.38 |  |
| r2 | 0.81 |  | 0.67 |  | 0.72 |  | 0.67 |  |
| r2 - log values | 0.87 |  | 0.83 |  | 0.87 |  | 0.64 |  |
| Coeff. eff. | 0.77 |  | 0.07 |  | 0.71 |  | 0.55 |  |
| Coeff. eff. - log values | 0.86 |  | 0.82 |  | 0.86 |  | 0.61 |  |
|  | Obs. | Sim. | Obs. | Sim. | Obs. | Sim. | Obs. | Sim. |
| Mean flow (m3 s-1) | 1.81 | 1.83 | 1.84 | 1.88 | 9.45 | 8.95 | 0.28 | 0.33 |
| Mean flow (m3 s-1)-log values | 0.35 | 0.34 | 0.23 | 0.21 | 1.44 | 1.43 | -2.13 | -1.97 |

1 Gauging weir capacity

Since low-flow generation processes are predominant during hydrograph recessions, an analysis of recessions would highlight the success with which a model simulates these processes. The distribution of recession ratios, which are the ratios of present day discharge to discharge the preceding day when discharge is below mean daily flow, are shown in fig. 6. The percentage of observed recession ratios below 0.94 are 9%, 29%, 46% and 47% respectively for Sabie1, Sabie11, Mooi and Diep (N = 1511, 1654, 1598, 2829). The corresponding figures for simulated runoff are 3%, 10%, 29% and 16% (N = 2261, 2096, 2065, 4012). Sabie11 is not shown in figure 6 since its recession response is similar to Sabie1. Klaassen & Pilgrim (1975) found the range of recession constants to be 0.2-0.8 for surface runoff, 0.7-0.94 for interflow and 0.93-0.995 for groundwater. Recession ratios normally fluctuate within the same ranges. An example of simulated soil baseflow recession ratios (fig. 7) shows that they vary within the correct range, with more than 70% lying between 0.7-0.94. Thus the deficiency of ratios below 0.94 cannot be attributed to a systematic conceptual error in the model. It may, however, indicate an insufficient duration or frequency of simulated soil water baseflow and surface runoff dominated recession. A low frequency of such events is exhibited by the lack of small scale flow variations and minor events in the simulated hydrographs (figs. 2-5) and is reflected by the much larger number of recession days in the simulated data (higher number of recession ratio values). Since these minor events are characterised by rapid decays of their recession limbs, their undersimulation is the primary cause of the paucity of low recession ratios. Other fluctuations in observed low response affecting recession ratios can be attributed to variations in river abstraction.

The poor simulation of minor flow fluctuations can be observed in seasonal hydrographs (fig. 8). An exception occurs in Mooi where two simulated events appear for which there is no observed data. Winter freshes are poorly reproduced in Sabie1 but are better simulated in Mooi. This is attributed to differences in role of subsurface runoff in the hydrological regime of each catchment and the model's conceptualisation of these processes. Subsurface runoff in the model is driven by mean soil moisture content, small inputs of rainfall to sub-area would have a minor impact on mean soil moisture, especially where soils are deep, and would therefore be poorly reflected as increased runoff generation. Similar rainfall inputs, however, would generate substantially more runoff in a regime where saturation or infiltration excess runoff play a dominant role. Since Sabie1 has a hydrological regime strongly driven by subsurface processes, which contribute 95% of runoff (table 1), whereas Mooi is a predominantly surface driven regime with subsurface water contributing only 22% of runoff, Sabie1 shows a poorer simulated response to small events and dry season freshes are frequently not simulated.

The differences between the hydrological regimes, between observed and simulated hydrographs can also be a comparison of baseflow regimes. The baseflow index, or the ratio of baseflow to total flow, was determined from hydrograph separations using a digital filter (Smakhtin & Hughes, 1993; Nathan & McMahon, 1990). Results are shown in table 3. They indicate that Sabie1 is the most baseflow driven catchment and Diep the least. The larger errors in simulated baseflow index for Sabie11 and Diep indicate that the simulated origin of runoff may be faulty, such that runoff from subsurface sources is overestimated. Hence, the calibration may be 'correct for the wrong reasons'. Alternatively, it may indicate that the runoff response from subsurface pathways does not react rapidly enough to rainfall and does not contribute to storm hydrographs.

Table 3. Baseflow characteristics of Sabie1, Sabie11, Mooi and Diep.

|  | Sabie1 |  | Sabie11 |  | Mooi |  | Diep |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Obs. | Sim. | Obs. | Sim. | Obs. | Sim. | Obs. | Sim. |
| Baseflow index | 0.58 | 0.61 | 0.47 | 0.55 | 0.33 | 0.34 | 0.31 | 0.36 |
| Baseflow index % error | 5.2 |  | 17.0 |  | 3.0 |  | 16% |  |
| Mean daily baseflow (m3 s-1) | 1.05 | 1.12 | 0.88 | 1.07 | 3.07 | 2.83 | 0.09 | 0.12 |

The rate of response of subsurface pathways can be evaluated from the distribution of recession ratios. All four catchments also exhibit surpluses of simulated recession ratios above 0.98. Although this also may be partly attributable to poor simulation of minor events resulting in prolonged periods where low flows are controlled by groundwater, it implies that the groundwater baseflow function decays too slowly. For example, 934 out of 1511 recessions in Sabie1 are greater than 0.98, while the corresponding figure for simulated data is 1910 out of 2261 recession days. If the extra 750 recession days can be attributed to periods where minor flow events were not simulated and all simulated recessions during such time were groundwater controlled and these are subtracted from the simulated recession figure, then 226 excess recessions remain above 0.98, demonstrating that the rate of simulated groundwater recession is too slow. This can be attributed to a conceptual limitation of the groundwater intersection function, which simulates the re-emergence of groundwater from a regional water table. It therefore does not account for baseflow contributions from perched aquifers which may recede at a much higher rate after storm events. This may be especially significant in alluvial systems where bank storage, recharged during high flow events, results in localised ridges of higher hydraulic conductivity (Sklash & Farvolden, 1979) which contribute to baseflow as flow recedes. The hydraulic characteristics of such a system are likely to be considerably different than the regional aquifer. For example, hydraulic gradients near the river bank are likely to be much higher than the channel gradient (assumed to represent the regional gradient), and hydraulic conductivities are likely to be much higher than those of the underlying bedrock. It is therefore necessary to incorporate the dynamics of such systems into the surface-subsurface interaction functions if low-flow simulations are to be improved.

To assess the ability of the model to simulate the frequency of occurrence of low flows of differing extremity observed and simulated daily flow duration curves can be compared. These have been standardised by dividing daily flow by mean daily flow (fig. 9), with the exception of Sabie11 which was almost identical to Sabie1. With the exception of Diep, where low flows are appear to be overstimulated, it would appear that an adequate simulation has been achieved. However, duration curves are sequence independent, hence provide little information on the temporal continuity of flows below a specified threshold. For ecological and water quality purposes, what is required is an appreciation of the length of flow spells below a specified threshold.

Figure 9. Standardised flow duration curves for Sabie1, Mooi and Diep.

Durations of observed and simulated spells below Q75 (flow exceeded 75% of the time) are shown in figure 10. Simulated flows appear to generate too few short duration spells and too many long duration spells. This is attributed to the lack of small scale flow variations in the simulated hydrographs, which often break up long spells into several shorter ones. Since the length of spells may be critical for ecological functions, the difficulty of simulating small scale flow fluctuations may be a significant deficiency in the simulation approach. Only in the Mooi are spells adequately simulated. This success may be a consequence of the greater seasonality of the rainfall rather than the success of the model since few flow fluctuations or events can be observed in the dry winter months. Alternatively it may be related to the larger

catchment size, which cause many of the smaller events generated in the headwater regions to be attenuated.

For management purposes it is necessary to forecast expected durations and flow deficits of critical low flows. Although the simulation periods are rather short for meaningful statistical analyses, these have been performed for 5 year return period of low-flow spells to illustrate some of the differences which  occur between observed and simulated flows. Table 4 shows the expected duration of spells below thresholds of 75%, 50% and 40% of long-term mean daily flow and the flow deficits which would have to be overcome to maintain flow at these thresholds. The difference between observed and simulated values is illustrated in figure 11. The simulated results commonly overestimate the duration of spells, except for Mooi where observed and simulated spell durations are within 20 days of each other. This success can be attributed to the lack of minor flow fluctuations and the strong seasonality of the Mooi regime.

Table 4. Duration of spells below a threshold of 75%, 50% and 40% of long-term mean daily flow with a

return period of 5 years and associated flow deficits below these thresholds.

| Threshold |  | Sabie1 |  | Sabie11 |  | Mooi |  | Diep |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | obs. | sim. | obs. | sim. | obs. | sim. | obs. | sim. |
| 75% of ADF | Duration (d) | 198 | 224 | 171 | 208 | 181 | 163 | 91 | 178 |
|  | Deficit flow (Ml) | 10427 | 11699 | 12416 | 14316 | 83446 | 69233 | 1279 | 2501 |
| 50% of ADF | Duration (d) | 121 | 167 | 116 | 138 | 133 | 150 | 83 | 136 |
|  | Deficit flow (Ml) | 2282 | 3698 | 3880 | 4608 | 40268 | 35976 | 607 | 1024 |
| 40% of ADF | Duration (d) | 64 | 109 | 69 | 117 | 128 | 139 | 77 | 123 |
|  | Deficit flow (Ml) | 915 | 1269 | 1718 | 2529 | 28762 | 24934 | 393 | 588 |

Figure 11. Errors in simulated durations of spells of 75%, 50% and 40% of average daily

flow with a return period of 5 years.

Conclusions

Good visual hydrograph fits were obtained between observed and simulated hydrograph peaks and shapes, with high r2 values and coefficient's of efficiencies in all four study catchments. This would be indicative of successful simulations. However, when the simulations were interpreted using the low-flow analytical methods the utility of the simulations is less than would appear. An analysis of recession rates suggests that too few fluctuations in the long term recession response were simulated due to a poor simulation of small dry season events and an inability to simulate fluctuations in low flows resulting from variations in daily river abstraction. The model was found to adequately reproduce soil water baseflow recessions, however, groundwater recessions appeared to be undersimulated. Conceptual deficiencies were identified in the model which could affect the distribution of simulated recession rates in catchments driven primarily by subsurface processes and those where a rapid groundwater response plays a significant role. In the latter case simulated groundwater contributions may not decay rapidly enough.

Although a comparison of flow duration curves suggests that the frequency and severity of low flows was well simulated, an analysis of the duration of low-flow spells indicated that the model overestimates the duration of these spells. This occurred due to the poor simulation of freshes which break up extended dry season low-flow spells.

The inability of daily flow simulations, even those considered to be excellent by conventional hydrological analyses, to reproduce important low-flow characteristics may be of critical importance for ecological or water quality purposes. These findings suggest that model simulations need to be evaluated by a variety of low-flow performance indicators not commonly considered by water engineers before being utilised for water quality or ecological applications.

Acknowledgements

The authors gratefully acknowledge the Water Research Commission for financial support.

References

Arnold, J.G., Allen, P.M. & Bernhardt, G. (1993). A comprehensive surface-groundwater 	model. J. Hyd., 142, 47-69.

Chiew, F.H.S., McMahon, T.A. & O'Neill, I.C. (1992). Estimating groundwater recharge 	using an integrated surface and groundwater modelling approach. J. Hyd., 131, 151-186.

Hughes, D.A. & Sami, K. (1994). A semi-distributed, variable time interval model of

catchment hydrology-structure and parameter estimation procedures. J. Hyd., 155, 265-291.

Klaassen B. & Pilgrim, D.H. (1975). Hydrograph recession constants for New South Wales 	streams. Inst. Eng. Civ. Eng. Trans., CE17, 43-49.

Nathan, R.J. & McMahon, T.A (1990). Evaluation of automated techniques for baseflow and 	recession analysis. Water Res. Res., 26 (7), 1465-1473.

Sklash, M.G. & Farvolden, R.N. (1979). The role of groundwater in storm runoff. J. Hyd., 	43, 45-65.

Smakhtin, V.Y. & Hughes, D.A. (1993). The application of automated procedures for low 	flow analysis of South African rivers. Proc. 6th SA Nat. Hyd. Symp., 329-336.