Economic Viability of Irrigation Determined by

Using Stochastic Water Resource Analysis Techniques

H G Maré & P G van Rooyen

BKS Incorporated

P O Box 3173

Pretoria, 0001

Introduction

The advantages gained by using stochastic yield analysis in water resource systems has already been proved in practise to determine the yield capability of water resource systems (McKenzie, 1990) (Basson et al. 1992).  Currently stochastic yield analyses are widely used by the Department of Water Affairs and Forestry for the planning and operation of water resource systems in South Africa.

This paper describes further developments in the application of stochastic yield analysis, and describes the use of the technology in a study on the Marico-Bosveld Government Water Scheme.  (Maré HG, van Rooyen PG, 1995).  This study was requested by the Department of Agriculture and executed by BKS Inc.  The primary purpose of this study was to link stochastic water supply sequences, with appropriate economic analyses and to determine the historic and stochastic yield capabilities for Marico-Bosveld Dam.

Previously, the economic analysis of irrigation schemes was based on the average amount of water which could be supplied during the historic period, for 70 to 80 percent of the time.  It has been shown, however, that this method can provide misleading results, particularly in drier regions.  This is due to long dry periods experienced in such regions, which can often be 5 to 10 years in duration.  Irrigators in an over exploited scheme cannot survive financially during such extreme drought events.  Traditional methods of calculation, however, may indicate that irrigation is in fact economically viable.  (Louw AA et al., 1993).

Further developments by Louw and Groenewald (1993) used the historic yields observed in the past in an economic related model, to quantify the viability of an irrigation scheme. Basson (1992), however, showed in his comparison of Allemanskraal and Erfenis dams that historic yield figures alone, without quantifying the risk, can produce misleading results.

If one accepts that the availability of irrigation water has a large influence on the economic viability of irrigation schemes, it is clearly essential to assess the associated risk of water supply and include this in the economic analyses.  It was therefore suggested that the stochastic yield results from the system analysis be used, together with an economic model, to evaluate the economic viability of the irrigation scheme in terms of risk, including the effect of consecutive dry years, as experienced in many parts of South Africa.

Methodology

The methodology used in this new development involves the use of three different models and can be explained by means of the flow diagram shown in accompanying Figure 1.  The basic approach followed was to link existing resource yield models with existing economic analysis models with intermediate data manipulation software, in an attempt to illustrate the benefit of stochastic related economic analyses.  The purpose of the first model, the WRYM (Water Resources Yield Model) is to determine the yield characteristics of the water resource.  As input the WRYM needs data describing the physical system characteristics as well as a full set of hydrological data, which includes monthly natural run-off, rainfall, diffuse irrigation and afforestation files.

Historic and long-term stochastic yield results are output from the WRYM and represent the yield capability of the system while the short-term yield characteristics provide system storage dependant yield results, used to operate a water supply system.

Short-term stochastic yield results are therefore, provided as input to the second model, the Water Resources Planning Model (WRPM) to perform operational analyses.  In the WRPM the short-term stochastic yield characteristics are used for decision-making concerning the operation of the dam i.e. to indicate how much water can be supplied at certain risk levels depending on the water level in the dam.  This operating procedure is implemented by an allocation algorithm imbedded in the WRPM, which follows a curtailment strategy defined for the specific system by an allocation definition.  (Basson et al, 1994).  Operating rules other than those based on the short-term stochastic characteristic curves, can also be modelled with the WRPM.  As input the WRPM uses the same physical system characteristics and hydrology as the WRYM.  The output from the WRPM, which is used in the economic analysis, consists of monthly water supply values, representative for each of the stochastic or historic flow sequences analysed.

The economic model (third model) makes use of dynamic linear programming to maximise a farming unit's income, by means of a cash-flow analysis.  This optimisation procedure involves the selection of the optimum combination of hectares of particular crops, within defined constraints, relevant to the specific prevailing conditions.  The linear program problem is solved simultaneously for a period of 20 years, with the annual variable being the allocated supply of water.  The economic analysis model is therefore referred to as the Dynamic Linear Programming Model (DLPM).  Several link programs were developed to convert the results from the WRPM into the correct format to be used in the DLPM and to enable the DLPM to automatically process all the stochastic data sets.  Result presentation programs were also developed to process the stochastic results from the DLPM, which represents the probabilistic behaviour of economic variables.

The procedures followed in the methodology described above, applications as well as results obtained from this new development, can best be explained by means of an example.  For this purpose, the analysis carried out for the Marico-Bosveld Government Water Scheme is used.  Details of the scheme are given in the next section.

Marico-Bosveld Government Water Scheme

The Marico-Bosveld Government Water Scheme is located in the Western Transvaal near Groot-Marico town.  (Accompanying Figure 2).  Two dams, the Marico-Bosveld Dam and Kromellenboog Dam, supply Marico-Bosveld Irrigation Scheme with water.  On average, the Marico-Bosveld Dam provides for 82 % of the total scheme's requirements, while the remaining 18 % is supplied from Kromellenboog Dam.  The total irrigation area scheduled under the Marico-Bosveld Scheme is 2 544 ha, with an annual allocation of 5  300 m3/ha, resulting in a total demand of 13,85 million m3/a (losses included) to be supplied from Marico-Bosveld Dam.  Tobacco, chilies and wheat are the main crops cultivated at the Scheme.

When Government water schemes were first developed a social objective was pursued and capital expenditure for the schemes was fully subsided, while operating costs were only subsidised in part.  Privatisation of these schemes are now being considered, which also included the Marico-Bosveld Government Water Scheme.  Prior to the described development, the Directorate of Agricultural Economics of the Department of Agriculture had already investigated the possibility of privatising the Marico-Bosveld Government Water Scheme.  These investigations were based on results from a detailed economic survey and a dynamic linear programming analysis, using the historic water releases as indication of the availability of irrigation water.  The Marico-Bosveld Scheme therefore provided an ideal opportunity to evaluate the potential of this new development.

Owing to limited time and funds, this new development can be regarded as the first step or phase  and illustrates the potential of linking stochastic yield results and economic analysis.

Yield Results

In this section the yield results as obtained from the WRYM are discussed.  The system schematic used for the analysis, is given in accompanying Figure 3.  Results of the historic and long-term stochastic yield analyses for Marico-Bosveld Dam are given in Table 1.  The objective of these analyses was to determine the long-term yield capability of the water resource.

Table 1:	Summarised results of the historic and long-term stochastic yield analyses for Marico-Bosveld Dam

| Historic |  | Irrigation requirements (Quota of 5 300 m3/ha/a applies) |  | Long-term stochastic yield for given recurrence period or risk level (106m3/a) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Yield (106m3/a) | Recurrence interval (years) | Without losses (106/m3/a) | With losses (106/m3/a) | 1:10 (year) | 1:20 (year) | 1:50 (year) | 1:100 (year) | 1:200 (year) |
| 9,6 | 1:100 | 11,08 | 13,85 | 12,5 | 12,2 | 10,7 | 9,6 | 8,2 |

From the above results it is clear that the Marico-Bosveld irrigation scheme is presently operated at a relatively high risk for the occurrence of possible failures of less than 1:10 years.

Short-term stochastic yield characteristics were also determined for Marico-Bosveld Dam.  This information is used for decision-making concerning the operation of the dam and allocates water at the beginning of the planting season, depending on the selected risk level and the water level in the dam.  Summarised results of the short-term stochastic analyses are given in Table 2.

Table 2:	Short-term stochastic analyses results

| Initial volume of dam as a percentage of the live storage volume | Short-term stochastic yield for indicated risk level or recurrence interval (106m3/a) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1:10 year | 1:20 year | 1:100 year | 1:200 year |
| 100 % | 20,4 | 19,0 | 16,3 | 15,1 |
| 80 % | 18,0 | 16,3 | 13,5 | 12,6 |
| 60 % | 15,1 | 13,4 | 10,5 | 9,3 |
| 40 % | 12,2 | 10,5 | 7,8 | 6,5 |
| 20 % | 8,3 | 7,3 | 4,8 | 3,7 |
| 10 % | 4,9 | 4,2 | 3,0 | 2,6 |

From the results given in Table 2 it can be seen that if the 1:20 year recurrence level is selected, restrictions have to be imposed on the system demand of 13,85 million m3/a when the level of the dam drops below 60 % of the live storage volume.

Operational analyses of Marico-Bosveld Dam

The WRPM (Water Resources Planning Model) as indicated in the flow diagram accompanying (Figure 1) was used for the operational analyses of Marico-Bosveld Dam.  Various operating rules were evaluated.  The annual water supply sequences (historic and stochastic), as derived for the various operating rules, were used as input to the economic model.  The operating rules which were considered, are as follows:

i)	Existing operating rule which is presently being applied to the scheme, and which is based on the available volume of water in the dam at the start of the planting season, excepting no inflow to the dam for that season (Rule 1).

ii)	The use of short-term stochastic curves as operating rule, at a risk level of 1:10 years, with the existing quota of 5 300 m3/ha/year as maximum allocation (Rule 2).

iii)	The use of short-term stochastic curves as operating rule, at a risk level of 1:10 years, with the maximum potential allocation equal to the short-term stochastic yield capability of Marico-Bosveld Dam.  The maximum potential annual allocation at the 1:10 years risk level is 20,4 million m3/a as depicted in Table 2.  This rule implies that the irrigators can utilise more than the 5 300 m3/ha/year quota.  (Rule 3).

In accompanying Figure 4 the water supply as obtained from a historic run using operating rules 2 and 3, are compared.  From this comparison the excess water which can be supplied during wet years by using operating rule 3 is clearly evident.  The advantages gained by using the excess water during wet years are further illustrated in the stochastic water supply results given below, as well as in the results from the economic analyses as discussed in the next section.

In total, 201 stochastic flow sequences of 40 years each were analysed.  Results from the stochastic analysis are given in Table 3.

Table 3:	Summary of the stochastic water supply results obtained from the operational analyses

| Operating rule | Average water supply at given exceedance probability (106m3/a) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 10 % | 25 % | 50 % | 75 % | 90 % |
| Rule 1 | 13,85 | 13,85 | 13,85 | 12,00 | 8,50 |
| Rule 2 | 13,85 | 13,85 | 13,85 | 13,80 | 10,00 |
| Rule 3 | 20,00 | 19,00 | 17,50 | 13,00 | 9,50 |

From the results in Table 3 it can be seen that the operating rules based on the short-term stochastic curves (Rules 2 and 3) are advantageous at all levels of probability, particularly if the existing quota is not considered as a limitation (Rule 3).

Economic analyses

In the execution of the economic analyses 3 typical farming units or models, which are representative of the Marico-Bosveld Government Water Scheme, were considered.  An economic analysis was undertaken for each of these models using all three operating rules.  Details of the three farming models are given in Table 4.

Table 4:	Typical farming units or models

| Model | Listed irrigated area (ha) | Crops under irrigation |
| --- | --- | --- |
| Model 1 | 86 | Tobacco, chilies and wheat |
| Model 2 | 26 | Tobacco, chilies and wheat |
| Model 3 | 29 | Chilies and wheat |

In accompanying Figures 5 and 6 the results obtained from the cash flow analysis using stochastic water supply sequences, are given for farming models 1 and 3 respectively.   At the 95 % exceedance level the cash flow for model 1 remains positive throughout the 20 year analysis period.  Opposed to this result, the cashflow for model 3 at the 95 % exceedance level remains negative for 7 consecutive years.  This result therefore indicates that although water was supplied to both farming models at the same risk level, the economic outcome for the farming units still varies considerably.

In accompanying Figure 7 the cumulative nett present value of the cashflow over a 20-year period is given for farming model 1, considering operating rule 1.  In this figure the initial results obtained by using a historic water supply sequence is compared with the stochastic results.  From this analysis it can be seen that the historic analysis provided a very conservative result, which is mainly due to the assumption of a zero water allocation in year two of the analysis.

A summary of the stochastic results as obtained from the economic analysis are given in Table 5.

Table 5:	Summary of stochastic results obtained from the economic analysis

| Farming unit or model | Operating rule | Cumulative nett present value over 20 years at given exceedance probability (Million rand) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 % | 25 % | 50 % | 75 % | 90 % | 99 % |
| Model 1 | Rule 1 | 1,45 | 1,43 | 1,39 | 1,31 | 1,22 | 0,84 |
|  | Rule 2 | 1,46 | 1,45 | 1,43 | 1,35 | 1,25 | 0,90 |
|  | Rule 3 | 1,66 | 1,62 | 1,56 | 1,45 | 1,34 | 0,93 |
| Model 2 | Rule 1 | 0,43 | 0,42 | 0,40 | 0,36 | 0,31 | 0,17 |
|  | Rule 2 | 0,44 | 0,43 | 0,42 | 0,38 | 0,34 | 0,19 |
|  | Rule 3 | 0,51 | 0,49 | 0,46 | 0,41 | 0,36 | 0,19 |
| Model 3 | Rule 1 | 0,17 | 0,16 | 0,14 | 0,11 | 0,07 | -0,05 |
|  | Rule 2 | 0,18 | 0,17 | 0,16 | 0,13 | 0,09 | -0,04 |
|  | Rule 3 | 0,26 | 0,24 | 0,22 | 0,17 | 0,12 | 0,01 |

From the results in Table 5 it is clear that all three farming models can benefit economically from using operating rules based on the short-term stochastic curves (Rules 2 and 3).  At an exceedance probability of 50 %, for example, an increase in the cumulative nett present value of about 50 % is obtained for Model 3 when operating rule 3 is used instead of the current operating rule (Rule 1).  Although at this exceedance probability, the increase in the nett present value when considering Model 1, is not as significant as for Model 3, similar nett present values can be obtained at significantly higher exceedance probabilities, for example R1,45 million at 10 % for Rule 1 in comparison with R1,45 million at 75 % for Rule 3.

At exceedance probabilities of 99 % and higher, it appears that Model 3 is not at all economically viable.  A realistic exceedance probability whereby the feasibility of a scheme or model could be judged, would probably be 90 % or 95 %, which implies that there is a 10 % or 5 % chance that the farming unit will fail economically.  This exeedance probability might however differ, depending on the irrigation scheme analysed.

Summary and Conclusions

The results from the Marico-Bosveld System case study, clearly indicate the benefit of performing economic analysis based on stochastic yield results.  The methodology applied in the study made use of existing models and limited effort was spent on the development of stand alone, user friendly software.  In order to enhance the usability of the development techniques, further software developments have to be considered.

The economic analysis techniques used is of an optimisation nature, taking account of foreknowledge that water supply figures for a number of years are included in the linear programme definition.  The economical results therefore represent a theoretical optimum which cannot be achieved in practice, as foreknowledge of supply figures are not available.  It is envisaged that a simulation economic model will provide a more practical analysis mechanism, where fixed rules are applied and optimised.  The theoretical optimum results obtained from the linear programming analysis, will however provide valuable input to the selection of operating rules for a simulation model.

Recommendations in respect of further development and application possibilities

Meaningful results were obtained from the stochastic economic analyses, and it is therefore recommended that this analysis methodology should be used in future  wherever possible for economic analyses where irrigation plays a primary role.  It is also recommended that the methodology be further developed to obtain wider application possibilities, refine existing techniques and provide manuals in which the methodologies and procedures are properly described.

Typical additional application possibilities include:

i)	the ability to judge, in terms of risk, the economic viability of new and existing schemes by for e.g. examining the size of a scheme, farm sizes, the risk of water supply, etc.;

ii)	to serve as an indicator in the decision-making process for the provision of drought relief by indicating what risks are attached to "normal" droughts and "extreme" droughts;

iii)	to serve as an aid to financial institutions to determine the risk of irrigation farming for a specific area or farming unit;

iv)	to provide management information for on-farm decision-making (see the last recommendation); and

v)	to use the stochastic analyses to project future dam levels and the accompanying supply of irrigation water.  By coupling these results to an economic model, it will be possible to convey the risk attached to a specific economic outcome to the farmer already at the start of the planting season.  It is envisaged that a simplified model should be developed which could be used by irrigation boards, schemes and even a single farmer to make predictions about the risks attached to water supply and the planned investment.

References

Basson, MS & JA van Rooyen (1992).  Systems Analysis in Water Resources Management.  	SA National Water Week Conference, Pretoria.

Basson MS, RB Allen, GGS Pegram & JA van Rooyen (1994).  Probabilistic Management 	of Water Resource and Hydropower Systems.  Water Resources Publications.

Louw, AA & DJ Groenewald (1993).  'n Verwantskap tussen Ekonomiese 	Besproeiingswaarde en Hidrologiese Data in Besproeiingsgebiede.  Proceedings of the sixth South African National Hydrological Symposium Volume 1.

Maré HG & PG van Rooyen (1995).  Marico-Bosveld Staatswaterskema:  Stogastiese

waterbronontledings gekoppel aan ekonomiese evaluasies.

McKenzie RS & RB Allen (1990).  Modern water resource assessment techniques for the

Vaal System.  Proceedings of the Institute for Civil Engineers.  Part 1.