Low Flow Analysis in Tanzania

R J Mngodo & J I Matondo

Water Resources Engineering Programme

University of Dar Es Salaam

P O Box 35131

Dar Es Salaam, Tanzania

Introduction

The study area consists of 11 small catchments located in the Indian Ocean drainage basin. The average annual rainfall in this area is above 1000 mm. The increasing water demand on water resources inevitably leads to a better understanding of low flows processes.

Low flow characteristics are fundamental to a wide range of design and operational problems in areas of both water quality and quantity. The on going SADC-FRIEND project will be of value in solving some of these practical problems by evaluating the natural variability of hydrological extremes. In this study relationships between low flow statistics and catchment characteristics are developed in order to evaluate the main controlling influences on low flows. It also aims to provide methods of estimating low flows at ungauged sites Derived regression relationships are presented between single statistics, such as mean annual minimum and 50 percentile flow for given flow duration curve, and basin characteristics such as area, average annual rainfall, slope and length of main stream. The relationship between low flows of different durations and different frequencies is also discussed.

Catchment Selection and Determination of Parameters

A total of 11 catchments were selected with area ranging from 100 to 500 km2. The length of daily average discharge data record ranges between 10 and 30 years. Catchment characteristics such as area (AREA), the slope of the main river channel (SL1085) and length of channel, (L) were determined by means of a planimeter. Another catchment characteristic, the average annual rainfall (AAR) was determined by arithmetic mean. Table 1 presents the list of selected catchments with their characteristics.

Calculation and Presentation of Low Flow Statistics

Computer programs were developed to derive and present low flow statistics using procedures similar to those described in Low Flow Studies Report (Institute of Hydrology, 1980) and European FREND (Gustard et al.,1989). Figure 1 shows the flow duration curves of the selected catchments. Average daily flow (ADF), 50 percentile flow (Q50) and mean annual minimum (MAM) for 1 and 10 day duration, baseflow index (BFI) were calculated and are presented in Table 2. The average flow is expressed in m3/s while the one day 50 percentile flow, Q50(1), and mean annual 1 and 10 day minima, MAM(1) and MAM(10), are expressed in percentages of the average flow so as to enable comparison between catchments. The annual runoff yield in mm (AAY), coefficient of variation of baseflow (CVBFI) and the coefficient of variation of annual runoff (ARCV) were also calculated.

Table 1. List of selected catchments with characteristics

| Gauging station | Area (km2) | L (km) | SL1085 (m/km) | AAR (mm) |
| --- | --- | --- | --- | --- |
| 1B1B 1DB19 1DB2A 1G6 1GB1A 1GD29 1HC2 1KA10A 1KA22 1KB18B 1KB19 | 200.3 310.6 193.5 140.3 173.8 477.6 373.6 247.1 455.7 529.1 331.1 | 35.35 61.37 35.63 41.00 29.96 52.13 31.08 39.11 50.56 50.52 65.51 | 46.77 6.95 48.50 29.92 31.14 76.73 9.44 12.38 3.16 8.17 3.15 | 920.9 898.4 864.7 865.4 1070.9 1095.7 1155.5 726.3 1120.1 1154.9 1334.6 |

Regional Analysis of Low Flow Indices with Catchment Characteristics

The parameters affecting low flows were determined from relationships between low flow indices and catchment characteristics. The results of the correlation matrix are shown in Table 3. The correlation coefficients of two key statistics of low flow, Q50(1) and MAM(10), with catchment characteristics seem to differ and thus regional equations derived from them will be different. The catchment area (AREA) is highly correlated with Q50(1) but poorly correlated with MAM(10), while the length of main channel (L)  shows weak correlation with both key statistics of low flows. The  annual average rainfall (AAR) shows some good correlation with Q50(1). The average slope (SL1085) is weakly correlated with Q50(1). The baseflow index (BFI) shows some correlation with the area and length of main channel but is weakly correlated with AAR.

The number of parameters was varied in the multiple linear regression and the following nonlinear equation was obtained:

Q50(1) = a * AREAb * AARc * Ld * SLe 					(1)

Using this method, a statistically optimal model with the greatest regression coefficient and the minimal standard error of estimate was obtained. The regression models for Q50(1) and MAM(10) with catchment characteristics are presented in Table 4 and Table 5 with R2 and SE for each model. The comparison of regression coefficients R2 and SE in the Q50(1) model shows that the model with only three parameters, area (AREA), average annual rainfall (AAR) and length of main channel (L), have the highest R2 and the lowest SE while in the MAM(10) model shows that the model with two parameters, AAR and L, gives the highest R2 and the lowest SE. The regression equations for the region studied are as follows:

Q50(1) = 0.056 AREA0.283 AAR0.619 L0.267							(2)

MAM(10) = 7.639 AAR-0.332 L0.925								(3)

The values of coefficients R2 and SE are 0.874 and 0.096 for Q50(1) and 0.107 and 0.505 for MAM(10).

The regional equation for estimating the baseflow index, BFI, was derived by regression analysis between BFI and the catchment characteristics i.e  AREA, AAR, L and SL. The transformed variables improved the regression results. The results of the regression model:

BFI = const * AREAa * AARb * Lc * SLd					(4)

are given in Table 6. The analyses of the models show that BFI is best correlated with the average annual rainfall (AAR) and the length of the river (L). The obtained regression model with the highest R2 and the smallest standard error of estimate SE is presented in the following equation:

BFI = 0.018 * AAR0.309 * L0.384 * SL0.035  						(5)

Table 2. Calculated statistics of low flows.

| Gauging station | ADF (m3/s) | Q50(1) (% ADF) | MAM(10) (% ADF) | BFI | CV(BFI) | AAY (mm) | ARCV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1B1B 1DB19 1DB2A 1G6 1GB1A 1GD29 1HC2 1KA10A 1KA22 1KB18B 1KB19 | 1.37 1.10 1.83 3.08 8.41 2.75 26.02 2.94 4.06 5.99 3.04 | 50.14 59.82 38.26 46.83 51.02 80.51 55.56 42.18 80.27 73.66 72.34 | 16.1 18.2 10.9 55.8 14.5 47.6 19.1 34.7 44.1 24.0 27.6 | 0.828 0.768 0.664 0.673 0.590 0.786 0.617 0.569 0.826 0.868 0.813 | 0.337 0.327 0.265 0.235 0.349 0.308 0.274 0.416 0.220 0.308 0.259 | 216.2 111.4 298.7 693.4 1528.7 181.5 2198.3 375.4 281.2 357.1 289.4 | 0.819 0.563 0.604 0.561 0.679 0.487 0.554 0.918 0.327 0.544 0.360 |

Table 3. Correlation matrix of low flow indices and catchment characteristics.

|  | Q50(1 ) | MAM(1 0) | ADF | BFI | AREA | L | SL | AA R |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q50(1) MAM(1 0) ADF BFI AREA L SL AAR | 1.000 0.408 0.349 0.404 0.900 0.453 -0.457 0.593 | 1.000 0.007 0.176 0.258 0.430 -0.164 0.042 | 1.000 -0.376 0.242 -0.431 -0.204 0.478 | 1.000 0.543 0.688 -0.257 0.477 | 1.000 0.546 -0.474 0.582 | 1.000 -0.525 0.326 | 1.000 -0.408 | 1.00 0 |

Table 4. Regression models for Q50(1) with catchment characteristics.

| AREA | AAR | L | SL | R2 | SE |
| --- | --- | --- | --- | --- | --- |
| + + + + | + + + + | + - + - | + - - + | 0.870 0.829 0.874 0.735 | 0.092 0.112 0.096 0.139 |

+ variable is included in the model

- variable is not included in the model

Duration Relationships

The analysis of duration using the mean annual ten day minimum, MAM(10), as a key variable, showed that it is possible to translate the results from one key duration to the duration of interest. The coefficients of linear model:

MAM(D) = const + k * MAM(10)							(6)

were derived by regression analysis and are shown in Table 7. Likewise duration analysis was also done using the 50 percentile daily flow, Q50(1), as a key variable, showed that it is possible to translate the results from one key duration to the duration of interest. The coefficients of linear model:

Q50(D) = const + k * Q50(1)							(7)

were derived by regression analysis as shown in Table  8.

Table 5. Regression models for MAM(10) with catchment characteristics.

| AREA | AAR | L | SL | R2 | SE |
| --- | --- | --- | --- | --- | --- |
| - + + + | + + - - | - + - + | - - - - | 0.107 0.006 0.067 0.095 | 0.505 0.533 0.516 0.508 |

+ variable is included in the model

- variable is not included in the model

Table 6. Coefficients of regression model for BFI with catchment characteristics.

| const | a | b | c | d | R2 | SE |
| --- | --- | --- | --- | --- | --- | --- |
| 0.220 0.140 0.167 0.046 0.018 0.097 | 0.049 0.080 - 0.039 - 0.100 | 0.248 - - 0.196 0.309 - | 0.371 0.314 0.388 0.301 0.384 0.361 | 0.037 - - - 0.035 0.030 | 0.428 0.459 0.474 0.431 0.478 0.432 | 0.115 0.111 0.110 0.114 0.110 0.114 |

-  variable is not included in the model

Table 7. Coefficients of regression for estimating MAM(D) from MAM(10)

| Duration D (days) | Constant | k | R2 | SE |
| --- | --- | --- | --- | --- |
| 1 5 7 30 60 90 180 | -4.1108 -1.8756 -1.0091 3.5836 6.2719 10.5833 21.0964 | 1.038 1.016 1.005 0.983 1.037 0.985 0.905 | 0.9981 0.9996 0.9998 0.9909 0.9826 0.9692 0.8674 | 0.720 0.302 0.223 1.492 2.189 2.791 5.619 |

Table 8. Coefficients of regression for estimating Q50(D) from Q50(1)

| Duration D (days) | Constant | k | R2 | SE |
| --- | --- | --- | --- | --- |
| 5 7 10 30 60 90 180 | 22.174 23.625 24.964 27.920 36.867 43.028 58.436 | 0.743 0.728 0.717 0.711 0.608 0.550 0.408 | 0.7800 0.7609 0.7581 0.7525 0.7146 0.7157 0.6917 | 6.513 6.732 6.684 7.091 6.340 5.722 4.493 |

Accuracy of Analysis

The regression equations (2) and (3) were tested for catchments listed in Table 9. Calculated values of Q50(1) and MAM(10) using equation (2) and (3) were compared with measured values in Table 10. The average error between calculated and measured flows for prediction of Q50(1)  is about 8.7 % while that of MAM(10) is about 52.3 %. The Baseflow index calculated using equation (5) compared to the measured values gives an average error of about 11%. It can be concluded that the low flows of this region can be evaluated by the derived regional equation (2). However it should be noted that the derived regression equation gives satisfactory results for the range of catchment area between 100 -500 km2, the analysis gives satisfactory results. Similar study was done by Kobold and Brilly (1994) where average error was less than 20 % in estimation of both Q95(1) and MAM(10).

Table 9. List of catchments for testing the regional equations with their characteristics.

| Gauging station | Area (km2) | AAR (mm) | L (km) | SL (m/km) |
| --- | --- | --- | --- | --- |
| 1DA3A 1DB6A 1GD31 1KA7A 1KA56 1KB15 1KB15A 1N2A GD30 1G8 | 276.8 502.8 497.2 166.0 165.4 304.8 328.7 345.6 595.9 396.6 | 1380.6 878.2 1648.9 1100.0 900.0 1867.5 1867.5 1150.0 750.0 943.7 | 40.69 79.62 60.1 29.13 38.98 33.29 40.58 21.92 57.7 63.2 | 34.08 18.42 20.3 36.16 14.07 30.84 24.62 15.82 60.01 85.44 |

Table 10. Calculated and measured values of Q50(1) and MAM(10) by equations (2) and (3).

| Gauging station | Q50(1) calculated | Q50(1) measured | Error (%) | MAM(10) calculated | MAM(10) measured | Error (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1DA3A 1DB6A 1GD31 1KA7A 1KA56 1KB15 1KB15A 1N2A 1GD30 1G8 | 64.98 69.56 73.94 47.15 42.61 76.30 82.18 52.38 60.74 63.93 | 62.95 69.37 68.94 44.53 42.29 84.88 66.21 52.04 73.27 77.61 | 3.2 0.3 7.3 5.9 0.7 10.1 24.1 0.7 17.1 17.6 | 21.35 46.15 27.49 33.02 23.65 16.42 24.83 16.04 19.26 19.28 | 24.65 27.71 32.97 17.71 7.44 59.37 25.97 21.6 23.46 47.92 | 13.4 66.6 16.6 86.4 217.8 72.3 4.4 25.8 17.9 59.8 |
| Average Std.dev. |  |  | 9.4 7.9 |  |  | 52.3 63.2 |

Table 11. Calculated and measured values of BFI by equation (5).

| Gauging station | BFI calculated | BFI measured | Error (%) |
| --- | --- | --- | --- |
| 1DA3A 1DB6A 1GD31 1KA7A 1KA56 1KB15 1KB15A 1N2A 1GD30 1G8 | 0.789 0.869 0.839 0.666 0.659 0.799 0.856 0.573 0.762 0.858 | 0.703 0.735 0.766 0.748 0.490 0.893 0.790 0.874 0.722 0.821 | 12.1 18.3 9.6 10.9 34.6 10.5 8.3 34.5 5.6 4.5 |
| Average Std.dev. |  |  | 14.9 11.0 |

Application of Regional Equations

Low flows of ungauged catchments or where data is insufficient for statistical analysis can be evaluated from the derived interdependencies. Topographic maps of 1:50,000 scale are required to derive catchment characteristics. In the near future, studies will be conducted to test and improve the obtained accuracy and all catchments in Tanzania will be included to evaluate low flows on catchments below 100 km2 and those above 500 km2. Additional low flow characteristics such as soil index, altitude of gauging station and mean altitude of catchment will also be considered in the analysis.

References

Gustard, A., Roald, L., Demuth, S., Lumadjeng, H. & Gross, R. (1989) Flow Regimes

from Experimental and Network Data (FREND), vol.I. Institute of Hydrology, Wallingford, Oxfordshire, UK.

Institute of Hydrology (1980) Low Flows Studies Report. Wallingford, UK.

Kobold, M. & Brilly, M.(1994) Low Flow Discharge Analysis in Slovania. (Proc. 	Braunschweig Conference, October 1994). IAHS Publ. no. 221