The Influence of Changing Flow Regime on Hydraulic Parameters

and Sediment Dynamics on the Sabie River

A W van Niekerk, G L Heritage & L J Broadhurst

Centre for Water in the Environment

University of the Witwatersrand

P/Bag 3

Wits, 2050.

Introduction

Recent history has seen increasing concern regarding the effects of increased water abstraction on the ecological integrity of river systems in South Africa. Instream Flow Incremental Methodology (IFIM) is based on the premise that predictions of biological responses to modified flow regime can be achieved through a consideration of the influence of the modification on habitat availability for target species. IFIM has tended to focus on macro habitat (variables such as temperature and water chemistry which are more or less constant laterally and at a specific locality longitudinally) and microhabitat (variables such as water depth, velocity, substrate character and overhead cover which may vary significantly across the channel and longitudinally in a reach) (King and Tharme, 1994). Modification of flow regime, however, also leads to adjustment of the morphology of river systems (Schumm, 1977) with consequent physical habitat change. In order to investigate the ecological effects of changing flow regime, it is necessary to develop techniques for predicting and comparing the implications of regime changes on hydrodynamic variables (e.g. flow depth, flow velocity, wetted perimeter, stream power and sediment transport rates).

The Sabie River in the semi-arid eastern Transvaal lowveld is a mixed bedrock/alluvial system (van Niekerk et al., 1995). Four different primary channel types have been identified, namely single thread, braided, pool/rapid and anastomosing, with varying degrees of bedrock influence (van Niekerk et al., 1995) ranging from fully alluvial braided through to bedrock dominated anastomosing. Although alluvial and bedrock channels are generally considered to be fundamentally different in character (Ashley et al., 1988; Wohl, 1992), alluvial channels and those formed completely in bedrock may be regarded as occurring at opposite ends of a continuum of channels of varying sediment supply in relation to transport capacity (Ashley et al., 1988).

Flow depth monitored at a number of sites and related to discharge on different channel types along the Sabie River in the lowveld has enabled the development of stage/discharge relationships for the sites. Using these relationships and the geometry of the measured cross-sections, a number of flow parameters were calculated. Manning's roughness coefficients determined for detailed studies of various channel types (Broadhurst et al., this volume) were used to calculate friction slopes and hence bed shear stresses, shear velocities and ultimately sediment transport rates for a range of particle sizes for any particular discharge at each cross-section.

Measured average daily flows over a period of 45 years at an  upstream weir have been transformed to average daily flows at each site and have been analysed for wet, dry and intermediate periods. The hydraulic and sediment transport parameters were calculated for each measured discharge at each site and probability of exceedence curves for the wet, dry and total flow record. Comparison of results for different channel types highlights potential sediment accumulation zones and provides a means for evaluating morphologically and ecologically critical flow regimes. Considering the total, wet, and dry flow regimes as resulting from different dam management scenarios, the potential use of the model to compare flow scenarios and thus evaluate the effects of different management strategies is demonstrated.

Stage/Discharge Relationships

Twenty-five cross-sections have been established along the length of the Sabie River in the lowveld to sample the range of different observed channel types. Flow depth has been monitored at these sites over a range of flows and related to discharge, at nearby gauging weirs in order to establish stage/discharge relationships. Breaks in the stage/discharge curves were identified through visual inspection of discharge vs stage and log(discharge) vs stage plots of the data. Linear regression equations were determined for discharge and log(discharge) vs stage for the different portions of the stage/discharge curves and the ones with the lowest r2 values  were selected. Typical values of r2 range from 0.73 - 0.999 with the majority in excess of 0.95. The stage/discharge relationships were extrapolated beyond the range of measured data through the assumption that for high discharges

where Sf is the friction slope and n is the Manning roughness coefficient. k is a constant calculated for the upper limit of the measured data from

where Qmax is the maximum discharge for which stage was measured, A is flow area, R is the hydraulic radius = A/P and P is the wetted perimeter. Flow area and hydraulic radius are functions of the known stage and the measured geometry of the channel. This assumption is reasonable since firstly, the channel shape is regular for  high discharges and secondly local controls in the channel are drowned out at high discharge and do hence have limited effect on water surface slope. One problem is that Manning’s n values vary with discharge in the different channel types (Broadhurst et al., this volume), however, they are asymptotic to a constant value at high discharges for all channel types except the highly irregular bedrock anastomosing one (Fig. 1). The assumption of a horizontal water surface across a cross-section is also not necessarily correct for anastomosing channels, particularly those with a strong bedrock influence.

Extrapolated values for discharge Qext were calculated from

for extrapolated values of stage.

Hydrodynamic Parameters

The derived stage/discharge relationships were used to calculate flow depth, d, flow area, A, wetted perimeter, P, and average flow velocity, V, for different discharges, Q, at each of the cross-sections. Results of roughnesses determined form detailed studies on single thread, braided, pool/rapid, and bedrock and mixed anastomosing channel types (Broadhurst, et al., this volume),  were then used to estimate approximate Manning roughness coefficients for each of the cross-sections (bedrock anastomosing, n = 0.3, pool/rapid - n = 0.1, mixed anastomosing - n = 0.06, single thread - n = 0.075 and braided - n = 0.04).

Friction slopes, Sf, were then calculated for the different discharges from the Manning equation

Shear velocity u* was calculated from

where g is the gravitational constant and R is the hydraulic radius = A/P. Since the chosen

values of n are underestimated for braided, pool/rapid and mixed anastomosing channel types for low flows (Fig. 1), shear velocity and hence sediment transport rates will also be underestimated.

Sediment transport rates are calculated using the total load calculations of Ackers and White (1973) as follows. The dimensionless grain size Dgr is calculated from

where s is the relative density of the sediment, ν is the kinematic viscosity of the fluid and D is the sediment diameter. Coarse particles are transported mainly as bedload and fine ones mainly in suspension. For transitional particle sizes 1 < Dgr < 60

where a and b are exponents and C a coefficient in the sediment transport function and E is the value of Froude number at nominal initial motion.

The sediment mobility number Fgr is given by

and the dimensionless sediment transport rate Ggr

The sediment transport rate, Qs, in m3/s is then given by

These have been applied to a wide range of data and good results are claimed (Ackers and White, 1973) with

Unit stream power, P, was determined from the equation of Yang (1972)

Yang (1984) found that dimensionless stream power, Pd, provides a better indication of the bedload or gravel transporting capacity and this was calculated from

where ω is the particle fall velocity which was calculated from (Dietrich, 1982)

where W* is the dimensionless settling velocity.

Exceedence Probabilities

Limited average daily flow records at two weirs in the  downstream reaches within the Kruger National Park have been extended to 36 years through correlation with a weir further

upstream (Heritage et al., this volume). The record has been split into wet, dry and average flow spells following the recommendations of Tyson (1987) for evaluation of the effects of different flow regimes on the hydrodynamic variables. Probability of exceedence curves for average daily discharge have been determined for the total record and the wet and dry subdivisions at site 1 on the eastern boundary of the Kruger National Park (Fig. 2). It should be noted that rare extreme discharges above the weir capacities were not incorporated.

Flow depth, flow width, flow area, flow perimeter, hydraulic radius, average flow velocity, friction slope, unit stream power, dimensionless unit stream power, average bed shear stress, average shear velocity and transport rates of 0.5 mm, 1 mm and 2 mm size fractions were calculated for all average daily flows over the entire length of record at each cross-section. Probability of exceedence curves for all of these parameters were drawn up for the total

record and the wet and dry subdivisions at all sites.

The exceedence probability plot for sediment transport rate of the 1 mm particle size fractionfor the total record(Fig. 3) clearly demonstrates the different sediment transport capacities of the different channel types. The bedrock anastomosing channel (site 10) has a high potential long term sediment transport capacity (Fig. 3) and is continuously transporting the 1 mm size fraction (Fig. 3). By contrast, there is no sediment transport in the braided section (site 5) for approximately 15% om the time and the long term potential transport. This would indicate that site 5 (and other braided sections) are good potential zones of deposition and that site 10 (and other bedrock anastomosing sections) are far less likely to accumulate sediment. The other three selected sections (site 15 - pool rapid, site 1 - single thread and site 21 - mixed anastomosing) display similar sediment transport capacities with an intermediate potential to accumulate sediment.

The probability of exceedence plots for wetted perimeter at sites 5 (Fig. 4), 10 (Fig. 5) and 21 (Fig. 6) show a narrow range of wetted perimeter for the braided section (40 to 100 m) compared with the bedrock anastomosing (8 to 100 m), and the mixed anastomosing (80 to 200 m) ones. The dramatic increase in wetted perimeter at site 5 at around the 3%  probability level and at site 21 around the 15% probability level for the total and wet spells is associated with overbank flow and inundation of macro-channel features and is absent from the dry spell curve. There is a high variation for wetted perimeter exceedence probabilities for total, wet and dry spells at site 10 and to a lesser extent site 21 as opposed to site 5. This is due to abandonment of distributary channels in the bedrock anastomosing sections during dry spells with potential loss of habitat. A study of tree deaths on the Sabie River during the drought of the early 1990s showed that most tree deaths occurred in the anastomosing sections (van Coller, this volume), indicating that the loss of wetted perimeter may be critical ecologically. The frequency of exceedence curves for all of the hydrodynamic variables need to be evaluated by ecologists in order to assess the ecological effects of the short-term climatic variations.

As may be expected, sediment transport rates are generally lower during dry than during wet

spells (Figs 7-9). There is a dramatic increase in sediment transport rates in the braided section (site 5) and the mixed anastomosing section (site 9) during wet spells as opposed to dry.  This indicates that the wet period flow regime may be responsible for channel maintenance through scouring, particularly during flows where the sediment supply is limited. It is interesting to note that sediment transport rates for the bedrock anastomosing for wet and dry spells converge asymptotically to a minimum. This indicates continuous reworking and movement of sediments in the bedrock anastomosing section for all flows during both wet and dry periods and suggests that this section is resistant to geomorphological change through sedimentation. Although only wetted perimeter and sediment transport rates are considered here, similar data are available for all of the hydrodynamic parameters mentioned previously and can provide further clues to the natural functioning of the Sabie River and the potential effects of changes to the flow regime.

Conclusions

The dynamics of the Sabie River are highly variable both temporally and spatially. In order to assess instream flow requirements for maintenance of the ecological functioning of the system,  it is necessary to investigate the dynamics of several different channel types and to consider potential geomorphological change. For example, section 10 has been shown to be more resistant to sedimentation for different flow scenarios than either sections 5 or 21, yet the existing geomorphology results in dramatic loss of habitat during dry spells.

Hydrological modelling of dam scenarios in South Africa traditionally provides time series for monthly flow volumes at various locations in a catchment.  In order to investigate the hydrodynamics at ecologically relevant scales, however, daily time series of volume/discharge are required. The approach presented here, used in tandem with hydrological models for daily discharge provides a management tool for evaluating and comparing the dynamics of the river at different locations for different scenarios. Information is generated which 1) can be utilised by river ecologists to evaluate the potential ecological effects of different management strategies and 2) can be used in conjunction with conceptual models geomorphological change (Moon et al ., in press) to determine potential directions of change at the channel type scale.

Acknowledgements

We thank the Water Research Commission for funding, the Mazda Wildlife Fund for financial and logistical support, the Department of Water Affairs and Forestry for weir flow data and the Natioal Parks Board for logistical support in the Kruger National Park.

References

Ackers, P. and White, R.(1973) Sediment transport: New Approach and Analysis. Journal 	of the Hydraulics Division, ASCE, Vol. 99, No. HY11, pp. 2041-2060.

Broadhurst, L.J., Heritage, G.L., van Niekerk, A.W. and James, C.S. (this volume).

Translating discharge into local hydraulic conditions on the Sabie River, an assessment of channel roughness.

Dietrich, W. E. (1982) Settling velocity of natural particles. Water Resources Research, 	Vol. 18, No. 6, pp. 1615-1626.

Heritage, G.L., van Niekerk, A.W., Moon, B.P. and Kapur, K. (this volume). The influence 	of flow regime on the geomorphology of the Sabie River.

King, J. M. and Tharme, R. E. (1994). Assessment of instream flow incremental 	methodology and the initial development of alternative methodologies for South Africa, Water Commission Report No. 295/1/94, pp 590.

Moon, B.P., van Niekerk, A.W., Heritage, G.L., Rogers, K.H. and James, C.S. (in press). 	A geomorphological approach to the management of rivers in the Kruger National Park: The case of the Sabie River. Transactions, Institute of British Geographers.

Schumm, S. A. (1977). The Fluvial System. Wiley-Interscience, New York.

Tyson, P. D. (1987). Climatic Change and Variability in Southern Africa. Oxford

University Press, Cape Town, pp. 200.

van Coller, A. L, Heritage, G. L. and Rogers, K. H. (this volume). Linking riparian

vegetation distribution and flow regime of the Sabie River through fluvial geomorphology.

van Niekerk, A.W., Heritage, G.L. and Moon, B.P. (in press).  Hierarchical Classification 	of Rivers:  The Sabie River in the Eastern Transvaal.  South African Geographical Journal.

Yang, C. T. (1972). Unit stream power equation and sediment transport. Journal of the

Hydraulics Division, ASCE, Vol. 98, No. HY10, pp. 1805-1826.

Yang, C. T. (1984). Unit stream power for gravel. Journal of Hydraulic Engineering,

ASCE, Vol. 110, No. 12, pp. 1783-1797.