Can Vaal Dam Basin Runoff be Augmented by Convective Cloud-Seeding?

AHM Görgens & GPW Jewitt

Department of Civil Engineering

University of Stellenbosch

Stellenbosch

The Need for Augmentation

The Vaal Dam/Vaal Barrage configuration (see Figure 1) in Gauteng Province is the primary resource for the supply of water to the major part of the industrial heartland of South Africa, centered on the Greater Johannesburg.  Inability to meet the burgeoning water demands of the supply region, has necessitated inter-basin transfer schemes to augment Vaal Dam and its upstream successor Grootdraai Dam (see Figure 1).  The largest and most recent of these is Phase 1 of the Lesotho Highlands Water Project with a gross transfer (before transmission losses) of about 900 Mm3/a to Vaal Dam via the Liebenbergsvlei River by 1997.  At present the Department of Water affairs and Forestry (DWAF) is conducting the so-called Vaal Augmentation Planning Study (VAPS) to determine the optimal source of further augmentation of the Vaal system: further Lesotho Highlands Phases, increased transfers from the Tugela, Umzimvubu transfers, or transfers from the Orange and the Caledon Rivers.

The promising results of recent cloud-seeding experiments in the Highveld-Escarpment zone, irresistibly begs the question: Does augmentation of rainfall in the Vaal Dam catchment by operational cloud-seeding present itself as a further VAPS option in terms of potential delivery of additional runoff and in terms of unit costs of additional Vaal System yield?  This paper describes a pilot study that suggests an affirmative answer to this question on reasonable scientific grounds, and with certain caveats.

Cloud-Seeding in South Africa

Augmentation of rainfall by seeding of convective clouds has, in South Africa, been the subject of research for more than 15 years.  Initially, two parallel research projects, funded by the Water Research Commission and the Weather Bureau, focused on the Nelspruit/Carolina area and the Bethlehem area, respectively.  More recently, these two projects were merged in a single project called the "National Precipitation Research Programme with a view to rainfall enhancement" (NPRP) (Mather and Terblanche, 1993). During 1995 the northerly focus of the research shifted from the Nelspruit area to the Tzaneen area.

Milestones in the Cloud-Seeding Research

The backbone of the research has consisted of randomised seeding of selected convective clouds that meet certain physical criteria signifying suitability for positive seeding responses. Absence or presence of responses to seeding is determined by means of both detailed micro-physical measurements of changes in the clouds, and radar-based observations of changes in rainfall-related reflectivity in the clouds.

i)	Until 1991 dry ice was used as the seeding agent under the hypothesis of a glaciogenic (ice-based) raindrop-forming process.  The glaciogenic randomised seeding experiments resulted in clear findings of positive seeding effects when seeded storms were compared with non-seeded storms (Mather and Morgan, 1990).  These radar-based findings were translated into average potential seeding effects per seeded storm by Görgens and Rooseboom (1990), based on the concepts of mean rainflux, storm area, average rainfall intensity and the use of "time windows since seeding" to assess incremental seeding effects during the life of the storm.  Table 1 presents the average seeding effects proposed by Görgens and Rooseboom (1990).

TABLE 1: 	Most Likely Average Seeding Effects as Increases over Unseeded Storms (Görgens and Rooseboom, 1990)

| Time After Seeding (min) | Rainflux % Increase | Storm Area % Increase |
| --- | --- | --- |
| 0 -10 10-20 20-30 30-40 40-50 50-60 | 0 0 15 25 50 55 | 0 0 5 5 35 40 |

ii)	The improved understanding of micro-physical processes in convective clouds yielded by the above research led in 1992 to the local development of a new seeding agent and approach: Aircraft-borne flares deliver hygroscopic particles (as nuclei) to target clouds under the hypothesis of a coalescence raindrop-forming process.  Randomised seeding experiments during the past four years indicate positive seeding responses to the hygroscopic flares that seem more pronounced than that for the glaciogenic process summarised in Table 1 (NPRP, 1995).

iii)	These promising findings have spurred the initiation of the first tentative area-based seeding experiment in the drought-stricken Tzaneen region, where all reachable seedable clouds within the radar radius of 100 km are hygroscopically seeded.  This development inevitably raises the possibility that such an "operational" seeding approach could also be followed in the Vaal Dam catchment, with its immense need for augmented water supplies.  This paper explores the potential of such a cloud-seeding undertaking.

Cloudseeding impacts on Water Resources

Desk studies of potential augmentation of South African water resources by cloud-seeding can be divided into two groups: earlier studies of an ad hoc nature based on some assumed average increase in rainfall, as opposed to later WRC-steered studies utilising the actual radar-based seeding response according to the findings of the Bethlehem-Nelspruit-Carolina experiments.  The WRC embarked in 1991 on a phased programme of desk-studies of potential cloud-seeding impacts on catchments, following recommendations by Görgens and Rooseboom in 1990.

Maaren's Modelled Runoff Augmentations

Maaren (1984) conducted a seminal investigation for the then Department of Environment Affairs of the potential augmentation of runoff from a 372 km2 gauged catchment (gauging station C8H012), which is part of the Wilge River tributary of the Vaal River.  He used a conceptual lumped catchment model to generate daily streamflow from daily rainfall.  Court's (1979) weather type classification system served to identify seedable days during two summer seasons (one dry, one wet) from 1980 to 1982 and Maaren arbitrarily increased seedable days' rainfall by either 5% or 15%, depending on the weather type.

The simulated augmentation of seasonal runoff for each 1% increase in seasonal rainfall was 5,7% for the "drier" season (rain = 328mm) and 7,2% for the "wetter" season (rain = 639mm).  Maaren concluded that augmentation in wetter years or on more responsive parts of the Vaal Dam catchment could be expected to be more rewarding, but did not attempt a full-scale quantification of this effect.

Kriel's Rainfall-runoff Relationships

Kriel (1992) performed an almost classical analysis of annual rainfall and runoff patterns for the Vaal Dam catchment for the period 1909 to 1990.  He also carefully reconstructed rainfall-runoff curves for the period 1923-45 and 1969-91, respectively, to quantify the effect of upstream water- and land-use and to illustrate the highly non-linear nature of the runoff process. His relationship for the more recent period is:

Q = R4,95/1012,42	(r2 = 0,83)

where	Q = annual runoff in mm,

R = annual rainfall in mm.

On this basis, Kriel concluded that an (arbitrarily selected) seeding-related increase of 10% in the MAP of 669 mm for Vaal Dam catchment, should result in an increase in MAR of about 60% (Grootdraai Dam catchment included).  By its very nature Kriel's work did not address reservoir yield augmentation.

Seed's Augmented Rainfall Surfaces

Seed (1992) combined the average seeding effects per storm of Table 1 with the total available sample of seedable convective storm tracks observed by the respective radars at Bethlehem and Carolina.  This enabled him to build a sample of potential mean aerial daily rainfall increases due to seeding during daylight hours.  The distribution of this sample is nearly log-Normal, whereby most seeded days (mode) show a 9% increase in rainfall, with a few days showing a maximum of around 27% and a minimum of around 3%.

Seed (1992) incorporates this daily seeding distribution in software that produces gridded (3km x 3km) daily rainfall surfaces based on interpolation of available point rainfall records. Simulated seeding and augmentation of these daily surfaces are controlled by recognition of three classes of weather systems, identified by the spatial distribution of rain, using an adapted version of a meteorologically-based classification by Court (1979):

+	General rain-days: Days on which more than 50% of grid points display at least 5 mm of rain - non-seedable (these are days with efficient raindrop-forming mechanisms and are dangerous or unnecessary to seed).

+	Dry or no-rain days: Non-seedable.

+	Scattered rain days: Rainfall distribution between the other two classes above - these were the only seedable rainfall surfaces.

Stochastic sequences of dry, scattered and general rain-days are generated using a Markov chain model, driven by the transitional probabilities between these three states in the observed rainfall surface series.  Daily rainfall surfaces are then randomly sampled from the historical record of surfaces in correspondence with the dry, scattered or general class generated for that day.  A seeding effect is generated for each scattered rain-day from the above log-Normal distribution and the areal rainfall for that day suitably augmented.  For the Nelspruit-Bethlehem target zone the increase in mean annual rainfall is found to be 7,3%.

It is of interest that Seed's potential increase in MAP translates via Kriel's rainfall-runoff equation, above, to a 41% increase in MAR.

Ninham Shand's Modelled Runoff Augmentations

Howard & Görgens (1994) of Ninham Shand Inc. undertook an intensive modelling study for the WRC of the potential response of runoff, reservoir yield and timber yield in 13 gauged catchments scattered throughout the Bethlehem-Nelspruit corridor and including parts of all four major basins in the region: the Vaal, Olifants, Crocodile and Usutu.  For each catchment the ACRU daily catchment model (Schulze et al 1989) was configured from detailed soils, land-use and land cover information and verified by comparing simulated and observed historical runoff sequences.  The majority of the verifications were relatively successful.

For the simulations, daily average sub-catchment rainfall sequences were developed from spatially distributed daily rainfall surfaces, based on available daily rainfall station records. The rainfall surfaces, historical and stochastic, seeded and non-seeded, were generated with the software developed by Seed (1992), as described above.

It should be noted that for each catchment 100 sequences of daily rainfall surfaces, each 30 years in length, were stochastically generated, using a 30-year historical daily sequence as source.  Correspondingly, 100 30-year augmented daily flow sequences were analysed, each representing current-day water-use, land-use and farm and other dams in the catchments.

For the purposes of this paper, the findings by Howard and Görgens (1994) can be usefully summarised by the median augmentation in mean annual runoff (MAR) and reservoir yield for each gauged catchment.  They showed that the average median increase in MAR due to seeding over the 13 gauged catchments was modelled as 32%, while the firm (no-failure) yield increase for a one-MAR capacity dam came out at an average median value of 27% (including net reservoir evaporation losses).  The range in median increases was 20% - 48% for MAR and 14% - 42% for reservoir yield. (As an aside, it may be noted that the average median timber yield increase due to seeding was simulated at 22%).  Howard and Görgens found that the above results were relatively constant for full supply capacities between 0,5MAR and 1,5MAR.

Potential Impacts on two pilot Vaal Dam Sub-Catchments

Since completion of the 1993 study on the above 13 catchments, which included the sizeable Blesbokspruit tributary (C1H006) of the Vaal River, the simulated augmentation of runoff from a further sizeable catchment, the Liebenbergsvlei tributary of the Vaal, flow gauging station, C8H026, has also been completed (see Figure 1).  The availability of runoff augmentation findings on two relatively large sub-areas of the Vaal Dam catchment, C1H006 and C8H026, located on opposite sides of the catchment, has created this opportunity to make inferences about the potential runoff augmentation of the total catchment.

Catchment and Modelling Details

Table 2 presents some comparative details of the characteristics of the two modelled sub-catchments as configured and of the Vaal Dam catchment as a whole.  Catchment modelling details appear in Howard and Görgens (1994) for C1H006 and in NSI (1995) for C8H026.

TABLE 2 :	Primary Characteristics of Modelled Sub-Catchments and of the Vaal/Grootdraai Dam Catchment

| Characteristic | C1H006 | C8H026 | Vaal/G.draai |
| --- | --- | --- | --- |
| Area (km2) MAP (mm) Cultivated Area (km2) Irrigation Area (km2) U/s Dam Cap. (Mm3) Area/Raingauge (km2) | 1100 700 440 50 19 138 | 4970 720 2250 20 20 190 | 38600 669 - - - - |

Seed/Non-Seed Differences in Runoff and Yield

The box-and-whisker plots in Figures 2(a) and (b) present a useful overview of the differences between seeded and non-seeded cases for MAR and reservoir (firm) yield for a one-MAR dam (net evaporation included).  The primary pointers from these plots are:

+	the pronounced upwards shift of the complete distribution of MAR's, standard deviations and different yields

+	the large range of yield augmentations (as %)

+	the relatively low median yield increases (14% and 12%), compared with the 27% average median increase over 13 catchments reported by Howard and Görgens (1994).

Figure 2(a) C1H006								Figure 2(b) C8H026

Box-and-whisker plots of MAR and reservoir yield to show increases due to simulated seeding: 100 30-year flow sequences each.

The latter point leads to three primary considerations:

-	The two sub-catchments concerned have much lower mean annual rainfalls (MAP's) than most of the other 12.  This implies less seeding opportunity because of fewer rain-days, but also lower runoff responses because of drier soils.

-	The two sub-catchments concerned are heavily cultivated, which would exert immediate consumptive demand on any additional rainfall.

-	The average annual seeding effect on these two sub-catchments comes out at about 5%, i.e. much less than the 7,3% determined by Seed (1992) for the overall target zone.  This seeding effect is dependent on the number of "scattered" rain-days identified, which, in turn, is dependent on the definition of "general" rain-days, which are non-seedable.  It is probable that in the Vaal Dam basin, for larger sub-catchments, the "general" rain-day definition (given earlier) causes too many seedable days (or at least seedable parts of catchments) to be ignored by the seeding software, and should perhaps be reviewed.  In an operational undertaking, all seedable portions of a large catchment would probably be seeded routinely.

For this paper, some comfort can be drawn from the latter point, in that it indicates that the seeding scenario under discussion is likely to be a conservative one.  Further conservatism also follows from the use here of the Görgens-Rooseboom seeding effects (Table 1) for glaciogenic seeding, despite more pronounced seeding responses reported for the current hygroscopic seeding (NPRP, 1995).

INFERENCES FOR THE VAAL DAM CATCHMENT

Augmentation of Reservoir Yield

The 100 pairs of seed/non-seed firm yields derived from the stochastic sequences for the two sub-catchments concerned (summarised in Figures 2(a) and (b)), have been scaled by MAR and reorganised in frequency distributions as shown in Figure 3.  The recurrence interval (RI - in years) of failure to deliver a specific annual yield is indicated for the range of RI's normally of interest in water resources planning, i.e. 1:10 to 1:200 years. The  RI is determined from the well-known probability relationship:

r	=	1 - (1 - 1/T)N

where		T	= annual recurrence interval = 1/p

p	= annual non-exceedence probability

N	= number of years in sequence (30 in this case)

r	= total risk of non-exceedence.

Expressed as %MAR, the increments in firm yield are reasonably similar for the two sub-catchments concerned at any particular RI of failure.  It is therefore plausible to derive a set of average firm yield augmentations due to cloud-seeding (for current catchment conditions) for the range of RI's of interest, as shown in Table 3.  Given that these figures were determined for sub-catchments that form a substantial part of the Vaal Dam catchment, it would not be unreasonable to apply them to the whole Vaal Dam catchment, with the results as shown in Table 3.  For this purpose, the Grootdraai catchment is included in that of Vaal Dam, and the potential augmentation is therefore valid for the combined yield of the two dams.  The ratio of combined full supply capacity to combined MAR is about 1,4 and it is assumed that the one MAR capacity dam results in Figure 3 are still applicable.  The corresponding combined MAR is taken as 1900 Mm3 for current catchment conditions (BKS/SS&O, 1989).

TABLE 3  :	RANGE OF POTENTIAL YIELD AUGMENTATIONS

| RI (Years) | Yield Augmentation (% MAR) | Add. Vaal/Grootdraai Yield (Mm3/a) |
| --- | --- | --- |
| 1:10 1:20 1:50 1:100 1:200 | 13 11 9 8 7 | 247 209 171 152 133 |

The analysis suggests substantial gains in Vaal System yields over the full range of reliability of supply normally of interest in water resources planning.

Unit Cost of Augmentation

The recent development in the NPRP in the direction of quasi-operational area seeding experiments in the Tzaneen region makes a quantification of the true costs of large-scale operational cloud-seeding quite achievable.  For this paper we relied on feed-back from members of the NPRP on likely costs and have taken a certain amount of licence to develop a pilot cost scenario to provide a basis for a discounted cost comparison with the VAPS options currently under investigation.  The primary discounting assumptions are in line with the VAPS analysis and our costing scenario is as follows:

-	Annual augmentation - 150 Mm3.

-	Start-up capital investment -  R60 million spread over first three years.

-	First effective augmentation after three years.

-	Annual running costs - R30 million.

-	Base date of costs - 1994.

-	Real rate of interest - 8% p.a..

-	Discount period - 45 years.

-	VAT excluded.

The unit reference value (URV) concept is used for the discounting.  URV comprises discounting both the stream of annual capital and running costs over the 45 years and the stream of annual volumes of water delivered, and then expressing the two results as a ratio.

Figure 3 :	Standardised yield-reliability relationships for seeding of sub-catchments C1H006 and C8H026

For the cloud-seeding augmentation the URV is 24 cents/m3, while the URV's of the four leading VAPS options range from 69 - 136 cents/m3 (Little, 1995).  The cloud-seeding augmentation appears to be remarkably competitive.

Closure

Mindful of conservatism imbedded in our approach in terms of seeding opportunity classification, the use of glaciogenic seeding increases rather than the more promising hygroscopic-related ones and using a yield increase from the low end of the range (R.I.=1:100 years), we must conclude that augmentation by cloud-seeding appears to offer a competitive option for Vaal System augmentation.

Additional to more careful costing, further crucial issues that await clarification through feasibility-level studies are: How achievable, in a practical sense, is routine operational cloud-seeding at the required scale?  And: What are the legal and public acceptance issues associated with large-scale operational cloud-seeding?

Acknowledgements

This study was executed under the auspices of the Water Research Commission and benefited from assistance from the Hydrology Directorate of DWAF and the Computing Centre for Water Research at the University of Natal in Pietermaritzburg.

References

BKS/SSO (1989) Vaal River System Analysis: Vaal Dam subsystem analysis. DWAF

Report No. P C000/00/7187, Department of Water Affairs, Directorate of Planning, Pretoria.

Court, A.P. (1979) Rainfall characteristics of classification systems used by the 	BEWMEX project.  Progress Report No. 14, SA Weather Bureau, Department of Environment Affairs, Pretoria.

Görgens, A.H.M. and A. Rooseboom (1990) Potential impacts of rainfall stimulation in 	South Africa: A planning study.  Report by Ninham Shand Inc. to the Water Research Commission, Pretoria.

Howard, G.J. and A.H.M. Görgens (1994) The potential impacts of rainfall stimulation 	on water resources and forestry in the Nelspruit-Bethlehem target zone.  WRC Report No.439/1/94 by Ninham Shand Inc. to the Water Research Commission, Pretoria.

Kriel, J.P. (1992) Die jaarlikse reënval, afloop en verhoging in afloop met verhoogde

reënval vir die Vaalrivier-opvanggebied by Vaaldam. Water SA, 18 (2), 107-120.

Little, P.R. (1995) Personal communication, Ninham Shand Inc, PO Box 1347, Cape Town, 	8000

Maaren, H. (1984) Runoff augmentation and weather types: Practical implications. 	Proceedings of the South African National Hydrological Symposium, Technical Report No. TR 119, Department of Environment Affairs, Pretoria, 68-80.

Mather, G.K. and G. Morgan (1990) Programme for Atmospheric Water Supply, Phase 	Two, 1987 - 1989.  Volume 2: Aircraft and Radar measurements and cloud seeding experiments. WRC Report No. 133/6/90 by Cloudquest (Pty) Ltd to the Water Research Commission, Pretoria.

Mather, G.K. and D.E. Terblanche (1993) The National Precipitation Research 	Programme. WRC Report No. 133/8/93 by Cloudquest (Pty) Ltd and SA Weather Bureau to the Water Research Commission, Pretoria.

NPRP (1995) Progress Report, July 1994 - June 1995, National Precipitation Research 	Programme.  Water Research Commission, Pretoria.

NSI (1995) Untitled draft consulting report by Ninham Shand Inc. to the Water Research

Commission, Pretoria.

Schulze, R.E., W.J. George, S.D. Lynch and G.R.Angus (1989) ACRU-2: Theory User

Manual. Dept. Agric. Engineering, Univ.of Natal, Pietermaritzburg.  ACRUReport No. 36.

Seed, A.W. (1992) The generation of a spatially distributed daily rainfall database for

various weather modification scenarios.  WRC Report No. 373/1/92 by Directorate of Hydrology, DWAF, to the Water Research Commission, Pretoria.