A Qualitative Approach to River Process Modelling

C R Nicolson & C S James

Centre for Water in the Environment

Dept of Civil and Environmental Engineering

University of the Witwatersrand

Johannesburg, South Africa

In South Africa a great deal of emphasis is being placed on how to manage water allocations for environmental purposes.  Questions are being asked concerning the minimum amount of water required for sustaining riparian vegetation and faunal habitats, as well as for other more general conservation problems.  As increasing pressure is placed on our water resources, it becomes more and more likely that the natural flow regimes will be altered significantly, and in order to predict the impact of changes to the flow regimes of our rivers, some knowledge of the river geomorphology is required.  Are South African river systems changing?  If they are, then how, why, and over what time scales are changes taking place?  These questions are not trivial, since river systems change over a wide range of spatial and temporal scales, and answers at one scale might not be applicable at other scales.

At one extreme of the spatial scale continuum lies the river basin, which may cover thousands of square kilometres, and which  changes over geological time.  At the other extreme lies the microhabitat at a particular cross-section, where local velocities and flow depths can change in a matter of minutes to hours.  Our ability to model morphological change varies significantly.  For rivers which have reached equilibrium with their sediment and water load (so-called 'regime' rivers), we can predict their average morphological characteristics such as width, depth, and sinuosity with empirically derived regime relationships.  These, however, are only appropriate on a large scale, and because they are not based on the physical processes of river change, can tell us nothing about the rates of change over time.

At shorter time-scales there are unsteady-flow models which can predict the passage of a flood through a channel, giving water levels at time-intervals of less than a minute.  Some of these models (e.g MOBED (Krishnappan, 1981) and FLUVIAL-11 (Chang, 1984) incorporate sediment-routing and can predict changes in bed level and, to a limited extent, width as well.  Unlike regime equations, these models are process-based; they attempt to identify and simulate the actual river processes of flowing water, sediment transport, erosion and deposition.  This is a major strength, since it allows the prediction of the rate of adjustment through time.  However, attempts to apply computational models have neglected essential processes through an inability to understand them fully or to describe them mathematically.  Our understanding of the bank processes and the meander processes is particularly weak, and so to incorporate such processes into models generally involves making major assumptions.   With vegetation, its influence on morphology is widely recognised, but has nowhere been accounted for in dynamic models due to the difficulties in quantifying its effect mathematically.

Our ability to model river change is weakest at intermediate scales, namely tens of kilometres over several years.  At these scales, the data requirements for computational process-basedmodels become impractical, and our understanding of the relevant fluvial processes is not sufficiently advanced for detailed modeling.  Unfortunately, problems at this scale are particularly important, since most river modification works (dams, bridges, channelisation etc) and catchment management actions (land use, abstraction, pollution control etc) have effects which are seen at these spatial and temporal scales.

There is a strong case then for investigating modeling approaches which can overcome some of the limitations of conventional computational modeling.  This paper describes one such approach, namely qualitative rule-based (QRB) modeling.  At the heart of this paradigm is a simple, but powerful idea, one that draws a parallel between precise numerical equations and less detailed qualitative rules.

States and Rules vs. Real numbers and Equations

The real world has infinite detail.  From a great distance above the earth, a major river will look like a thin ribbon, but as we get closer, more and more detail can be seen.  Flying at a low altitude we would be able to make out the left and right banks of the river, and we could see how the width changes along the course of the river.  Standing on the bank we may see something of the shape of the cross-section and bedforms on the river bed.  Looking even more closely, we would notice how the water surface fluctuates minutely up and down, and how individual sediment grains are transported along the bed.  At a a microscopic levl we might see minute organisms or pollutants in the water, and it would be clear that the sediment grains all have different shapes and sizes.  We could even go beyond this, and look at the molecular structure of the various compounds.  The point is that the real world has as much detail as we care to see.

It is impossible to capture all of this detail in a model of the river, and depending on our purpose not all of it is necessary to include.  Generally, the features of the river which we put into the model are shaped by the objectives of the model, so if we are interested in the aquatic life, the quality of the water is an important feature, but if we are only interested in the sediment transport capacity, the pollution levels are unimportant.

The most common way to describe features in a model is to use numerical variables, with equations to assign values to these variables.  If, for example, we are concerned with the flood levels of a river, then the flow depth is an important feature to capture.  We could describe it as a real variable which has a name (depth, for example), and which takes on some real value (say the depth of flow measured in metres at the midpoint of the river).  To assign a value to this variable in the model, we could use a flow resistance equation (such as the Manning equation) which relates the flow depth to the discharge for a given channel.  Conventional river models use several equations such as this (e.g. flow resistance, sediment transport, continuity of water mass), usually in the form of partial differential equations (PDEs).  To solve these equations simultaneously generally requires numerical methods, and is computationally intensive.  River models such as HEC-6 (Hydrologic Engineering Centre, 1977), MOBED (Krishnappan, 1981) and IALLUVIAL (Karim and Kennedy, 1982) are all PDE-type models.

Often, however, we describe things in lower resolution than these models do.  Perhaps it is unnecessary for the model to predict flow depths to millimetre accuracy, and it is sufficient to know that the flow depth is either within the channel or is overtopping the banks.  These are qualitative descriptions which describe some quality of the flow depth.   In this case we could have a descriptive variable (again, with the name depth), which could have one of two states, namely in-channel or overtopped.  To predict changes from one state to another, simple IF-THEN type rules are applied in the form IF (some condition is met) THEN (the value of the state variable changes from one state to another).  In our example, the rule may be IF (discharge > 50 cumec) THEN (depth = overtopped).  This way of handling variables is called qualitative, rule-based modeling (Starfield, 1990)

There are obvious similarities in the two approaches.  Both paradigms use variables to describe some feature of the real world, and if we think of equations as simply being 'rules' which govern the assigning of values to certain variables, both paradigms have 'rules' too.  The primary difference between the PDE approach and the QRB approach lies in the level of resolution of the variables.

An extension of the idea of QRB modeling is the construct of frames (Starfield et al. 1993).  In many systems, the temporal dynamics can be partitioned into several distinctive states.  For example, if we were to monitor a forest for a very long time, we might notice distinctive stages of plant succession.  Ecologists could characterise these states, and establish rules which describe when the forest would switch from one successional stage to another.  To model this system, each stage is represented by an independent sub-model called a frame.  This sub-model then simulates the processes that have been identified as the key processes of interest within that frame.  The model watches for a combination of conditions that precipitate a switch from one frame to another, and, when they are met, it stops using the one sub-model and begins to use another.

These innovations, qualitative modeling and frame-based modeling, have potential application in the field of fluvial geomorphology.  As an example to illustrate some of the concepts and how we might use them, we will consider the interaction between vegetation and river channel characteristics.

A hypothetical example: The influence of vegetation on river morphology

a) Background

River managers have long known that certain tree species and grass can play a role in stabilising river banks, and hence in modifying river behaviour.  But this is not the only way in which vegetation influences river morphology: another effect which has been observed is that of reed beds growing within the channel itself.  It has been suggested that reeds may be important for the stability of in-channel bars (also known as core bars), and a conceptual model has been put forward for the way in which core bars develop.  During periods of low flow, reeds begin to grow in the river bed, and if there are several consecutive seasons of low flow, these reeds can become well-established.  When the stage rises, the sediment transport rate increases and the reeds act as a sediment trap, thus forming a bar in the channel.  The reeds stabilise this bar, and continue to trap more sediment when the flow is high.  The areas immediately upstream and downstream of the bar will be zones of deposition, increasing the bar's length.

This hypothesis, then, proposes a mechanism whereby the river can change from a single-thread channel to a channel with core bars, dividing the flow into two or more sub-channels.  This is useful if we want to model the morphological changes within a particular reach of river, since we could identify single-thread and braided as two morphological states, or frames, and use the hypothetical process outlined above to predict when the river changes from one state to another.

The model described in the next section is hypothetical.  Its purpose is to illustrate some of the principles of QRB modelling, rather than to describe and predict river change in a rigorous way.  However, to demonstrate the sort of applications which this type of model could have, the questions which it is designed to answer are real ones:

1. How easily will a particular channel morphology undergo change?

2. If it does change, what are all the possible changes (and probabilities of change) which it could undergo?

b) The model

The time-step chosen for the model is to be one year.  The driving variables are:

stage:

a qualitative variable which describes the average flow depth during the year.  It can be low, average or high, depending on whether it is deep enough to submerge the reeds.

flood:

can be either yes (if there is at least one significant flood event during the year) or no (if there are no big flood events).

Given the complexity of river processes it is simplistic to assume that only these two driving variables influence the morphology, but for our purposes they will be adequate to demonstrate the principles of QRB modelling.

The model has three channel types, or frames, based of the river's planform characteristics: Straight, Sinuous and Braided.  The Straight and Sinuous frames represent single-thread channels, while the Braided frame represents a channel which splits into two around a core bar (Figure 1).

The Straight frame.

Although it is unusual for a reach of natural river to be straight for a distance exceeding 10 channel widths (Leopold and Wolman, 1957), some rivers are constrained by their underlying geology and are unable to meander.  The Sabie River is an example of this, having several reaches which could be described as straight in planform (Vogt, 1992).  For the purposes of this model, the Straight frame represents a river reach in which a bedrock macro-channel is partly filled by sediment deposits.  The river channel is incised into this alluvium (see the cross section in Figure 1).  The reach is of the order of a kilometre long, and could be idealised by a trapezoidal cross-section.

The morphology switches from Straight to Sinuous if large enough bars are deposited alternately on either side of the channel.  It is well documented in the literature that straight channels tend to develop meander tendencies by depositing small bars at regular intervals on alternate sides of the channel (for a review of this effect, see Jaeggi, 1984).  To represent this, we introduce a variable bars to this frame, which can have the states none, alternating or core depending on where the bar is placed in the channel.  If we begin with  bars = none , when the stage is average, the channel automatically develops small alternate bars as it transports and deposits sediment, but if there is a flood during that year the river washes out any developing bars, and the model will reset bars to none.  The bars referred to in this case are relatively small, just below the water level we describe as average.  If the water level rises the following season to high, the bars will be submerged, and the extra stream power of the deeper flow will wash the bars out (in a similar way to a flood).  A similar rule applies to core bars:  beginning with bars = none, when the stage is low the river channel tends to divide around small sediment deposits in mid-channel.  Again, these small core bars can be washed out by a flood or by the higher stream power of deeper flows.

To investigate the role of vegetation, we need to introduce a new variable, reeds, which can be none, young, mature or stressed.  This applies to the state of the vegetation on the bars described above.  The normal cycle of growth is from no reeds to young reeds to mature reeds in three consecutive time-steps.  This cycle can be short-circuited in two ways: either by floods or by a high flow level.  Under a flood, reeds which are not well established can easily be destroyed:  this is represented in the model by the variable reeds being reset from young to none if flood=yes.  If the water level is high, the reeds will be submerged and this subjects them to stress, making them more vulnerable the following year:  to represent this, if stage=high and reeds is either young or mature, reeds is set to stressed.  In the following time-step, stressed reeds can either regenerate to become young (if flow is low with no flood), or they could be destroyed (if there is a flood or if stage is average or high).  The vegetation cycle is summarised in Figure 2.

Since we are interested in the stabilising effect of reeds on the bars, our rules should involve the vegetation-reed interaction in some way.  According to our conceptual model,the reeds act as a sediment trap, assisting the development of bars.  This is represented in the model as follows:

i)	If reeds = (none or young or stressed), either a flood or a high flow level will wash the bar away.  The reeds have no significant stabilising influence.

ii)	If (reeds = mature) and (stage = average or flood = yes) then the reed bed traps sediment, thus increasing the size of the bar.  This triggers a change of frame from Straight to Sinuous (if the bars are alternate) or from Straight to Braided (if the bars are core bars).

This completes the description of the Straight frame.

The Sinuous frame:

This frame represents the same length of river reach as the Straight frame does.  It may seem inconsistent to describe the planform morphology as Sinuous after explaining that the sinuosity is constrained by the underlying geology, but the meandering all takes place within the confines of the bedrock macro-channel (Figure 1).  The characteristic feature of the Sinuous frame is the alternating bars which cause the river to meander back and forth within the macro-channel.  To describe these sedimentary features, we introduce the variable barsize, which refers to the size of the alternating bars and can be either medium or large.  The other variable is again reeds, with the same four possible states as in the Straight frame.

The distinction between medium and large alternating bars is made on the basis of water level.  Medium bars are exposed at low flow depths and will be just below the water level when the stage is average.  Large bars will only be submerged by high flows.

The interaction between the reeds and the sediment transport follows a similar pattern to the Straight frame.  Unvegetated, the bars have a low resistance to erosion, and a flood will simply erode the bars to a much smaller size.  This is represented in the model by a switch from Sinuous (with barsize = medium) to Straight (with bars = alternating).  This switch could also take place if the reeds are stressed and stage = high and flood = yes.  If the reeds are mature and there is a flood or a season of high stage, the reeds trap sediment and the bar increases in size from medium to large.  Table 1 summarises the rule-base for the Sinuous frame, listing the state changes for every combination of the variables and flow conditions.

The Braided frame:

The same rules which applied to the Sinuous frame were used in the Braided frame.  This was done for the sake of simplicity, since most of the logic for the rules applies equally to alternating and to core bars.

c)  Results

The model can be used in one of three different running modes.  First, one can perform a simulation in which values for the driving variables are entered by the user at each time-step.  This allows one to go slowly through the model, step by step, checking for 'bugs' in the programming or for unrealistic output (which reveals logical inconsistencies or faults in the rule-base).  By checking the model in this way, the modeller can gain confidence in the correctness of the model's processes.

Having tested the output of the model in this way, the second way to use the model is run it for several timesteps (say 20), with the computer generating values for the driving variables at each time-step from specified probability distributions.  This introduces stochasticity into the model, hopefully simulating the randomness of natural events such as floods.  The user's task in this case is to define the probability distributions at the beginning of the simulation, and to examine the output to ensure that the model is behaving reasonably.  Figure 3 is a sample output generated like this, and shows the morphology changing from Straight to Sinuous as the alternating bars become vegetated and trap sediment.

It is important to realise that Fig 3 shows results from a single replicate only.  Running the model again, with identical probability distributions,will produce different results depending on the timing of floods and on the sequence of water levels in successive time-steps.  Because of the stochastic nature of the model, if one wants to detect trends in the

| Reeds | Barsize |  |
| --- | --- | --- |
|  | Medium | Large |
| None | A: Medium + Young B: Medium + None C: Medium + None D: * Str Frame (Alt + None) E: * Str Frame (Alt + None) F: * Str Frame (No bars + None) | A: Large + Young B: Large + Young C: Medium + None D: Medium + None E: Medium + None F: * Str Frame (Alt + None) |
| Stressed | A: Medium + Young B: Medium + Young C: Medium + None D: Medium + None E: Medium + None F: * Str Frame (Alt + None) | A: Large + Young B: Large + Young C: Large + None D: Large + None E: Large + None F: Medium + None |
| Young | A: Medium + Mature B: Medium + Mature C: Medium + Stressed D: Medium + Stressed E: Medium + Young F: Medium + None | A: Large + Mature B: Large + Mature C: Large + Stressed D: Large + Young E: Large + Young F: Large + None |
| Mature | A: Medium + Mature B: Medium + Mature C: Large + Mature D: Large + Mature E: Large + Mature F: Large + Mature | A: Large + Mature B: Large + Mature C: Large + Stressed D: Large + Mature E: Large + Mature F: Large + Stressed |

Table 1:	Rules for state changes in the Sinuous frame.

The rows represent the state of the reeds at the beginning of the time step, and the columns the size of the bars.  For each combination of these variables (2 x 4 = 8 combinations) , there are six possible combinations of the input variables, stage and flood.  These are labled A to F:

A: stage = low, flood = no

B: stage = average, flood = no

C: stage = high, flood = no

D: stage = low, flood = yes

E: stage = average, flood = yes

F: stage = high, flood = yes

The asterisk (*) represents a switch from the Sinuous frame back to the Straight frame.

morphology it is essential to look at the output from large numbers of replicates.  This is the third way in which the model can be used, and it is a useful way to examine how the morphology responds for example to different probabilities of floods or water level.  Figure 4a is an example of the output one can obtain from running large numbers of replicates of the model.  The graph shows the probability of switching from the Straight frame to Sinuous or Braided (and of not switching at all) within 40 years, under different probabilities of high, average and low flows.  This helps to give us some idea of the morphological robustness of the Straight frame under various flow conditions.  Figure 4b is a similar graph, but shows the probability of switching from the Sinuous frame back to Straight.  The lines show the influence of vegetation:  replicates for the lowest dashed line were started with unvegetated, medium-sized bars, while the solid line represents replicates of mature-reed-covered medium bars.  This gives us some idea of the reversibility of the morphological change, and also helps to clarify the effect of vegetation.

Discussion

The purpose of this paper has been to introduce a modelling paradigm which may be useful for modelling certain aspects of river morphology.  To conclude, there are a number of comments to be made about QRB modelling in general, about the reed-bar model in particular, and about the suitability of the QRB paradigm for modelling river processes.

a) Some benefits of QRB models

i)	QRB models are easy to understand, and therefore make good communication tools between experts and non-experts in a particular field.  One example of this is the communication between resource managers and modellers: there are no complicated formulae, and managers can easily understand and comment on the rules which drive the model.  This may give them greater confidence in the results, since they understand the way in which it works.

ii)	The paradigm lends itself to rapid prototyping, so ideas can be tested fairly quickly and easily.  The model described in this paper took an afternoon to plan and a day to code-up on computer.

In general, models are a useful way to conduct thought experiments.  They allow us to explore the consequences of a hypothesis we have, such as the hypothesis of reeds influencing morphology.  Where our hypotheses are qualitative in nature, qualitative models will often be the best way (if not the only way) to test these hypotheses.  Building a PDE-type model to explore the stabilising effect of reeds would require equations to predict the velocity distributions in reed beds as well as the rate of sediment deposition and the rate of reed density increase under different flow conditions.  Each of these requirements would be a major research undertaking in itself.  Our model uses intuitive, qualitative understanding of these processes and then explains the consequences and outcome of our qualitative conceptual understanding.

b) The reed-bar model itself

The first thing which must be said of this model is that it is far too simple to yield interesting results.  Because there are only two processes built into the model (reed growth and bar development), the results it gives can probably be imagined without recourse to a formal model.  Generally the human brain is able to juggle two or three ideas simultaneously, but is unable to extend this to four or five interacting ideas.  A model can test the implications of five interacting processes as easily as it can handle the two simple processes of our model above.  Although the model is very simple, the concepts of QRB are still demonstrated, and this really is the purpose of the paper.

A second weakness is that the model does not utilise the concept of frames to nearly its full potential.  The novelty of the frame-based approach lies in the idea of constructing separate sub-models for each frame, and using rules to decide whether conditions have changed so as to trigger a switch from one frame to another.  At any time only one simple model is operating; it is totally independent of all the other models and does not call on them or interact with them in any way.  The usefulness of the frame-concept is that each frame can have different processes: a bedrock dominated channel may behave quite differently to an alluvial single-thread channel, and this can be captured by modelling different processes in the different frames.  The model in this paper uses the same basic processes in the Sinuous and the Braided frames, so does not fully exploit the potential of frame-modelling.  With a more sophisticated conceptual model, frames may well be both necessary and useful.  We chose to include frames in our example in order to demonstrate the basic principles.

In spite of these criticisms of the model, it has still been valuable in several ways.  First, it forced us to think more clearly about all the possible paths of morphological change from one channel type to another, and has given us some idea of the reversibility of these changes.  Second, it focused our attention on the mechanisms which drive change, and in particular helped us to make a start on testing one hypothetical mechanism (i.e. the influence of reeds).

Most importantly, however, the simple reed-bar model shows the kind of input, representation, processes and output which we can use for modelling morphology.  It turned out to be a valuable learning experience in QRB modelling, giving some idea of the kind of questions a qualitative model can answer.

c) Modelling river processes with QRB

The problem of predicting morphological change is complex.  Often we do not have a long enough record from which we can correctly infer an understanding of the processes, and even if we did understand the processes, they may not be equally applicable at another point further down the river.  Dealing with the issue of uncertainty in predicting river change Schumm (1983) concluded that "prediction in geomorphology is of low resolution".  In slightly different terminology, we could say "prediction in geomorphology must be essentially qualitative."

This is especially true when the predictions required are at scales of tens of kilometres and over years to decades.  The data requirements and computational effort to run QCI models at these scales make them impractical, and it is quite possible that the processes which the models use are not applicable at these larger scales.

Qualitative, rule-based modelling seems to be a useful paradigm for a certain class of river problems at intermediate to large scales.  As long as enough is known to construct a rule-base, and as long as the level of detail is carefully matched to the objectives of the model, it is almost certainly likely to be useful to build QRB river models.

References

Chang, H H, (1984), Modelling of River Channel Changes, J. Hydr. Eng 110(2):157-172

Hydrologic Engineering Centre, (1977), HEC-6: Scour and deposition in rivers and 	reservoirs, Users' Manual, Technical Report, U.S. Army Corp of Engineers, Davis, California

Jaeggi, M N R,(1984), Formation and effects of alternate bars, J. Hydr. Eng 110(2):142-156

Karim, M F and Kennedy, J F, (1982), IALLUVIAL: A computer-based flow- and sediment- 	routing model for alluvial streams and its application to the Missouri River, IIHR Report 250, Iowa Institute for Hydraulic Research, Iowa City

Krishnappan B G, (1981), MOBED: Unsteady, non-uniform, mobile boundary flow model,

Users' Manual, National Water Research Institute, Burlington, Ontario

Leopold L B and Wolman, M.G, (1957), River Channel Patterns - braided, meandering and 	straight,  U.S.Geol.Survey Prof. Paper 282

Schumm, S A, (1983),  River Morphology and Behaviour: Problems of extrapolation,

in Elliot, C.M. (ed), River meandering: Proceedings of the Conference Rivers '83, pp16-29, ASCE, New York

Starfield, A M, (1990),  Qualitative, rule-based modeling, Bioscience 40:601-604

Starfield, A M, Cumming, D H M, Taylor, R D and Quadling, M S  (1993),  A Frame-

Based Paradigm for Dynamic Ecosystem Models, AI Applications in Natural Resource Management 7(2):1-12

Vogt, I, (1992),  Short-term Geomorphological Changes in the Sabie and Letaba Rivers in 	the Kruger National Park,  unpublished M.Sc. dissertation, Univ of the Witwatersrand, Johannesburg