Stochastic System Analysis and Manufactured Hydrology

- A New Perspective

1G G S Pegram, 2A H M Görgens, 3F A Stoffberg & 2P du Preez

1Civil Engineering, University of Natal, P Bag X10, Dalbridge, 4014

2Ninham Shand Inc, PO Box 1343, Cape Town, 8000

3Department of Water Affairs and Forestry, P Bag X313, Pretoria, 0001

Introduction

The drought in the early 1980's led to the Vaal River System Analysis and the accepted use of stochastic hydrology in the quantification of the reliability of large water resource systems.  The methodology documented by Basson et al (1994) has become a standard tool used by many water resources engineers and practitioners.  There is a danger that because of the routine, almost recipe-like format of the analyses, a certain amount of complacency could creep into the processing of the results in order to get to the reporting stage.  This tendency should be arrested because of the large amount of work using these procedures on the Vaal Augmentation Planning Study (and other system analyses) currently being done for the Department of Water Affairs and Forestry and others.

It is the aim of this paper to highlight some problems and their solutions which arose in a particular study - that of the  Algoa System undertaken for the Department of Water Affairs and Forestry and the City of Port Elizabeth by Ninham Shand Inc.

To be more specific, it was found that when the last available nine years of recorded hydrological data were used in the analysis of the Algoa System, a large change in the estimated reliability of the System occurred.  This led us to take a careful look at the performance of the stochastic hydrology, thence via an assessment of the causes for the apparent anomalies back to the recorded and manufactured hydrological sequences and therefore to the catchments from which the information came.

The conclusions we came to are that the observed sequences were legitimate and the way the hydrology was manufactured in those catchments where few or no data were available using the Pitman model, in some cases did not generate low enough flows during the dry periods.  This might be related to rainfall sequences used.

The lesson learned from this exercise was that the  stochastic hydrology and its use in the definition of long term characteristic curves for the system and sub-systems, when examined critically, is a helpful indicator of those areas where greater understanding of the underlying hydrological processes is not only desirable, but necessary to avoid error.

The Evidence of the Paradox

The map of the Algoa System's catchments is given in Figure 1.  With MAP varying from 1000 mm in the coastal forested catchments like the Elands (catchment number 42) to 300 mm in the inland Baviaans (catchment number 11) a mere 50 km away and a topography

characterised by long valleys in heavily folded sand-stone and shale deposits, the system is bound to produce some interesting hydrological puzzles.  The documentation of the hydrology, which was prepared in a conventional manner by means of the monthly Pitman model, taking into account temporally changing land- and water use, can be found in DWAF Report No. PM 000/00/0893 (NSI,1994).

As is well known, long term system yield curves are generated using stochastically generated flow sequences fed through a system simulation program.  The break-point where the yield just does not support the demand defines a point on the yield characteristic curve, an example of which is given in Figure 2. The historical flows used to generate the stochastic model parameters cover the period 1927-1991.

The first sign of difficulty which prompted the investigation reported here appears in Figure 3.  This figure shows the long term yield characteristic curves for the Krom and Diep River sub-system feeding Churchill and Impofu dams, respectively numbered catchments 22 and 21.  Two different yield curves were produced: for flow sequences ending in 1983 (when the last firm yield analysis was done - NSI, 1983) and for those ending 1991. It will be seen that analysis based on the longer flow series, including a serious recent drought (1983-91), has lowered the sub-system yield by between 20% and 25%. This decrease is of course also reflected in the change in firm yield ("HFY") derived from the two historical sequences of differing length, i.e. from 54 to 31 Mm3/a. Figure 3 shows further that addition of the 1983-91 drought increased the recurrence interval of failure of the HFY from a pedestrian 1:50 years to a remarkable 1:1000 years.

Uncomfortable questions now arose: Are the stochastic parameters for this region so fragile that addition of 9 years to a 56 year sequence can completely redefine the yield-reliability characteristic?  Why did the 400 regenerated stochastic sequences based on the 56 year sequence not trap equally severe low flow spells? Is there a snafu somewhere in the 1927-91 hydrology?

We started off by building some understanding of the critical low flow behaviour of components of the Krom-Kouga system: Figures 4 (a) to (h) are a selection of the yield-capacity curves produced by the GENTST program, based on the extended 1927-91 naturalised sequences used in the system analysis model.  Most of the runoff in the Kouga catchment (i.e.into Kouga Dam) is summarized by the curves in Figures 4(a) to (d) which are respectively the curves for catchments numbered  11, 12, 13, and 1311.  The first, 4(a) is typical of the bulk of the dry interior catchment responses; 4(d) is typical of the many small sub-catchments in the headwaters on the coastal divide.

What characterizes these four curves is the behaviour of the historical yield capacity curves when compared to their stochastic offspring.  Catchments 11 (and others in the interior such as 10) and to a lesser extent 13, respectively  depicted in Figures 4 (a) & (c), show that the historical curves lie above the  median of the stochastic curves.  This behaviour is symptomatic of historical sequences which are either negatively correlated relative to their stochastic offspring, or have shallower than normal critical flow sequences.  That for 1311 is neutral so seems well-

Figure 2 :	Long-Term Characteristic Curve : Churchill/Impofu Sub-System

(1927 - 1991 Stochastic Parameters)

Figure 3 :	Long-Term Characteristic Curve : Churchill/Impofu Sub-System

(1927 - 1983 and 1927 - 1991 Stochastic Parameters)

behaved.  The odd one out is that for catchment 12 in Figure 4(b), where the historical curve lies well below the median stochastic curve.  This could be due either to positive correlation which has not been properly modelled or to an inordinately deep historical critical sequence.  Note the flatness of all the curves indicating dry catchment response with a consequently relatively high coefficient of variation.

Figure 4(e) shows the yield-capacity curves for the Loerie Dam input, catchment 14.  Again we get the symptom of a deep critical flow sequence.  This record is an observed one spanning the wet and dry years of 1970-90, extended backwards by use of the Pitman model.

The last three figures in the set are 4(f), (g) and (h), respectively the incremental catchment for the Diep River above Impofu Dam and below Churchill Dam (catchment 21), the catchment above Churchill Dam (catchment 22) and a neighbour of 22, the small pristine coastal-forest covered Elands catchment 42. These were chosen because the records statistically closest to 22 are 12 and 42, with which it is cross-correlated with coefficients 0,86 and 0,91 respectively.  The periods when flow was recorded for these catchments are:

●	21: 1983-91

●	22: 1948-91

●	42: 1961-91

All were extended backwards using the Pitman model, 21 from a very dry calibration period.  The consequence is that the capacity yield curve for 21 appears reasonable but it is much flatter than its upstream 22 which is the real maverick.  By contrast the curve for 42 is reasonably well behaved and much steeper, being a wetter coastal catchment.

The questions become: why are some of the incremental sub-catchments of the Kouga system (notably 10, 11 & 13) behaving differently from each other and from 22?  Where does 42 fit?

Unravelling the Historical Puzzle

The constituent sub-catchments of the Kouga System are shown in Figure 1.  The first step was to examine the relation between 12 and 22 in more detail.  The trusty tool of the hydrologist, the double-mass (or cusum) plot of the overlapping recorded periods for gauges 12 and 22 appears in Figure 5(a), with a  scatter plot of the corresponding data in 5(b), where the outlier is identified as being in 1967.

For comparison, the cusum and scatter plots of 42 vs 22 appear in Figures 5(c) and (d).  The contrast between the two sets is marked.  The first shows a reasonably linear relation, the second exhibits a strange non-linearity associated with the low flows, particularly evident in the drought in the last years which appears as a kink in the top right corner of Figure 5(c).  This is high-lighted in what appears to be the superimposition of two different inter-relationships as shown in Figure 5(d).

Figure 5a :	Double-Mass Plot : Naturalised Runoff : 		   Figure 5b :	Scatter Plot : Naturalised Runoff :

Sub-catchments 12 vs 22 (1961 to 1991)						Sub-catchments 12 vs 22 (1961 to 1991)

Figure 5c :	Double-Mass Plot : Naturalised Runoff :		 Figure 5d :	Scatter Plot : Naturalised Runoff :

Sub-catchments 42 vs 22 (1961 to 1991)						Sub-catchment 42 vs 22 (1961 to 1991)

The next set is the rainfall/runoff plots for catchment 22: the cusum and scatter plots for years 1948 to '91 appear as Figures 6(a) and (b) while the scatter plot for the 1927-'91 extended sequence appears in Figure 6(c).  Figures 6(a) and (b) echo 5(c) and (d).  In fact 6(a) shows (like 5(c)) that proportionately much less runs off from catchment 22 in dry periods than does in wet periods, and that this is a feature of the record as a whole, but only stands out in the long drought.  The scatter plot in 7(b) shows the non-linearity in a marked way, whereas the scatter plot for the extended sequence seems to smudge the different regimes.

There is clearly support from both the rainfall on catchment 22 and the runoff from catchment 42 for the non-linear conjecture.  Is this supported by better evidence than that of the correspondence between 22 and 12?

This question is asked because the only useful “gauge” on the Kouga catchment is Kouga Dam, whose record spans 1961-91.  Nearly all the remaining sub-catchments' incremental records were manufactured using the Pitman model, using the cumulative inflows into Kouga Dam for calibration.  The remedy is to compare the flows into Churchill Dam with those into Kouga Dam.  The cusum and scatter plots of these appear in Figures 7(a) and (b), giving a better relationship than that between 22 and 12 evident in Figures 5.

There appear to be two sets of conflicting hydrological relationships that catchment 22 has with its surrounding catchments.  The two sets are: on the one hand, 21 and 42 with which 22 seems to be dissimilar and on the other hand 14 and Kouga Dam catchment as a whole, with which 22 seems to have similarities.  The reason for these apparent enigmas stems from the following:

●	on the one hand

21 - the record was extended based on a short dry  period

42 - is a verdant, coastal, high rainfall catchment

●	on the other hand

14 and Kouga Dam are inland, low rainfall catchments with similar geology and vegetation to 22.

These facts indicate that generally the flows of the Krom into Churchill (22) are reasonable as far as accuracy is concerned, since they have been corroborated with measured flows from hydrologically similar catchments with fairly long records; it remains to examine the record of the Krom River in more detail.

The Apparently Anomalous Flows into Churchill Dam

The record of the Krom River flows into Churchill Dam before 1948 was extended using the monthly Pitman model.  The whole annual sequence 1927-91 is shown in Figure 8(a).  The GENTST analysis shows that the sub-sequences before and after 1983 and the sequence as a whole have no significant serial correlation.  The logarithms of the ranked flows are shown in Figure 8(b) where the most recent ten annual values have been shaded.  It is noteworthy that, in the lowest 14 years of flow, only half belong to the prolonged drought starting in 1983 and ending in 1990.  Evidently, correspondingly low runoff years also occurred in the

Figure 6a :	Double-Mass Plot : Rainfall vs Runoff : 		 Figure 6b :	Scatter Plot : Rainfall vs Runoff :

Sub-catchment 22 (1948 to 1991)							Sub-catchment 22 (1948 to 1991)

Figure 6c :	Scatter Plot : Rainfall vs Runoff :		 	Figure 7a :	Double-Mass Plot : Naturalised Runoff :

Sub-catchment 22 (1927 to 1991)							Kouga Dam vs Churchill Dam (1961 to 1991)

Figure 7b :	Scatter Plot : Naturalised Runoff : Kouga Dam	 Figure 8a :	Time Series of Naturalised Annual Runoff

vs Churchill Dam (1961 to 1991)							into Churchill Dam

Figure 8b :	Ranked Naturalised Annual Runoff into			       Figure 9a :   Comparison of Observed vs Simulated Runoff into

Churchill Dam (log-scale) 								 Churchill Dam (Sub-catchment 22)

(1948 to 1982)

pre-1983 period.  Is the recent drought sequence reasonable?  From the view-point of the streamflow alone, it is a very rare phenomenon.  However, the extended series were used for the analysis that leads to that conclusion and the extension (pre-1948) does not appear to have annual flow totals correspondingly as low as the observed record. Is there a problem with the flow extension?

Evidence to support this view-point might be sought in Figures 9(a) and 9(b).  There, the (patched) recorded flows are compared with those produced by simulation using the best fit Pitman model; first for the full period 1948-82 (9a), and then for the drought period 1983-91 (9b).  It can be seen that, despite an acceptable fit over the rest of the period, there is an over-estimation (by 30%) of the 9-year observed MAR during the drought.  However, this overestimate comes off a low base, and does not seem to be reflected in the relatively good reproduction of deficient flow-duration-frequency characteristics of the pre-1983 sequence- as Figure 9(c) testifies.

Nevertheless, Figure 8(a) does show that, of the 14 lowest annual flows marked in Figure 8(b), only 1 occurs in the 22 years of the extended sequence.  Is this evidence of imbalance?  To explore this, a cusum plot of annual rainfall against runoff for catchment 22 for the extended part of the sequence appears in Figure 9(d).  The manufactured flows also hint at the puzzling dual rainfall-runoff regime dominating Figure 6(b) (which comes from the measured period).  The manufactured flows do seem relatively blameless, so where is the problem?

Casting back to Figure 8(a) shows that a 50:50 split-sample division of the full 65 year series yields 3 in the first half and 11 in the last half.  If this is an imbalance, then the rainfall might hold the key.  Consequently, we look at the sequence of annual catchment rainfalls in Figure 9(e): The division of very low annual rainfalls between earlier and later parts of the 65 year period corresponds well with that of the streamflows.  This picture holds for a variety of rainfall station combinations.  Finally, the loop of our search seems to close - if the historical hydrology holds the key to the strong impact of the 1983-91 flows on the stochastic yield characteristic, then the next stage of detailed investigations should focus on critical sequences in the spatial rainfall. We have to leave this task to the future.

In summary, the measured inflows of Churchill Dam seem to be reasonable even though it contains a rare drought.  The staggered non-linearity of the catchment response has not been explained, but is likely to be a symptom of the physical properties of the catchment, also reflected in the behaviour of the Kouga Dam and Loerie catchments.  These characteristics are particularly the heavily broken, folded geological formations and the rainshadows caused by the orientation of the long narrow valleys parallel to the coast and perpendicular to the rain-bearing weather systems.  The extended half of the historical flow series appears to be on the wet side as far as the low flows are concerned, but not by much - this might be related to the rainfall sequences that happened to be available for the study.

Revisit the Puzzling Yield-Reliability Curves

Having ascertained that the partially reconstructed regional hydrology is not unreasonable, let us now consider the apparent fragility of the streamflow statistics and of their offspring,

Figure 9b :	Comparison of Observed vs Simulated Runoff into	Figure 9c : Flow-Duration-Frequency Curves : Observed vs  Churchill Dam (Sub-catchment 22) (1983 to 1991)		     Simulated Runoff into Churchill Dam

(Sub-catchment 22) (1948 to 1982)

Figure 9d :	Scatter Plot : Sub-catchment 22 : Rainfall vs			Figure 9e :	Time Series of Annual Rainfall in

Runoff (1927 to 1947)								Sub-catchment 22

the stochastic yield-reliability curves a la Figure 3.  As an example we look at two of the Dams with long observed series:

|  | Churchill Inflows |  | Kouga Dam Inflows |  |
| --- | --- | --- | --- | --- |
|  | 1927-83 | 1927-91 | 1927-83 | 1927-91 |
| MAR (Mm3) SD (Mm3) CV | 60,9 37,8 0,62 | 57,3 37,2 0,65 | 197,9 178,6 0,90 | 186,2 171,1 0,92 |

The main impact of the 80's drought is to lower the MAR by 6% in both cases, while its disproportionate weighting of the lower tail of the annual flow distribution in Figure 9(b) ensures that more stochastic sequences contain more extreme low flow years, than what the pre-83 case would allow.  Hence the sizeable drop in stochastic yields.  These sample statistics also provide insight into the relatively low annual variability of flows in this region (CV's in the Vaal catchment tend to be much greater than 1,0).  From the work of McMahon and Mein (1978) it is well-known that low CV's are associated with stable yields, which translate to unstable reliabilities.  This is reflected in the very flat yield-reliability curves of Figure 3.  (The Vaal River displays much steeper curves.)  The result is that any shift in the mean will have a considerable effect on the reliability of any target draft.

The stochastic yield analysis suggests that there is no point in over-refinement of decisions based on the history, without recognising that future flows of an extreme variety, will change the statistical picture.  The changes will arguably not be as dramatic as those due to the recent drought in the Algoa System, but there could be worse droughts and bigger floods in future than those observed historically.  The best strategy is to regularly update one's estimate of system performance by including recently measured streamflows and re-evaluating the reliability based on new demand scenarios.

Conclusion

The hydrology of the Algoa System is varied, complicated and apparently enigmatic.  With the help of the diagnostic tools developed to monitor the performance of the stochastic hydrology in comparison to the historical hydrology, useful insights were gained into the sources of the apparent anomalies.  In fact we were led back to attempting to explain the behaviour of the processes underlying the hydrological responses.  This was done in a top-down, post-mortem mode, the reverse of the usual process of analysis.

The exercise has convinced us that the two branches of applied hydrology - on the one hand that dealing with deterministic modelling of processes and on the other hand that dealing with the statistical properties of the data coming from the processes - both benefit by being worked together in tandem, enriching the understanding one derives each from the other.

Acknowledgements

The material discussed in this paper formed part of the Algoa Water Resources System Analysis, executed by Ninham Shand Inc. for the Department of Water Affairs and Forestry.  We thank the Department for permission to publish this material.

References

Basson, M.S., R.B. Alan, G.G.S. Pegram & J.A. van Rooyen (1994).  Probalistic Management  of Water Resource and Hydropower Systems, Water Resources Publications, Colorado.

McMahon, T.A., and R.G. Mein (1978).  Reservoir Capacity and Yield.  Elsevier, Amsterdam.

NSI. (1983).  Design Report: Elandsjagt Dam. Report No 845A/2973 by Ninham shand Inc to Dept. of Environment Affairs, November 1983.

NSI. (1994).  Hydrology Report.  Algoa Water Resources System Analysis.  DWAF Report No PM 000/00/0893, RSA Dept Water Affairs & Forestry, Pretoria.