Convective Rainfall Characteristics for Incorporation into Conceptual Models of Stochastic Rainfall

Bruce Kelbe

Department of Hydrology

University of Zululand

Kwa Dlangezwa, 3886

INTRODUCTION

Numerical models of hydrological processes have been developed for varying catchment scales.  One of the basic functions of these hydrological models is to assist with predicted impacts of selected catchment management options.  While hydrological models are generally verified and calibrated for each application they are nevertheless crucially dependent on specific information for their application.   Undoubtably the most crucial aspect relates to the rainfall information which drives all of these models.   There are very few instances and locations where there is sufficient long term rainfall data available for the application of these models for management purposes.  One method of overcoming this problem is the development and incorporation of rainfall models for extending the rainfall series.  These rainfall models can take two basic forms which have been classified as (1) Deterministic models incorporating well defined components of the system which are reproducible and probably stationary or (2) Stochastic models which are defined by statistical properties that would represent only one of many possible states.

In principle, deterministic models would provide sufficient information for long term modelling if all the components were known.  Unfortunately, all deterministic models incorporate a significant error or stochastic component because many of the driving forces are unknown.   Consequently, the tendency is to use or incorporate stochastic models for long term modelling requirements and design storm analyses.

Kelbe (1989) developed a multi-site stochastic model of spatial daily rainfall fields for the Crocodile River catchment.  This model is being expanded to include specific rainfall producing systems which are treated as stochastic processes leading to the specification of meteorological states (systems) affecting large (synoptic) scale regions.  Within these regions the rainfall fields are conceptualized as events with randomly varying spatial locations and dimensions.  To incorporate rainfall fields of these events within a stochastic rainfall model, their spatial characteristics need to be specified.  Since these rainfall processes are scale dependent, the spatial dependence must incorporate a description of the scale dependence in these components.

This paper presents statistical and fractal analyses of spatial rainfall fields derived from radar data which are used to describe the convective scale rainfall systems suitable for inclusion in conceptual models of stochastic daily rainfall processes.  The fractal analysis derives the probability functions for simulating the fraction of arbitrary size catchments receiving some rainfall.  The statistical analyses derives functions of the rainfall for simulating the spatial structure of the rainfall.

SCALE ANALYSIS

The description of rainfall is a scale dependent problem. The characteristic spatial and temporal scales of meterological phenomena which affect rainfall are shown in Figure 1. The rainfall in a typical catchment is determined by meteorological processes operating at a scale many orders of magnitude larger than the catchment dimensions.  The global circulation drives the synoptic circulation within which is embedded smaller scale systems operating at the meso and convective scales. Consequently, extended simulation periods require models incorporating those processes operating at several temporal and spatial scales.

The global circulation is driven by the solar cycles which have been described and modelled by Tyson (1985) and Preston-Whyte and Tyson (1988). The synoptic systems have been examined by Preston-Whyte and Tyson (1988) for the east coast of South Africa and the variance in specific characteristics have shown deterministic cycles which could be used to model aspects of the synoptic systems. The meso-scale and convective-scale rainfall characteristics have been examined by Kelbe (1985) for the Free State as indicators of spatial patterns in daily rainfall for inclusion in stochastic rainfall models.

The rainfall distribution in space is derived from meteorological systems (states) which exhibit very different spatial characteristics. Forced convective systems (mainly frontogenesis) produces extensive widespread (general) light rainfall. On the other hand, free convective systems generally produce localized intensive short duration rainfall fields which may occur as isolated elements or they may be composed of several scattered elements.  These isolated or scattered convective elements are sub-components of the larger synoptic system which creates the favourable instability for free convection to occur. Models seeking to incorporate the long term climatic trends as dynamic features which control the larger scale meterological systems that invade an extensive region and initiate convective scale rainfall must incorporate a description of these meteorological states.

The possibility of incorporating deterministic elements of the large scale meteorological systems has provided the incentive to develop a conceptual model of these systems operating at a REGIONAL scale (eg KwaZulul-Natal). This conceptual model would then provide simulated series of meteorological conditions (probability of rainfall in KwaZulul-Natal in a similar manner to the daily forecasts on national Television) which would be used as the driving function for simulating smaller scale rainfall systems at various catchment scales. This paper concentrates on the description of rainfall characteristics at the meso and convective scale for determining the probability of rainfall occurring in segments of varying size catchments. This characterisation of the daily rainfall involved the application of fractal techniques from chaos theory. Once the fraction of a catchment receiving rainfall has been simulated, the spatial probability of concurrent rainfall within the receiving segments and the cross-correlation structure describing rainfall depth can be introduced to simulate the spatial rainfall fields.

DATA SOURCE AND LOCATION

The spatial description of the daily rainfall fields has been done using radar derived rainfall from two location along the KwaZulu-Natal border at Carolina and Bethlehem. The spatial rainfall fields for various classes of convective situation was provided by Seed (1992) who derived them from regular radar volume scans at these two locations. The spatial rainfall fields derived from radar data comprised 4 to 8 minute volume scans of all available storms on those days when convection occurring within a 100 km radius of the radar sites. Based on the Palmer relationship between return echo and ground rainfall, Seed (1992) derived rasterized images of the integrated rainfall fields at a spatial resolution of 0,6km by 0,6km. These fields formed the basis of the fractal and statistical analysis.

Figure 2 and Figure 3 show the spatial distribution of rainfall for a typical rainfall field which would be described as scattered and general (meteorological state) rainfall situations.  A large proportion of the area covered by the radar experiences NO rainfall on these days. Catchments of arbitrary shape and size within this region have varying chances (probabilities) of receiving rainfall and these probabilities depend on the catchment size and its location.  If it is assumed that the rainfall distribution in the REGION is randomly distributed at the scale of the meteorological state (ie no preferred location for scattered or isolated storms) and therefore independent of the topographical surface, each catchment in the region represented by Figure 2 would have the same probability of receiving rainfall.  Any topographical surface dependence (preferential heating etc) would be site specific and a conceptual model for that region would require some additional descriptions of the topographical link.

FRACTAL ANALYSIS

The box counting technique (Pietgen, Jurgens & Saupe, 1992) used to estimate Mandelbrot's fractal dimensions was used to determine the probability of various size sub-catchments receiving some rainfall within the radar coverage area (200*200 km2). The technique is shown diagrammatically in Figure 4, where a square catchment of a specified dimension (pixel count "S" in Figure 4) was viewed as a window which moved through the radar coverage. The number of pixels (N[s]) which were completely covered by radar derived rainfall were counted as a fraction of the catchment area.  This technique provided :-

1) 	the probability of each size of catchment (S2*0.36km2) receiving NO rainfall for each specific meteorological state (class of rainday).

2) 	the probability that any specific fraction of the catchment will receive some rainfall for each specific meteorological state.

The probability field of varying fractions of selected size catchments (from 3km2 up to and including catchments of  81km2) are shown in Figure 5 and Figure 6 for the two selected rain-days shown in Figure 2 & Figure 3.

The probability of NO rainfall (α) in an arbitrary catchment of 3.24km2 (a catchment window of S=3*3pixels) is 61% and 29% for the two meteorological cases shown in Figure 2 and Figure 3.  As the catchment size [S] increases to 81km2, the probabilities of NO rainfall in these size catchments drop to 27% and 9% respectively.  The more isolated the storms the greater the probability of NO rainfall being received in any catchment in the region. The larger the catchment the greater the chance of intercepting some rainfall. Clearly these probabilities are scale dependent and also functions of the meteorological state.

The probability of the entire catchment (ie 100%) being covered by rainfall for an arbitrary catchment size of 3.24km2 is 26% (or 1-0.84 in Figure 5) and 46% (or 1-0.54 in Figure 6) for the same two cases presented above and shown in Figure 5 and Figure 6.  These probabilities decrease to almost 0% and 24% when the catchment increases to 81km2 for these two states (Figure 5 and Figure 6).

Most of the daily rainfall fields analyzed for Carolina showed the probabilities rising fairly linearly from the point where there was no rainfall (the interception point on the Y axis labelled α) to a state where the catchment was completely covered by rainfall (start of vertical line labelled ß).  This is shown diagrammatically in Figure 7.  These two points (no rain and complete coverage) connected by a straight line were used as an initial model to describe the probability of fractions of the catchment area receiving rainfall for the various size catchments and for each meteorological state (class of rain-day). This two parameter model (α,β) showed a range of values that are a function of the catchment size and the meteorological state (general, scattered & isolated).  The distribution of the α and β parameters which have been determined show frequency distributions which suggest a continuous spectrum of values rather than (artificial) discrete classes based on the traditional classification of rain-days. An effort is being made to identify a probability distribution function which will adequately describe this model and the characteristics described below.

The probability of various fractions of the catchments receiving rainfall increases fairly linearly from zero (α) to complete coverage (β).  This increasing function  is defined in Figure 7 as a slope of [β-α]/[%coverage].  This slope generally ranges between 10 an 20% for the smallest size catchment but increases considerably for the larger size catchments (Figure 8).  The difference  between αL for the largest size catchment (225km2) and αS for the smallest catchments (3.25km2) for the different meteorological states is fairly regular (Figure 9) so the differences between catchment size in the slopes shown in Figure 8 are functions of β.

In Figure 8, the storm events have been sorted in ascending order of α the Probability[no rainfall].  The first few events on the left of Figure 8 represent GENERAL type rainfall events when extensive rainfall occurred from frontal type systems.  The slopes on the right hand side are representative of isolated convective systems. There is a difference in the slope which can be attributed to the effect of meteorological condition,  particularly for the larger size catchments in Figure 8 where the slope drops from approximately 70% to less than 50% for the smaller systems.

METEOROLOGICAL EFFECTS

The Prob[No rainfall] in the catchment (α) is presented in Figure 10 for all events ranked  on αS (the smallest size catchment).  The lowest probabilities are associated with GENERAL type rainfall situations (left hand side of Figure 10) and the higher probabilities with isolated situations.  The ranked values show a smooth (nearly) straight line trend which implies a continuous probability function.  The Prob[100% rainfall coverage] in the catchment (β) also shows a consistent trend from high values under GENERAL meteorological conditions (Figure 11) to low values for the more isolated systems.  The β values decrease to near zero for many of the scattered and isolated meteorological conditions.  This would normally be expected as the storm size approaches the catchment size and more than one storm would be required to cover the catchment.

The fractal analysis provided an indication of the expected fraction of the area of various size catchments which are likely to receive rainfall when the indicated meteorological state exists.  However, it is still necessary to simulate how this fractional area is actually distributed throughout the catchment. In order to achieve this it is necessary to define the spatial distribution of probabilities and cross correlations at the relevant catchment scale.

SPATIAL PROBABILITY

Convective rainfall has a characteristic spatial structure which provides a basis for modelling the catchment rainfall distribution (Kelbe, 1987).  Prob[rx|r0], the conditional probability of concurrent  rainfall (rx) at all the locations (xi) surrounding (or relative to) each point (pixel) which has received some rainfall (r0 at x0 : the central origin) is shown in Figure 12 and Figure 13 for the meteorological states displayed in Figure 2 and Figure 3.  Prob[rx|r0] decreases with increasing radius (τ) from 1.0 at the origin (located at [30,30] in Figure 12 and Figure 13) at a rate which is dependent on the spatial extent of the rainfall (ie meteorological state).  Kelbe (1987) has described this dependence by a radial (τ) and azimuthal (ω) function for rainfall in the Free State region of South Africa.  For those regions where there is preferential alignment of storms or preferred storm movement, these functions may become elliptical and require an additional parameter for their description (Kelbe, 1987).

The cellular nature of the rainfall is clearly evident in Figure 12 and Figure 13 for both rainfall conditions displayed in Figure 2 and Figure 3.

Once the centroid of the rainfall field has been simulated in the stochastic model (which may fall either within or outside the catchment boundaries), Prob[rx|r0] can be used to synthesize rainfall occurrence over  the catchment until the simulated fraction of the catchment is occupied by rainfall states.

SPATIAL CORRELATIONS

The rainfall depth within any convective system shows strong spatial distribution. The rainfall usually consists of multiple cells which  have their highest rainfall at or near the centre of the storm (see Figure 2 and Figure 3).  This rainfall decreases rapidly away from the storm centre.  Analyses of the cross-correlation structure for one of the two examples is presented in Figure 14.  The cross-correlation fields show similar cellular structure to Prob[rx|r0] which have been described as spherical function of radius (τ) and azimuth (ω) by Kelbe (1987).

The cross-correlations in Figure 14 show some additional cellular structure surrounding the centre cell.  Because storm cells have characteristic dimensions which typically attain diameters of 10-20 km (see Figure 14), those catchments of larger dimensions are likely to be influenced by more than one cell on a scattered rainfall day like Figure 2.

Once the depth of the rainfall at the centroid has been synthesized, the correlation structure can be used to synthesize the rainfall amount at each pixel which has been classified as a wet state by the probabil wet state by th"elbe 1987) from a raingauge network of daily rainfall over a large area of the Free State.  Additional spatial analyses using fractal techniques have provided spatial probability distributions for stochastic simulation of the fraction of varying size catchments with rainfall inundation.  These distributions have been derived for differing meteorological conditions imbedded in larger scale synoptic systems.  The statistical analyses of the radar derived rainfall fields have provided spatial fields of conditional probability of concurrent rainfall at sites (area) relative to the occurrence of rainfall at an origin.  The statistical analyses also provided cross-correlation fields in the rainfall depth.  These probability distributions and correlation fields of the convective scale rainfall are being derived as parametric models for inclusion into a conceptual model of stochastic daily rainfall.

ACKNOWLEDGMENTS

I am grateful to N. Snyman and V. Smakhtin for providing valuable assistance in creating the programmes for the statistical analyses. The support of the Water Research Commission under project K5/346 is also gratefully appreciated.

REFERENCES

Kelbe, B.E., 1987; Spatial constraints in extrapolating daily rainfall in the semi-arid interior of Southern Africa. J. Agric & Forest Meteor. 40 51-59.

Kelbe, B E, 1989; A multisite stochastic daily rainfall model. Paper presented at the Fourth South African  National Hydrological Symposium, Pretoria 55-62..

Pietgen H O, H Jurgens & D Saupe, 1992, Fractals for the Classroom, Springer-Verlag, Berlin.

Preston-Whyte R A and P D Tyson, 1988; The atmosphere and weather of southern Africa. Oxford Press.

Seed AW, 1992; Generation of a spatially distributed daily rainfall datbase for various weather modification scenario. Water Research Commission report 373/1/92, Pretoria.

Tyson P D, 1986; Climatic change and variability in Southern Africa, Oxford University Press, Cape Town.