Impact of the Lesotho Highlands Water Project

on the Orange River

1R.S. McKenzie & 2F A Stoffberg

1BKS Inc., P O Box 3173, Pretoria, 0001.

2Department of Water Affairs & Forestry, P Bag X313, Pretoria, 0001.

Introduction

The Orange River basin (see Figure 1) is the largest river basin south of the Zambezi with a total catchment area of approximately 1 million km2.  More than half of the catchment is inside South Africa  with the remainder in Lesotho, Namibia and Botswana.  The portion of the catchment inside Botswana contributes via the Molopo and Nossob rivers to large pan areas with the result that effectively no water from Botswana ever reaches the Orange River.

Figure 1:	Location plan of the Orange River basin

The Vaal River is the most important tributary of the Orange River and its catchment is highly developed. It is the main water source for the Gauteng province which is generally regarded as the powerhouse of Africa.  It is the main metropolitan and industrial centre in South Africa supporting more than 40% of the country's population and producing over 50% of the country's wealth.  The Vaal River also supplies water to various irrigation schemes, several major gold mines, the country's two largest petro-chemical plants, various large industries such as Iscor and to a number of large thermal power stations in the Eastern Transvaal Province which produce 85% of the country's electricity.

As a result of the developments in the Vaal basin, the water demands already exceed the natural resources of the catchment (approximately 2200 million m3/a) as can be seen in Figure 2.  Four major interbasin transfer schemes are already in operation and a fifth scheme, the Lesotho Highlands Water Project (LHWP), is under construction.  The various water transfers are shown on Figure 3.

Figure 2:	Growing water demands in the Vaal River basin

(from DWAF, 1994)

Figure 3:	Water transfers into and out off the Vaal River basin

Due to the magnitude of the original planned transfer of water from Lesotho to the Vaal basin, some 2 200 million m3/a, the DWAF commissioned a study to assess the influence of such a transfer on the reliability of supply to the other users of Orange River water.  The study was completed in 1992 and the preliminary yield results are given in the project report (DWAF, 1992).  Several issues were addressed in the study and some of the key findings are discussed in the remainder of this paper.

INFLUENCE OF LHWP ON THE YIELD OF THE ORANGE RIVER PROJECT

Gariep and Vanderkloof dams form the two major storage elements of the Orange River Project (ORP) They are currently the two largest reservoirs in South Africa with a combined capacity in excess of 8 500 million m3.  These reservoirs support various water demands (mainly for irrigation) along the Orange River as well as demands in the Eastern Cape Province and the Riet River basin (see Figure 1).  If water is transferred from Lesotho to the Vaal River basin, it will obviously have an influence on the remaining water available to the ORP.

The LHWP as proposed in 1986 (LMC/OSC) was intended to be developed in up to five phases with an ultimate transfer to the Vaal River basin of 70 m3/s (i.e. 2200 million m3/a).  When this planning was undertaken, the question of the impact of the scheme on the remaining resources of the Orange River was not considered to be a major issue.  The resources of the Orange River at that time were thought to be sufficient to support the full LHWP scheme and leave some excess available for further developments within the ORP.  It was understood, however, that the water resources of the Orange River would be completely re-evaluated before implementation of the LHWP was allowed to proceed beyond Phase 1.

In 1988 the influence on the yield available from the ORP on the various phases of the LHWP was examined as part of the Orange River System Analysis.  This investigation was commissioned by DWAF and undertaken by consultants BKS Inc. in association with SSI.  The yield analyses were undertaken using the system modelling techniques developed by DWAF for use in the previous Vaal River System Analysis which was the first detailed system analysis to be undertaken in South Africa.  The results from the assessment are shown in Figure 4 (DWAF,1993) and as can be seen, the total system yield increases from 4456 million m3/a to 5102 million m3/a as new storage elements are added in the different phases of the LHWP.  As would be expected the yield available from the ORP decreases (i.e. 4456 million m3/a to 2755 million m3/a) due to the reduction of flow in the Orange River system caused by the Lesotho transfers.

Figure 4:	Influence of LHWP on ORP yields (from DWAF, 1993)

The significance of these preliminary results is not immediately apparent until the demands of the ORP on the Gariep and Vanderkloof dams are also taken into account.  Figure 5 indicates the balance between supply and demand which clearly indicates that supply problems are likely to be experienced after Phase 2 of the LHWP has been completed.  The values given in Figures 4 and 5 were preliminary estimates based on certain assumptions used in previous water balance assessments.  They were considered of sufficient accuracy, however, to identify a major problem with the implementation of the LHWP which has yet to be resolved.  The results, however, clearly indicated that there will be no problem supporting the various ORP demands after Phase 1 of the LHWP has been commissioned.  Phase 1 is currently under construction and will be completed by the year 2003.

Figure 5:	Preliminary indication of available ORP yield versus demands for various phases of the LHWP

INFLUENCE OF THE LHWP ON THE FLOW REGIME IN THE ORANGE RIVER

An integrated system model was set up to analyse the water resources of the Orange River as well as the transfers from Lesotho into the Vaal River basin.  A large portion of the system was based on work from previous and current Vaal River System Analyses (Van Rooyen, 1995) which was required in order to build up a complete system model for the Orange River basin.

The inflows from the Vaal River are of importance to the Orange River from both quantity and quality viewpoints.  It was initially expected that the return flows from the Vaal River would increase significantly due to the transfers from Lesotho and these must therefore be included in any future assessment of the resources of the Orange River.  Recent simulations indicated that the return flows from the Vaal River may not increase and could even decrease as more water is transferred from the LHWP.  The quantity of the return flows from the Vaal River into the Orange River is very sensitive to the water quality constraints imposed on the abstractions from the Vaal Dam and the Vaal Barrage.  This issue is extremely important and is currently under further investigation.

A conceptual schematic of the system model used in the recent Orange River system analyses is given in Figure 6 which indicates the complexity of the model prepared for the analysis.  Each block in Figure 6 is a complete sub-system and contains numerous reservoirs, river channels and hydrological inputs.  The total integrated system includes in excess of 50 reservoirs, more than 600 river channels, 120 hydrological inputs and numerous interbasin transfers as well as river loss channels along the Lower Orange River.  Numerous irrigation return flow channels were also required.

Figure 6:	Conceptual schematic diagram for the integrated Orange River System (from DWAF, 1995).

Using the system modelling techniques to simulate the flows in the Orange River basin including the contributions from the Vaal River basin and the Fish River basin (Namibia), the likely flows at various sections of the Orange River were examined for different development scenarios.  The model was also used to estimate the natural flows that would occur if no developments had taken place.  Such natural flows are of interest to environmentalists and of great help in any environmental impact assessment or the determonation of instream flow requirements.

Figure 7 indicates the likely flows for a river reach inside Lesotho just upstream of the Senqu/Senqunyane confluence.  The analysis is based on the original transfers and compensation flows provided in the LMC reports and shows the dramatic reduction in flow that could be expected from the upstream developments if the LHWP were to be fully developed through to Phase 5.  As can be seen, the flows in the river are seriously affected by the upstream developments and are quite unacceptable from an environmental viewpoint, especially beyond Phase 2.

Obviously the upstream developments will not be permitted to change the flow regime in the Senqu or Orange River to the extent indicated in Figure 7 for the full development of the LHWP.  The figure does, however, demonstrate one of the many uses of the system analyses techniques by highlighting the impact of possible catchment developments on river flows.

Figure 8 provides similar information on typical natural streamflow patterns along the Orange River.  These are preliminary figures based on estimates of river losses along the Lower Orange River which are currently the subject of a separate study for the water Research Commission.  The estimated natural flows may be revised as new information becomes available.  Of particular interest are the natural flows at the Orange River Mouth which is a registered RAMSAR site and has been the subject of several environmental studies.

Figure 8 suggests that in the past the flow at the river mouth ceased frequently under natural conditions, something that has not occurred since Gariep and Vanderkloof dams were commissioned.  Unfortunately it is difficult to determine the frequency of zero flows at the Orange River mouth from the system analyses used to produce Figure 8.  The model used is a monthly time step model and does not take the effects of streamflow lag into account.  This is not normally a problem with most of the system analyses undertaken in South Africa where the monthly time step is acceptable.  In the case of the Orange River, however, it can take as much as two months for water to travel from the Vanderkloof Dam to the river mouth.  If the model indicates a zero flow at the river mouth in a particular month, it is likely that some water will continue to flow, supported by the considerable storage in the river itself.  If the model indicates zero flow for two or more consecutive months, it seems likely that the flow would have ceased at the river mouth.  The zero flows indicated in Figure 8 are being cross-checked where possible against historical observations.

Figure 7:	Influence of the LHWP on the flow in a selected river reach in Lesotho (from DWAF, 1995)

Figure 8:	Simulated natural flows at various sections of the Orange River

(from DWAF, 1995)

Influence of LHWP on Hydro-Power Generation

The two largest conventional hydro-electric power plants in South Africa are situated at the Gariep and Vanderkloof dams on the Orange River.  Due to the magnitude of possible transfers via the LHWP it was necessary to examine the influence of the transfers on the ability to generate power at the dams.  The first analyses of the hydro power were presented in 1993 (DWAF, 1993) in which a relatively simple operating rule was selected.  The analysis clearly indicated that there were likely to be long periods during which no power could be generated and that problems with power generation could materialise even before the LHWP was in operation.

Shortly after the report, power generation at both dams was stopped for the first time since the dams were commissioned.  Subsequent studies on the power production potential have been undertaken (Van Rooyen, 1995) for DWAF and Eskom.  The aim is to ensure that the available water resources are used to the maximum benefit of the country by optimising the trade-off between power generation and water supply.  Eskom is currently supporting further hydro-power analyses in order to derive new operating rules applicable to existing development and demand levels as well as for the likely future situation.

Conclusions

The results from the various analyses clearly demonstrate the importance of undertaking a detailed system analysis.  By using a holistic system approach, it was possible to investigate the influence of the LHWP on a number of important aspects such as yield, river ecology and hydro-power generation.  The initial analyses clearly indicated that further detailed analyses of certain key issues are required to ensure that the available resources are utilised to the maximum benefit without having a detrimental effect on the environment.

To that end, separate detailed investigations have already been initiated to estimate river losses with more confidence and to determine the most beneficial operating policies with regard to hydro-power generation and environmental conservation.  Further inter basin transfer schemes to supplement the Orange River or to reduce the demand on the LHWP are also under investigation.

Acknowledgement

The authors wish to thank the Department of Water Affairs and Forestry for its support of the studies discussed in this paper and for permission to present the paper.

References

DWAF (1993).  Yield analysis up to PK le Roux Dam.  Report number P D000/00/1292 	by BKS Inc. (in association with SSI) to the S A Department of Water Affairs and Forestry, Private Bag X313, Pretoria, 0001. (author Mckenzie, R.S.).

DWAF (1994).  Future water demands and return flows : Vaal River supply area.

Report number P C000/00/14492 by BKS Inc. to the S A Department of Water Affairs and Forestry, Private Bag X313, Pretoria, 0001. (authors Potgieter, W.F. & Craig, A.C.).

DWAF (1995).  Hydrology and system analysis : Orange River Basin.  Report number 	P D000/00/4695 by BKS Inc. (in association with NSI) to the S A Department of Water Affairs and Forestry, Private Bag X313, Pretoria, 0001. (authors Mckenzie, R.S. & Mare, H.G.)

Mckenzie, R.S. & Stoffberg, F.A. (1993).  Orange River - System Analysis. Proceedings 	of the ASCE Hydraulics Division Symposium, San Francisco, USA, pp 1145 - 1150, July 1993

OSC/LMC (1986). Lesotho Highlands Water Project.  Supporting reports to the Kingdom 	of Lesotho, Ministry of Water, Energy and Mining by the Lahmeyer Macdonald Consortium and the Olivier Shand Consortium, April 1986.

Van Rooyen, P.G. (1995).  Personal Communication, July 1995.  BKS Inc.,  PO Box 3173, 	Pretoria 0001.