The use of a Multiple Linear Regression Model in Upland River Valleys

in Western Zambia

L van den Brand & A Huizing

1Land and Water Management Project, Department of Agriculture, P O Box 910399, Mongu, Zambia.

2Land and Water Management Project, Ministry of Agriculture, P O Box 31905, Lusaka, Zambia.

Introduction

Wetlands play an important role in the agricultural development of Western Province.  The study of their hydrology is necessary in order to understand their agricultural potential.  Due to the size of the Province and the scarcity of hydrometeorological data the use of hydrological models is indispensable.

This paper describes a multiple linear regression model for the spreadsheet program LOTUS 123, which relates rainfall to runoff.  It will show that the model can be successfully applied to the small and medium sized rivers in Western Province in order to analyze their hydrology and their agricultural potential.

The paper starts with a description of Western Province and the different types of wetlands found in the Province.  The theoretical background will be illustrated by the case study of the Luena river.  In the last part the applications of the model will be explored, using the example of the Lui River.

Wetlands in Western Province, Zambia

Zambia's Western Province is situated between latitudes 14° and 17° south and longitude 22° and 25° east.  It is Zambia's border province with Angola and Namibia.  With a surface area of 126,400 km², it covers approximately 17% of the total land surface of Zambia (Chiyala et. al, 1995).  The Province has a population of 607,500 people and a population density of 4.8 persons/km2 (Heemskerk, 1995).  With less than 1000 km of paved roads, major parts of the Province are barely accessible.

A similar lack of infrastuctural development can be found in the collection of hydrometeorological data.  Only four stations exist with long records of rainfall and other climatological data.  Water level and discharge measurements exist for a small number of rivers, in particular for the Zambezi.  Hence there exists the need to develop methods, such as the model under review in this paper, which can make an optimum use of the available data.

Except for the eastern part of the Province, the Province is covered by the Kalahari Sands, with a depth ranging from 50 to 200 m (IUCN, 1992).  The notable physical features of the Province are the Zambezi river, which runs from north to south, and its extensive flood plain; and the undulating uplands, which rise 30 to 70 m above the plain and which dominate the landscape on both sides of the Zambezi river.  Scattered over the uplands deflation pans, or pan dambos, are found.  The uplands are dissected by river valleys, most of them tributaries of the Zambezi river.  Flood plains similar to the Zambezi Flood Plain, although smaller, are associated with the lower reaches of these river valleys.  The flood plains, with the exception of isolated higher spots, are flooded from January to May - June and dry from July to December.

Thus three wetland types can be distinguished in Western Province: the Zambezi and its flood plain, the pan dambos and the inland river valleys.  Wetlands are usually described as lands that have water on or near the surface and experience a temporary or seasonal occurrence of this water.  These two elements can be found also in the different wetland types of Western Province and it is the presence of water near the surface and the seasonal inundations that make them the most important areas in the agricultural system in Western Province.  They are used for agriculture, cattle grazing and fishing.  The crops that are grown in the wetlands and on the wetland fringes are: maize, rice, cassava, millet and vegetables, such as sweet potato, pumpkin and onion (Chiyala et. al, 1995).  This paper deals with two examples of the third wetland type: the Lui and Luena river.

On the Kalahari Sands soils consist predominantly of medium to course sands, with a high permeability and a low moisture holding capacity.  In the wetlands soil development is strongly influenced by the hydrological circumstances, in particular on the edge of the wetland with the upland, where year-round seepage has led to the development of peat soils.  In the eastern part of the Province, around Kaoma, soils have developed on the quartsites and basalts of the Karroo System (Offergelt, 1980).  Here soils consist of reddish deeply weathered loamy sands and sandy clay loams, with a moderate permeability and moisture holding capacity.

Four climatic seasons can be distinguished.  From October to December the Province experiences dry and hot weather, with occasional rainfall.  The main rainfall period runs between December and April.  The period from April to June is dry, characterized by the building up of high pressure and cool nights.  The fourth season is from July to September, when high pressure, cold nights and dry weather are dominating and night frosts may occur (van Gils, 1988).

The rainy season usually starts in November and ends in March or April, with a peak between December and February.  More than 90% of all the rain falls in this period.  Annual rainfall decreases from north to south.  The northern stations Mongu and Kaoma receive an average of 940 mm, Senanga receives 770 mm, while Sesheke receives only 680 mm.  In all areas the rainfall varies strongly over the years.  These variations range, on a 1 out of 5 year probability, from 250 mm in the north to 280 mm in the south.

The Rainfall - Runoff Model "RAINRU"

This model is based on two publications.  Its theoretical concept has been elaborated in "Principles of Hydrology" (Savenije, 1994), while a version for the Symphony spreadsheet program has been developed by Ketelaars (1991).  Its use and adaption to the LOTUS  spreadsheet program has been recommended during a hydrological backstopping mission (Savenije & Mutale, 1994) in October 1994.

The theoretical concept will be illustrated with the example of the Kasambamezi station on the Luena river.  This station has been established by the Department of Water Affairs in 1963.  The data for this station and its stage - discharge relationship have recently been reviewed and will be used in this study (SADDC, 1993).  The records for this station contain large periods with incomplete data.  The longest period for which the daily records are complete is the period from 1972 to 1980.  The daily water level observations during this period and the stage - discharge relation have been used to calculate the monthly runoff in mm for this station.  The rainfall used in the model is the average rainfall for the stations Kaoma, Mongu and Senanga.

A large number of models and empirical formulas have been developed in order to establish a relationship between rainfall and runoff.  The amount of rainfall which will leave the catchment as runoff depends on a large number of factors which are specific for the catchment, such as the shape, slope, soils, drainage pattern and vegetation; and on climatic factors such as the rainfall pattern, rainfall intensity, seasonality and evapotranspiration.  To compare rainfall and runoff, with all terms expressed in volume per time (e.g. mm/month) the water balance of a catchment can be expressed as:

R = P - L - ΔS /Δt		(1)

where R is runoff, P is precipitation, L are losses and ΔS /Δt is the change in storage over time t.  The presence of the storage and losses component in the water balance makes the establishment of a straightforward relationship between rainfall and runoff difficult.  The importance of the storage and losses is shown in figure 1, which shows the rainfall and runoff for the Kasambamezi station for the period 1972 - 1980.

From figure 1 it is clear that only 2 - 5 % of the rainfall results in runoff, the remainder being losses through interception, evaporation and transpiration and changes in storage through recharge of the groundwater and seepage.  Given the inaccessibility of the area and the limited amount of hydrological data storage and losses are difficult to quantify, in particular for periods of a month or a dekade.

Figure 2 shows a plot of the monthly rainfall against the monthly runoff for the Luena, with the numbers indicating the month, starting with January.

Figure 2 shows that runoff is low in the dry season and in the first months of the wet season, but increases strongly later in the wet season, in the period January - April.  This difference is caused by the change in storage.  During the dry season the storage has been depleted, which will be recharged during the first part of the rainfall season.  During the last part of the rainfall season storage recharge will be small, while seepage will increase.

Furthermore not all rainfall will be effective rainfall.  There exists a threshold below which rainfall will cause little or no runoff.  This threshold accounts for losses such as interception, storage recharge, evaporation, transpiration and surface retention (Savenije, 1994).  By introducing this threshold loss and the changes in groundwater storage a relationship between rainfall and runoff can be established.  The threshold can be introduced as a threshold value for rainfall, with the assumption that an amount of rainfall below the threshold value will not result in runoff.

The changes in groundwater storage are depended on the rainfall in the previous periods. Therefor a relation must be used which relates the runoff in a given period to the effective rainfall in that period and in the preceding periods.  A part of the runoff will be the base flow, which is introduced as a constant Rb.  Thus for the relation between runoff at time t and rainfall at time t to t=i a linear backward relation is used, incorporating the rainfall threshold (Savenije, 1994):

Rt - Rb = a + b0 (Pt - T) + b1 (Pt - 1 - T) + b2 (Pt - 2 - T) ..... + bn (Pt - n - T)	(2)

The coefficient bi is the coefficient that represents the contribution of the effective rainfall for that particular period to the runoff of the period under consideration.  The constant a becomes zero, when all losses and rainfall would be taken into account.

In a spreadsheet program equation (2) can be used to determine the coefficients a and bi by multiple regression.  By placing Rt - Rb and (Pt - T) .... (Pt - i - T) in the columns of the spreadsheet with m observations of Rt and P in effect equation (2) has been transformed into the matrix equation:

Rt - Rb = a + B (P - T)	(3)

where Rt - Rb is a 1 by m vector, a is a constant, B is a i by 1 matrix and (P - T) is a i by m matrix, containing on each row the effective rainfall of the present and previous months.  From equation (3) it is clear that with a equals zero the sum of bi becomes the runoff coefficient C, with C being:

C = R/(P - T)	(4)

The input for the model is the amount of rainfall and runoff per month or dekade in mm and the two variables: the rainfall threshold T and the base flow Rb.  The model will then calculate the values of bi for the timesteps t to t = t-i by multiple regression.  The output of the model are the correlation coefficients and the sum of bi for each set of timesteps and a graph comparing the measured runoff with the computed runoff.  These outputs will enable the optimization of the model for each data set.  The most important criterium is the correlation coefficient between the measured runoff and the effective rainfall.  This correlation coefficient should be at least 0.70 (Ketelaars, 1991).  Schematically the model can be presented as follows:

↓

↓

→

↓

↓

↓

↑							↓

←

↓

To illustrate the theory and the use of the model the model will be applied to the Luena river.  The Luena river is a tributary of the Zambezi and has a catchment area of 23,775 km2.  The river flows in from east to west, starting 25 km east of Kaoma.  It can be divided in three distinct parts.  For the first 130 km the river flows through the region where soils consist predominantly of the red, deeply weathered soils originating on the quartsites of the Karoo System.  In the first 70 km the river is small, with many little tributaries and a small floodplain of 0.5 to 1.5 km wide.  It has a slope of 0.3 m/km.  After 70 km the flood plain disappears and the river is confined to the riverbed only.  Here the Luena is joined by its main tributary, the Luampa, which originates 80 km south of Kaoma in the Kalahari Sands.  The slope of this second part is with 0.9 m/km considerably higher.  The third part starts after 140 km where the river reaches the Kalahari Sands and broadens to a flood plain with a width of 3 km.  Here the slope decreases to 0.2 m/km.  After 50 km this flood plain gradually widens until it ends in the Luena Flats.  The Luena Flats are associated with the Zambezi Flood Plain.  During the wet season the Luena discharges in the Zambezi river, but in the dry season the discharge of the Luena is too small and the river disappears in the swamps and marshes of the Luena Flats.

Kasambamezi station is located along the second part of the river, some kilometers downstream of the confluence of the Luena and Luampa river.  It has a catchment area of 15,444 km2.  The highest discharges occur in March and range from 20 to 180 m3/s.  The discharge at the end of the dry season is 3 m3/s, which equals approximately 0.5 mm/month.  The rainfall data of Mongu, Kaoma and Senanga have been used, but it should be noted that Senanga does not form a part of the Luena catchment.  It is included to represent the catchment of the Luampa river, which extends southward in the area with lower rainfall, where unfortunately no other station is available.

The input data for the Luena river at Kasambamezi are given in table 1 and 2.

Table 1	Input data runoff (mm)

1972	1973	1974	1975	1976	1977	1978	1979	1980

Oct		0.61	0.53	0.26	0.48	0.66	1.07	0.59	1.03	0.80

Nov		0.75	0.42	0.55	0.73	0.57	0.98	0.52	1.34	1.45

Dec		0.91	0.68	1.27	2.87	1.15	1.43	1.68	4.82	4.41

Jan		1.66	0.74	2.96	6.23	2.39	1.55	3.75	3.70	3.53

Feb		1.73	0.90	7.11	6.30	3.29	2.68	8.01	2.29	3.70

Mar		2.64	1.36	11.24	11.37	11.27	7.51	19.62	2.56	4.80

Apr		2.87	0.68	2.67	6.68	11.31	2.98	11.10	2.27	3.35

May		1.63	0.47	1.98	2.39	4.63	1.79	4.65	1.38	1.90

Jun		0.96	0.35	1.27	1.58	2.25	1.23	2.29	1.03	1.40

Jul		0.85	0.35	1.15	1.41	1.83	1.12	2.02	1.02	1.29

Aug		0.71	0.36	0.93	1.19	1.42	0.96	1.67	0.98	1.14

Sep		0.54	0.27	0.66	0.93	1.08	0.76	1.23	0.74	0.85

Total		15.9	7.1	32.0	42.2	41.8	24.1	57.1	23.2	28.6

Table 2	Input data rainfall (mm)

1972	1973	1974	1975	1976	1977	1978	1979	1980

Oct		8.9	17.7	36.5	21.8	12.1	30.2	6.7	63.0	41.9

Nov		63.7	14.3	80.5	137.3	22.6	71.9	60.7	134.3	145.2

Dec		151.6	164.3	239.2	263.9	208.4	168.2	202.3	180.1	232.8

Jan		233.5	87.7	224.9	208.8	207.2	136.7	208.0	138.1	126.0

Feb		69.1	113.3	292.6	140.0	217.5	220.7	258.9	70.4	165.8

Mar		211.5	64.8	101.6	153.9	241.3	219.6	252.4	127.9	117.6

Apr		57.7	1.5	24.4	17.1	71.1	25.7	80.6	0.4	65.9

May		0.9	1.5	19.4	1.7	20.6	1.2	5.8	0.0	0.0

Jun		0.0	0.0	0.0	0.0	0.0	0.0	0.0	0.0	0.0

Jul		0.0	0.0	0.0	0.0	0.0	0.0	0.0	0.0	0.0

Aug		0.0	0.0	0.0	0.0	0.0	5.3	0.0	3.5	0.0

Sep		0.3	7.0	0.0	5.5	1.3	4.7	8.2	0.9	0.0

Total		797.1	472.1	1019.0	950.0	1002.2	884.2	1083.6	718.6	895.3

The results of the multiple regression are shown in table 3, for a threshold T of 105 mm and a base flow of 0.3 mm.  The average runoff coefficient C for the nine years of observation is 0.097 when calculated with the measured runoff and 0.080, when calculated with the computed runoff.

Table 3	Summary of the correlation

i	t	t-1	t-2	t-3	t-4	t-5	t-6	t-7	t-8	t-9	t-10

r2	0.196	0.550	0.655	0.734	0.734	0.735	0.741	0.751	0.755	0.758	0.758

Se	2.75	2.07	1.82	1.60	1.61	1.62	1.61	1.58	1.58	1.58	1.59

sum(bi)	0.028	0.051	0.064	0.077	0.079	0.080	0.086	0.095	0.102	0.108	0.108

The best fit is obtained by using the timesteps t to t - 7, which gives a correlation coefficient of 0.751 and a sum of bi of 0.095.  With the values for constant a and coefficients bi for timesteps t to t - 7 equation (2) becomes:

Rt - Rb =	- 0.319 + 0.016 × (Pt - T) + 0.03 × (Pt - 1 - T) + 0.015 × (Pt - 2 - T) + 0.023 × (Pt - 3 - T) + 0.002 × (Pt - 4 - T) - 0.002 × (Pt - 5 - T)

+ 0.003 × (Pt - 6 - T) + 0.008 × (Pt - 7 - T)	(5)

From the correlation coefficients in table 3 and from the coefficients bi in equation (5) it is clear that the major improvement in the correlation is the result of inclusion of the timesteps t - 1 to t - 3 in the calculation of the runoff. The effect of the inclusion of the timesteps t - 4 to t - 7 results only in a slightly better fit of the recession curve.

Figure 3 gives the graph of the measured and computed runoff versus the time, using equation (5).

The Applications of the Model: The case of Lui River

The applications of the model will be discussed, using the example of the Lui river.  Lui river has a catchment area of 11,190 km2.  The river starts 40 km east of Mongu and flows in southwest direction, where it enters the Zambezi at 10 km below Senanga.  The total length of the river is 130 km and along 100 km the river has formed a flood plain.  The width of the flood plain varies from 2 km in the north to 4 km in the south.  Its average slope is 0.4 m/km. Soils in the catchment are coarse to medium sands.  In the flood plain soils are sandy with a shallow top layer of organic matter.  In several places the inflow of seepage water has caused the formation of peat soils.  Local rainfall and seepage from the uplands cause extensive flooding from January to June.

Since 1988 a gauging station exists at Litawa, halfway along the river.  At this station water levels and discharges have been measured and a stage - discharge relation has been developed (Bastiaansen, 1995).  Figure 4 shows the stage - discharge relationship for Litawa.

The Litawa station has a catchment of 4,590 km2. The highest discharges occur in March and vary from 25 to 60 m3/s. At the end of the dry season the flow is reduced to 0.15 m3/s, or 0.03 mm/decade.

Many farmers around Lui river practice the cultivation of rice under natural flooding conditions, without any form of control of the water level.  In order to evaluate the feasibility of this form of agriculture the variation of the annual maximum water level has to be analyzed.  To do so the model can be applied in two ways: by simulating historical water levels, using historical rainfall data; and by predicting water levels for the coming season, based on a seasonal rainfall forecast.

To fit the model to Lui river the dekadal runoff was calculated based on the water levels measured from 1989 to 1993 and the average of the dekadal rainfall between the staions of Mongu, Kaoma and Senanga, which surround the Lui catchment.  The best fit, with a regression coefficient of 0.86, was found for a threshold T of 25 mm per dekade and a base flow of 0 for timesteps t - 1 to t - 9.  Figure 5 shows the measured and computed runoff for the period of 1989 to 1993.

Using the stage - discharge relationship the runoff can be converted to water levels. Figure 6 shows the measured and computed water level. Due to the shape of the stage - discharge curve the differences between computed and measured runoff are greatly reduced after the conversion to water levels.

Based on a seasonal rainfall forecast it is possible to predict the water level.  This is shown in figure 7, where the rainfall for the 1993/94 season is used as input in the model, using the T, Rb and timesteps t to t - 9 from the model calculated above.  The predicted water level fits the observed water level very closely.

It should be noted that the seasonal forecasts which are currently issued by the Department of Meteorology do not yet have the necessary precision to make this application of the model a reliable tool.

For the simulation of historical runoff the dekadal rainfall for Mongu, Kaoma and Senanga

for the period 1953 to 1995 has been used as input in the model.  Using the stage - discharge relationship the simulated runoff can be transformed into simulated water levels.  After ranking the simulated water levels are found to be normally distributed.  Figure 8 shows the simulated water levels plotted versus the probability of non-exceedance.  This figure shows that for a probability of non-exceedance of 10 % the maximum water level equals 1028.93; and for a probability of non-exceedance of 90 % the maximum water level is 1029.22.  This means that for an 80 % reliability interval, i.e. between the percentages of non-exceedance of 10 % and 90 %, the variation in the annual maximum water level is confined to 0.29 cm.  With this relatively small variation it can be concluded that field locations can be found which have an adequate flooding depth of 25 cm in dry years, while in wet years the flooding depth will not exceed 55 cm.  This variation is small enough for the local medium straw rice varieties.  It shows that, as far as the hydrology is concerned, the flood plain of the Lui river is suitable for the cultivation of rice under natural flooding conditions.

Conclusion

From the two examples it can be concluded that the "RAINRU" model can be successfully applied to the small to medium sized rivers in Western Province.  It enables the researcher to analyze the flood regime, despite the lack of hydrometeorological data and as such it is a valuable tool in areas where such data are scarce.  By applying the model to the data of the Lui river it can be shown that the flood plain of this river is suitable for the cultivation of rice under natural flooding conditions.  Prediction of runoff and water levels can be done, but its reliability depends strongly on the reliability of the seasonal rainfall forecast.  At present this forecast does not yet have the required precision, but this can be expected to improve in the future.

Acknowledgements

The authors would like to express their gratitude to the staff of the Department of Agriculture of Western Province, in particular to the Land Use Planning Section, who have participated in most of the fieldwork; to the Departments of Meteorology and the Department of Water Affairs, who have supplied a large part of the data for this study; and to the Directorate for Development Cooperation (DGIS) of the Ministry of Foreign Affairs, The Netherlands, which through their financial support to the Land and Water Management Project under code ZM/91/014 have enabled the authors to perform this study.

References

Bastiaansen,  C.J.M.  (1995)  Lui River Valley Model and some of its applications. 	Report 55, Department of Water Resources, Wageningen Agricultural University, The Netherlands

Chiyala, K.  A. Huizing & L. van den Brand  (1995) Wetlands in Western province, the role 	of wetlands in agricultural development. Presented on the symposium on "a systems approach to long term soil productivity, Lusaka, Zambia

Gils, H. van  (1988)  Environmental profile Western Province, Zambia.  ITC, Enschede,

The Netherlands & Provincial Planning Unit, Mongu, Zambia

Heemskerk,  W.  (1995)  Vulnerability assessment Western Province.  Farming Systems

Research Team Western Province, Department of Agriculture

IUCN  (1992)  The IUCN review of the Southern Okavango Integrated Water Development 	Project. IUCN

Ketelaars,  A.L.E.  (1991)  Multiple linear regression model on Symphony spreadsheet for 	monthly rainfall runoff relations.

Offergelt,  C.  (1980)  Detailed soil survey of the Mushwala area, Kaoma District, Western 	Province.  Soil Survey Report No. 64, Soil Survey Unit, Land Use Branch, Department of Agriculture

Oosterbaan,  R.J.  (1986)  Frequency and regression analysis.  Lecture notes for the 25th 	International Course on Land Drainage, ILRI, Wageningen, The Netherlands

SADCC AAA.3.4  Hydroelectric Hydrological Assistance Project - Phase 1  (1992)  Review 	of the Upper Zambezi Hydrometeorological Network. Lusaka, Zambia

Savenije,  H.H.G.  (1994)  Principles of Hydrology. Lecture notes for the IHE, Delft, The 	Netherlands

Savenije,  H.H.G.  &  M. Mutale (1994)  Report of the Hydrological Backstopping Mission 	Land and Water Management Project, Mongu, Zambia

Viessman,  W.  ,G.L. Lewis & J.W. Knapp  (1989)  Introduction to Hydrology, 3rd edition.  	Harper & Row, New York, USA